3GPP TSG-RAN WG2 Meeting #123	R2-2307744

# Toulouse, France, August 21 – August 25, 2023

Source:	Qualcomm Incorporated

Title:	Proposal on additional enhancement for service continuity

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

In last RAN2 meeting, there are some proposals to propose additional enhancements. This contribution discusses whether these enhancements need to be supported in Rel-18.

# Discussion

According to the WID description[1] for service continuity enhancement for the following scenario.

Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

From the WID, service continuity enhancement in Rel-18 should focus on basic functionalities supporting the scenario A, B, C, D as WID.

There are some proposals captured in [2] to further enhance service continuity as following.

- simultaneous relay UE’s inter-gNB HO and connected remote UE’s path switching

- selection of relay UE in RRC_IDLE or RRC_INACTIVE state

- relay UE’s cell reselection or HO during indirect path switching of the remote UE

- prolonged inter-gNB signaling over Xn interface for inter-gNB path switching

- CHO-like path switching solution for remote UE

- DAPS like path switch solution for remote UE

- group handover for relay UE and remote UE(s)

For bullet 1, 3, 7, the enhancements is group mobility for Relay UE and Remote UE, group mobility is is the same issue for all the scenarios of intra-gNB and inter-gNB handover, should be out of Rel-18.

For bullet 2, selection of Relay UE in RRC-IDLE or Inactive state can be up to gNB implementation.

For bullet 4,5, 6, these proposals are to further improve service continuity performance during mobility. These are not simple issues, and even in existing Uu mobility, there are several releases on Uu mobility enhancements. Then it is impossible to improve the service continuity performance due to limited time in Rel-18.

Then, it is proposed RAN2 does not pursue the following enhancements in Rel-18.

Proposal: RAN2 does not pursue the following enhancements in Rel-18.

- Simultaneous relay UE’s inter-gNB HO and connected remote UE’s path switching

- Selection of relay UE in RRC_IDLE or RRC_INACTIVE state

- Relay UE’s cell reselection or HO during indirect path switching of the remote UE

- Prolonged inter-gNB signaling over Xn interface for inter-gNB path switching

- CHO-like path switching solution for remote UE

- DAPS like path switch solution for remote UE

- Group handover for relay UE and remote UE(s)

# Conclusion

This contribution discusses RAN2 part for UE-to-UE relay discovery and (re-) selection and provides the following proposal.

Proposal: RAN2 does not pursue the following enhancements in Rel-18.

- Simultaneous relay UE’s inter-gNB HO and connected remote UE’s path switching

- Selection of relay UE in RRC_IDLE or RRC_INACTIVE state

- Relay UE’s cell reselection or HO during indirect path switching of the remote UE

- Prolonged inter-gNB signaling over Xn interface for inter-gNB path switching

- CHO-like path switching solution for remote UE

- DAPS like path switch solution for remote UE

- Group handover for relay UE and remote UE(s)

# Reference

[1]    RP-223501 Revised WID on NR sidelink relay enhancements

[2]    R2-2306559 Summary of AI 7.9.3 on service continuity (vivo)	vivo