3GPP TSG-RAN WG2 Meeting #123	R2-2307745

# Toulouse, France, August 21 – August 25, 2023

Source:                Qualcomm Incorporated

Title:	Open issues on multi-path relay for scenario 1 and scenario 2

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for REl-18 Relay enhancement, and this contribution discusses the open issues for scenario 1 and scenario 2.

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work.

Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline.

Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

# Discussion

## 2.1 SRB configuration

In last RAN2 meeting, RAN2 had way forward on primary path configuration of split SRB1 and SRB2 as following.

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

It was agreed that PCell is only on direct path and following existing DCCA mechanism, the SRB1 and SRB2 are configured in the same path as PCell. it should be taken as baseline; otherwise, RAN2 needs to spend time to review whether all the existing procedures can work well.

Uu coverage is more larger than PC5 coverage, it is more possible that UE moves outside or inside of PC5 coverage frequently, and then indirect path may lost coverage more frequently. There are two hops on indirect path, any hop failure will lead to indirect path failure. So direct path is more reliable than indirect path.

Observation 1: Direct path is more reliable than indirect path, due to 1) Uu coverage is much larger than SL coverage, 2) any hop failure on the indirect path will lead to indirect path failure.

Observation 2: UE implementation complexity will be introduced if allowing various configurations in specification.

Observation 3: Both SRB1 and SRB2 are configured on the path of PCell currently, RAN2 needs to spend time to review whether existing procedures will be broken if deviating from existing design principle.

Then it is proposed to reuse the existing mechanism.

Proposal 1: Confirm the WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.

In NR-DC MCG fast recovery, when MCG is failure, UE can send MCGFailureInformation over SCG path of the split SRB1.

Proposal 2: Same as existing MCG fast recovery, UE can send MCGFailureInformation over indirect path of slit SRB1 in case MCG is failure.

Proposal 3: Non-split SRB1 is configured only on direct path.

## 2.2 Trigger Relay UE entering connected state

It has been discussed and agreed on how to trigger Relay UE entering CONNECTED state as following.

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

If split SRB1 is configured, the Remote UE send RRCReconfigurationComplete message over both of direct and indirect paths, then IDLE and Inactive state can be triggered into connected state.

If split SRB1 is not configured, then the Remote UE can send PC5-RRC message to trigger Relay UE to enter connected state.

Proposal 4: If split SRB1 is configured, the Remote UE send RRCReconfigurationComplete message over both of direct and indirect paths; If split SRB1 is not configured, then the Remote UE can send PC5-RRC message to trigger Relay UE to enter connected state.

There is one issue, the legacy PC5-RRC message and parameters cannot trigger Relay UE into CONNECTED state as current specification, then a new PC5-RRC message or parameters or procedure need to be introduced.

Proposal 5: A new PC5-RRC message or parameter needs to be introduced to trigger Relay UE into CONNECTED state.

However, currently, Remote UE does not to know whether the candidate Relay UE is legacy Relay UE or new Relay UE, that means whether the target Relay UE supports the new PC5-RRC message or parameter. In order to let Remote UE know the Relay UE capability, Relay UE needs to include the capability into the discovery message.

Proposal 6: Relay indicates the capability of supporting PC5-RRC based method to enter into RRC_CONNECTED state in discovery message (i.e. AS container). Send LS to SA2 and CT1.

In order to assist gNB to select the target Relay and provide SRB1 configuration, the Remote UE needs to inform the gNB whether the candidate Relay UE support PC5-RRC based method to enter into CONNECTED state or not.

Proposal 7: Remote UE indicates candidate Relay UE’s capability of supporting PC5-RRC based method to enter into CONNECTED state when reporting candidate Relay UEs to gNB.

## 2.3 Path failure handling

For direct path failure recovery, it was agreed to use MCGFailureInformation to report direct pati failure, but it is FFS on whether addition IE needs to be introduced.

For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

Since there is no change introduced on direct path failure detection, then there is no motivation to add new IE in MCGFailureInformation message

Proposal 8: No additional IE needs to be introduced in MCGFailureInformation message for reporting direct-path failure via indirect-path.

For direct path failure reporting, there were several options to report indirect path, e.g. MCGFailureInformation, SCGFailureInformation, SidelinkUEInformationNR, or a new RRC message. MCGFailureInformation is used for direct path failure reporting, then it is not suitable for indirect path failure reporting. SidelinkUEInformationNR cannot be used for scenario 2. It is beneficial to introduce a new RRC message to indicate indirect path failure for both Scenario 1 and Scenario 2.

Proposal 9: For both of Scenario-1 and 2, introduce a new message to indicate indirect path failure.

Proposal 10: In the following conditions, UE should initiate indirect path failure

- Failure on sidelink, e.g. sidelink RLF

- Failure on indirect path configuration, including Uu part or sidelink part configuration failure

- Failure on Relay Uu link, e.g. Relay Uu RLF

2.4 Path addition, removal and change

Measurement events for path addition, removal and change

For indirect path, the current Event e.g.Y2 can be reused to trigger indirect path addition. When the Remote UE discovers a Relay UE and the radio quality on sidelink satisfies Event Y2, the Remote UE can send measurement to the gNB, including the candidate Relays and serving cells. The gNB will select the target Relay which serving cell is in the same gNB as Remote UE Uu cell. The gNB will configure the indirect path as secondary path using existing relay configuration mechanism.

Similarly, the existing Event e.g. A4 can be reused for direct path addition and reuse existing Uu reconfiguration procedure; and it should be gNB to select the target cell which is in the same gNB as the Relay UE’s serving cell.

Proposal 11: The existing measurement report event, e.g. Event Y2 reported by Remote UE can be used for indirect path addition.

Proposal 12: The existing measurement report event, e.g. Event A4 reported by Remote UE can be used for direct path addition.

Similar with MP path addition, the existing measurement event e.g.X2 can be reported by Remote UE and is reused for indirect path removal and measurement event A2 reported by Remote UE can be used for direct path removal.

Proposal 13: The existing measurement report event, e.g. Event X2 or Event A2 reported from Remote UE can be used for indirect path removal or direct path removal respectively.

Similarly, the indirect path can be removed due to Relay UE Uu link quality change, e.g Uu RLF, serving cell quality is below a threshold (i.e, Event A2), HO the measurement report from Relay UE triggered by existing events, e.g. Event A2 in case that all of the serving cells are getting worse.

Proposal 14: The existing Relay UE Uu link situation change (e.g. Uu RLF, A2 reporting, HO) could trigger indirect path removal.

Indirect path change, the newly introduced Z1 measurement event for indirect-indirect path swithing can be reused to trigger indirect path change.

Proposal 15: The newly introduced Z1 measurement event for indirect-indirect path swithing is reused to trigger indirect path change.

Proposal 16: No new measurement report events will be introduced for indirect/direct path addition, removal or change.

Timers handling,

There are some FFS issues on the timer handling during path addition and change.

Agreements:

For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behaviour.

For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

FFS if these two timers are new or reuse the existing timers.

In NR-DC, when T304 timer expires, UE informs network about the reconfiguration failure by initiating MCGFailureInformation message. For direct path addition and change, same handling should be reused during direct path addition and changes.

Proposal 17: When T304-like timer for direct path addition and change expires, UE informs network about the reconfiguration failure by initiating MCGFailureInformation message

In existing path switching to indirect path, the T420 timer is stopped when PC5 RLC acknowledgement is received from target L2 U2N Relay UE. For indirect path addition, if SRB1 is configured on indirect path, then UE will send RRCReconfigurationComplete message, and T420-like timer is stopped when acknowledgement is received form the Relay UE; if SRB1 is not configured on indirect path, then UE will send RRCReconfigurationComplete message over direct path and T420-like timer is stopped when acknowledgement is received from gNB.

Proposal 18: UE stops T420-like timer for indirect path addition and change on the condition of RRCReconfigurationComplete message is acknowledged by gNB or by the Relay UE.

Since the stop condition of T420-like timer is different with existing condition, then a new timer needs to be introduced for indirect path addition and change. For direct path addition, there is no different UE behavior, then existing T304 timer can be reused.

Proposal 19: Reuse the existing T304 timer for direct path addition and change; introduce a new timer for indirect path addition and change.

2.5 Authorization for Scenario 2

SA2 discussed the authorization for scenario 2 MP relay and provides the following feedback to RAN2 in [3] and leaves the issue to RAN2.

Therefore, SA2 would like to provide the following answer to the question from RAN2:

■ Multi-path transmission authorization and subscription function for Scenario 2 in all cases is out of SA2 scope.

Currently, the UE-UE link association and authorization is out of 3GPP, so it is necessary to authorize the UE to configure MP relay in Scenario 2; otherwise, any un-authorized UEs can request MP configuration as long as the Remote UE can connect with a Relay UE. This will be out of network control and waste network radio resource.

Currently, there is authorization informed to gNB for Scenario-1 MP relay, then one possible way is that gNB determines whether to provide Scenario-2 MP relay based on Scenario-1 MP relay authorization. That means Scenario-1 and Scenario-2 MP relay share the same authorization informed to the gNB.

Proposal 20: Authorization for Scenario-2 MP relay is needed and Scenario-2 MP relay shares the same authorization information with Scenario-1 informed to the gNB.

However, subscription and authorization should be decoupled with UE radio capability. That means sharing the same authorization does not means UE supporting Scenario-2 relay has to support Scenario-1 relay. From RAN2 perspective, it should be separate UE  radio capabilities to support Scenario-1 MP relay and Scenario-2 MP relay.

Proposal 21: UE radio capabilities to support Scenario-1 MP relay and Scenario-2 MP relay are separate.

2.6 Relay UE identity for Scenario 2

And it is not reasonable to request the Relay UE entering CONNECTED state before it is added to MP relay, in case that the gNB does not configure MP relay to the Remote UE. For example, when gNB is overload and will not configure MP to Remote UE after receiving report from the Remote UE. But the Remote UE will trigger the Relay UE entering CONNECTED state again and again in order to request MP relay operation. It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB.

Observation 4: If Relay UE is required to be in CONNECTED state when the Remote UE reports the Relay UE ID to the gNB,  in case that gNB dos not configure MP relay due to overload, Remote UE will trigger the Relay UE entering CONNECTED state again and again in order to request MP relay operation. It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB .

When Relay UE enters CONNECTED state from IDLE state, Relay UE will indicates PLMN ID (MCC, MNC), GUAMI(AMF Region ID, an AMF Set ID and an AMF Pointer), and 5G-S-TMSI as shown in following message.

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

registeredAMF                       RegisteredAMF                                   OPTIONAL,

guami-Type                          ENUMERATED {native, mapped}                     OPTIONAL,

s-NSSAI-List                        SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI  OPTIONAL,

dedicatedNAS-Message                DedicatedNAS-Message,

ng-5G-S-TMSI-Value                  CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

ng-5G-S-TMSI-Part2                  BIT STRING (SIZE (9))

}                                                                                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1610-IEs                      OPTIONAL

}

RegisteredAMF ::=                   SEQUENCE {

plmn-Identity                       PLMN-Identity                                   OPTIONAL,

amf-Identifier                      AMF-Identifier

}

–	AMF-Identifier

The IE AMF-Identifier (AMFI) comprises of an AMF Region ID, an AMF Set ID and an AMF Pointer as specified in TS 23.003 [21], clause 2.10.1.

AMF-Identifier information element

-- ASN1START

-- TAG-AMF-IDENTIFIER-START

AMF-Identifier ::=                      BIT STRING (SIZE (24))

-- TAG-AMF-IDENTIFIER-STOP

-- ASN1STOP

Currently, the 5G-GUTI and 5-S-TMSI definition is as following in TS23.003,

<5G-GUTI>:= <MCC> <MNC><AMF Region ID> <AMF Set ID> <AMF Pointer><5G-TMSI>

<5G-S-TMSI> := <AMF Set ID> <AMF Pointer> <5G-TMSI>

Then when Remote UE reports Relay UE’s 5G-GUTI, the gNB should be able to identify the Relay UE based on the information received from the Relay UE.

Observation 5: Today, gNB can obtain Relay UE 5G-GUTI during RRC setup procedure.

For Inactive state Relay UE, the I-RNTI can be used to identify the Relay UE.

Proposal 22: For Scenario 2, RAN2 confirms target Relay UE can be in IDLE and Inactive state.

Proposal 23: Using the following ID to identify the candidate Relay UE

- If the Relay UE in IDLE state, Remote UE reports candidate Relay UE’s 5G-GUTI to     identify the Relay UE;

- If the Relay UE is in Inactive state, Remote UE reports candidate Relay UE’s I-RNTI to identify the Relay UE;

2.7 Relay UE configuration for Scenario 2

It was agreed in RAN2 #119bis-e that gNB provides bearer mapping information to relay UE through dedicated signalling, one further issue is which message should be used. But it is FFS on how to configure the mapping of Remote UE bearer and Relay UE Uu RLC Channel as following.

Working assumptions:

Proposal 3A: Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path.  FFS how to configure the mapping.

There could be two options to provide configuration to Relay UE in scenario 2.

Option 1: Reuse RLC-BearerConfig in CG configuration

As shown in the following, gNB can provide Relay UE RLC Channel configuration for the Remote UE bearer using RLC-BearerConfig  in CG configuration. In order to distinguish the bearer is for Remote UE or for Relay UE, an indication need to be added into RLC-BearerConfig for the RLC Channel configuration.

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]],

[[

rlc-Config-v1700                            RLC-Config-v1700                                    OPTIONAL,   -- Need R

logicalChannelIdentityExt-r17               LogicalChannelIdentityExt-r17                       OPTIONAL,   -- Cond LCH-SetupModMRB

multicastRLC-BearerConfig-r17               MulticastRLC-BearerConfig-r17                       OPTIONAL,   -- Cond LCH-SetupOnlyMRB

servedRadioBearerSRB4-r17                   SRB-Identity-v1700                                  OPTIONAL    -- Need N

]]

}

Option 2: Reuse SL-L2RelayUE-Config

Another possible way is the existing sl-L2RelayUE-Config-r17 can be reused as following. The gNB only needs to configure the yellow highlight IEs; for the unnecessary mandatory IE in green highlight, the Relay UE can ignore it.

–	SL-L2RelayUE-Config

The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, e.g. SRAP-Config.

SL-L2RelayUE-Config information element

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17           SL-SRAP-Config-r17                                                    OPTIONAL,    -- Need M

...

}

-- TAG-SL-L2RELAYUE-CONFIG-STOP

-- ASN1STOP

–	SL-SRAP-Config

The IE SL-SRAP-Config is used to set the configurable SRAP parameters used by L2 U2N Relay UE and L2 U2N Remote UE as specified in TS 38.351 [66].

SL-SRAP-Config information element

-- ASN1START

-- TAG-SL-SRAP-CONFIG-START

SL-SRAP-Config-r17 ::=                  SEQUENCE {

sl-LocalIdentity-r17                    INTEGER (0..255)                                                 OPTIONAL, -- Need M

sl-MappingToAddModList-r17              SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17          OPTIONAL, -- Need N

sl-MappingToReleaseList-r17             SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17     OPTIONAL, -- Need N

...

}

SL-MappingToAddMod-r17 ::=              SEQUENCE {

sl-RemoteUE-RB-Identity-r17             SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17              Uu-RelayRLC-ChannelID-r17                                        OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17             SL-RLC-ChannelID-r17                                             OPTIONAL, -- Need N

...

}

SL-RemoteUE-RB-Identity-r17 ::=         CHOICE {

srb-Identity-r17                        INTEGER (0..3),

drb-Identity-r17                        DRB-Identity,

...

}

-- TAG-SL-SRAP-CONFIG-STOP

-- ASN1STOP

Proposal 24: RAN2 discuss which option is used to provide mapping of Uu RLC Channel and Remote UE bearer to Relay UE in scenario 2:

Option 1: Reuse RLC-BearerConfig in CG configuration, add a new indication to distinguish the Remote UE’s bearer or Relay UE’s bearer for each RLC Channel configuration

Option 2: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

# Conclusion

This contribution discusses the aspects of architecture, bearer management, path management for multi-path operation, and provides the proposals.

SRB configuration,

Observation 1: Direct path is more reliable than indirect path, due to 1) Uu coverage is much larger than SL coverage, 2) any hop failure on the indirect path will lead to indirect path failure.

Observation 2: UE implementation complexity will be introduced if allowing various configurations in specification.

Observation 3: Both SRB1 and SRB2 are configured on the path of PCell currently, RAN2 needs to spend time to review whether existing procedures will be broken if deviating from existing design principle.

Proposal 1: Confirm the WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.

Proposal 2: Same as existing MCG fast recovery, UE can send MCGFailureInformation over indirect path of slit SRB1 in case MCG is failure.

Proposal 3: Non-split SRB1 is configured only on direct path.

Trigger Relay UE entering connected state,

Proposal 4: If split SRB1 is configured, the Remote UE send RRCReconfigurationComplete message over both of direct and indirect paths; If split SRB1 is not configured, then the Remote UE can send PC5-RRC message to trigger Relay UE to enter connected state.

Proposal 5: A new PC5-RRC message or parameter needs to be introduced to trigger Relay UE into CONNECTED state.

Proposal 6: Relay indicates the capability of supporting PC5-RRC based method to enter into RRC_CONNECTED state in discovery message (i.e. in AS container).

Proposal 7: Remote UE indicates candidate Relay UE’s capability of supporting PC5-RRC based method to enter into CONNECTED state when reporting candidate Relay UEs to gNB.

Path failure handling,

Proposal 8: No additional IE needs to be introduced in MCGFailureInformation message for reporting direct-path failure via indirect-path.

Proposal 9: For both of Scenario-1 and 2, introduce a new message to indicate indirect path failure.

Proposal 10: In the following conditions, UE should initiate indirect path failure

- Failure on sidelink, e.g. sidelink RLF

- Failure on indirect path configuration, including Uu part or sidelink part configuration failure

- Failure on Relay Uu link, e.g. Relay Uu RLF

Path addition, removal and change,

Proposal 11: The existing measurement report event, e.g. Event Y2 reported by Remote UE can be used for indirect path addition.

Proposal 12: The existing measurement report event, e.g. Event A4 reported by Remote UE can be used for direct path addition.

Proposal 13: The existing measurement report event, e.g. Event X2 or Event A2 reported from Remote UE can be used for indirect path removal or direct path removal respectively.

Proposal 14: The existing Relay UE Uu link situation change (e.g. Uu RLF, A2 reporting, HO) could trigger indirect path removal.

Proposal 15: The newly introduced Z1 measurement event for indirect-indirect path swithing is reused to trigger indirect path change.

Proposal 16: No new measurement report events will be introduced for indirect/direct path addition, removal or change.

Proposal 17: When T304-like timer for direct path addition and change expires, UE informs network about the reconfiguration failure by initiating MCGFailureInformation message

Proposal 18: UE stops T420-like timer for indirect path addition and change on the condition of RRCReconfigurationComplete message is acknowledged by gNB or by the Relay UE.

Proposal 19: Reuse the existing T304 timer for direct path addition and change; introduce a new timer for indirect path addition and change.

Authorization for Scenario 2

Proposal 20: Authorization for Scenario-2 MP relay is needed and Scenario-2 MP relay shares the same authorization information with Scenario-1 informed to the gNB.

Proposal 21: UE radio capabilities to support Scenario-1 MP relay and Scenario-2 MP relay are separate.

Relay UE identity for Scenario 2

Observation 4: If Relay UE is required to be in CONNECTED state when the Remote UE reports the Relay UE ID to the gNB,  in case that gNB dos not configure MP relay due to overload, Remote UE will trigger the Relay UE entering CONNECTED state again and again in order to request MP relay operation. It is a huge burden for Relay UE to enter CONNECTED state every time the Remote UE reports the Relay UE ID to the gNB .

Observation 5: Today, gNB can obtain Relay UE 5G-GUTI during RRC setup procedure.

Proposal 22: For Scenario 2, RAN2 confirms target Relay UE can be in IDLE and Inactive state.

Proposal 23: Using the following ID to identify the candidate Relay UE in scenario 2

- If the Relay UE in IDLE state, Remote UE reports candidate Relay UE’s 5G-GUTI to     identify the Relay UE;

- If the Relay UE is in Inactive state, Remote UE reports candidate Relay UE’s I-RNTI to identify the Relay UE;

Relay UE configuration for Scenario 2

Proposal 24: RAN2 discuss which option is used to provide mapping of Uu RLC Channel and Remote UE bearer to Relay UE in scenario 2:

Option 1: Reuse RLC-BearerConfig in CG configuration, add a new indication to distinguish the Remote UE’s bearer or Relay UE’s bearer for each RLC Channel configuration

Option 2: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

# Reference

[1]    RP-221262  New WID on NR sidelink relay enhancements

[2]    RAN2#123 Chair Notes

[3]    R2-2307057 Reply LS to SA2 on authorization for multi-path Scenario 2 (S2-2307707; contact: LGE)	SA2