3GPP TSG-RAN WG2 Meeting #123	R2-2307761

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.4.1

Source:		Futurewei

Title:	Discussions on delay status reporting in BSR for XR

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #122, RAN2 agreed on the following with respect to delay status reporting in BSR for XR [1]:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In this contribution, we further discuss the details for the delay status reporting in BSR for XR.

# Discussions

## On how to convey remaining time information for XR

There are two general approaches for conveying the remaining time information to the gNB, namely explicit indicating and implicit indicating.

In the explicit indicating approach, as we previously discussed [2], the gNB can send the remaining time information explicitly, e.g., along with the buffer size information in the BSR. To keep the signaling overhead to the minimal, only the remaining time information of the oldest (earliest) data burst in the buffer is explicitly signaled to the gNB, e.g., along with the data volume information of the oldest data burst. If there are any data bursts (i.e., non-oldest data burst) arrived during a subsequent traffic cycle and buffered at the same LCG while the oldest data burst is still valid (i.e., still within its PDB/PSDB) and being transmitted, the data volume information of the non-oldest data burst(s) can be separately included in the BSR without including the remaining time information associated with them. The gNB can estimate the remaining time associated with the non-oldest data burst(s) based on the remaining time reported for the oldest data burst.

In the implicit indicating approach, as we previously discussed [3], even the remaining time information of the oldest (or the most urgent) data burst in the buffer is not explicitly signaled in the BSR. Instead, the UE classifies the PDUs buffered at the LCG as urgent PDUs or non-urgent PDUs, e.g., by comparing their associated remaining time (or dwelling time, i.e., the time elapsed since the PDU arrives at the PDCP, if the remaining time may be artificially shorten to a non-zero value by the discard operation) to a pre-configured time threshold. The UE reports the data volumes of the urgent PDUs and the non-urgent PDUs separately in the BSR for the LCG. Based on the BSR sent from the UE, the gNB will ensure that the UE is scheduled with sufficient radio resource to deliver at least the urgent PDUs within the current traffic cycle. For example, if the CG-scheduled radio resource within the current traffic cycle is insufficient, the gNB will supplement it with additional DG(s) for the UE within the current traffic cycle.

The explicit approach provides the gNB with more accurate delay information but requires extra signaling overhead and requires the gNB to keep tracking the remaining time. On the other hand, the implicit approach gives the gNB sufficient information regarding whether the currently scheduled UL resource for the UE is sufficient for delivering at least the urgent data within the current traffic cycle, without the extra signaling overhead and without a need for the gNB to track the remaining time. If RAN2 decides to support both explicit and implicit approaches, the gNB can use dedicated RRC signaling to configure a UE to use one of the two approaches for conveying the remaining time of the urgent data in its BSR for the XR LCG.

In either of these two approaches, the data volume of the oldest data burst or of the urgent PDUs (referring to as the urgent data volume) is reported separately from the data volume of the remaining data buffered at the same LCG (referring to as the non-urgent data volume) for the LCG in the BSR.

Proposal 1. RAN2 study the two general approaches, i.e., explicit indication vs. implicit indication as described, for conveying the remaining time information to the gNB and consider adopting one from them.

Proposal 2. If RAN2 decides to support both explicit and implicit approaches, the gNB can use dedicated RRC signaling to configure a UE to use one of the two approaches for conveying the remaining time of the urgent data in its BSR for the XR LCG.

Proposal 3 The urgent data volume of an LCG is reported separately from the non-urgent data volume of the LCG (i.e., the total data volume of the data buffered at the LCG subtracted by the urgent data volume of the LCG) in the BSR.

## On the BSR MAC CE format for XR

We propose that the urgent data volume is conveyed to the gNB by using a new XR BSR MAC CE. The XR BSR MAC CE includes a Buffer Size field carrying information indicating the urgent data volume. If explicit delay status (DS) report is configured, the XR BSR MAC CE further includes a DS field indicating the remaining time associated with the urgent data volume being reported. On the other hand, if implicit DS report is configured, the XR BSR MAC CE itself implies that the remaining time associated with the urgent data volume being reported has dropped below the pre-configured time threshold, which is known to (e.g., configured by) the gNB.

There is no critical need for conveying the remaining time information associated with the non-urgent data volume, because there is no urgency for the UE to transmit the non-urgent data within a current traffic cycle. The UE can transmit any non-urgent data opportunistically if there are still granted UL resource left. When any remaining non-urgent data becomes urgent data at a future time, the UE will duly report the data volume of these data by sending another XR BSR MAC CE to the gNB. Hence, the non-urgent data volume can be reported using one of the legacy BSR MAC CEs, without an explicit indication of the remaining time associated with. However, gNB implementations may estimate the remaining time associated with the non-urgent data based on the traffic periodicity and the remaining time associated with the urgent data being reported.

Therefore, for XR, a BSR can be reported using one of the following:

- One XR BSR MAC CE:

  - used when reporting only for the XR LCG and only with the urgent data volume of it.

- One of the legacy BSR MAC CEs:

  - used when reporting for the non-XR LCG(s) and/or for the XR LCG but only with the non-urgent data volume of it (e.g., when no XR PDUs can qualify as the urgent ones).

- One XR BSR MAC CE plus one of the legacy BSR MAC CEs:

  - used when reporting both the urgent data volume of the XR LCG and one of the following: the non-urgent data volume of the XR LCG, the legacy data volume(s) on the non-XR LCG(s), or both.

Proposal 4. The urgent data volume of an LCG associated with the XR QoS flow is reported using a new XR BSR MAC CE.

Proposal 5. If explicit delay status (DS) report is configured, the XR BSR MAC CE further includes a DS field indicating the remaining time associated with the urgent data volume being reported.

Proposal 6. If implicit DS report is configured, the XR BSR MAC CE itself implies that the remaining time associated with the urgent data volume being reported has dropped below the pre-configured time threshold, which is known to (e.g., configured by) the gNB.

Proposal 7. The non-urgent data volume of an LCG associated with the XR QoS flow is reported using one of the legacy BSR MAC CEs, without an explicit indication of the remaining time associated with. However, gNB implementations may estimate the remaining time associated with the non-urgent data based on the traffic periodicity and the remaining time associated with the urgent data being reported.

# Conclusions

The followings are proposed:

Proposal 1. RAN2 study the two general approaches, i.e., explicit indication vs. implicit indication as described, for conveying the remaining time information to the gNB and consider adopting one from them.

Proposal 2. If RAN2 decides to support both explicit and implicit approaches, the gNB can use dedicated RRC signaling to configure a UE to use one of the two approaches for conveying the remaining time of the urgent data in its BSR for the XR LCG.

Proposal 3 The urgent data volume of an LCG is reported separately from the non-urgent data volume of the LCG (i.e., the total data volume of the data buffered at the LCG subtracted by the urgent data volume of the LCG) in the BSR.

Proposal 4. The urgent data volume of an LCG associated with the XR QoS flow is reported using a new XR BSR MAC CE.

Proposal 5. If explicit delay status (DS) report is configured, the XR BSR MAC CE further includes a DS field indicating the remaining time associated with the urgent data volume being reported.

Proposal 6. If implicit DS report is configured, the XR BSR MAC CE itself implies that the remaining time associated with the urgent data volume being reported has dropped below the pre-configured time threshold, which is known to (e.g., configured by) the gNB.

Proposal 7. The non-urgent data volume of an LCG associated with the XR QoS flow is reported using one of the legacy BSR MAC CEs, without an explicit indication of the remaining time associated with. However, gNB implementations may estimate the remaining time associated with the non-urgent data based on the traffic periodicity and the remaining time associated with the urgent data being reported.

# References

R2-2306542, RAN2-122 Report on LTE MUSIM QoE XR, Vice Chairman (Nokia).

R2-2302527, Discussion on BSR enhancements for XR, Futurewei.

R2-2304864, Further discussions on BSR enhancements for XR, Futurewei.