3GPP TSG-RAN WG2 Meeting #123	R2-2307763

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.4.2

Source:		Futurewei

Title:	Discussions on PSI-based discard operation for XR

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #122, RAN2 agreed on the following with respect to discard operation for XR [1]:

2: PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

FFS how/whether to minimize additional UL signalling after this indication.

FFS if the NW indication is a one-shot or also subsequent packets

In this contribution, we discuss how PSI impacts discard operation (e.g., do we have one or two timers) and how the network indicates the UE to apply PSI-based XR discard. We also discuss potential impact of discarding PDU Set on PDCP receive operation.

# Discussions

## On how the network indicates the UE to apply PSI-based XR discard

Although RAN2 has agreed that, when the UL is congested, the network indicates UE to apply PSI-based XR discard mechanism via dedicated signaling, what is in the dedicated signaling has not been agreed yet.

In one option, the dedicated signaling can be sent to indicate a PSI value as a threshold. If the UE receives such dedicated signaling, any PDU Set subsequently received from upper layer with a PSI value associated with a less importance than the importance associated with the indicated PSI value is subject to early discarding, while any PDU Set subsequently received from upper layer with a PSI value associated with a same importance as or a more importance than the importance associated with the indicated PSI value isn’t subject to the early discarding, i.e., they are subject to the normal discarding based on the legacy discardTimer or a positive acknowledgement from the PDCP Status Report.

In another option, the dedicated signaling indicates a congestion level and the UE maps the indicated congestion level to a PSI threshold value. Then, any PDU Set subsequently received from upper layer with a PSI value associated with a less importance than the importance associated with the indicated PSI value is subject to early discarding, while any PDU Set subsequently received from upper layer with a PSI value associated with a same importance as or a more importance than the importance associated with the indicated PSI value isn’t subject to the early discarding, i.e., they are subject to the normal discarding based on the legacy discardTimer or a positive acknowledgement from the PDCP Status Report.

When the UL is no longer congested, the network indicates UE, via another dedicated signaling, to stop applying the PSI-based discard mechanism.

Proposal 1. RAN2 discuss and decide whether the network indicates UE, via the dedicated signaling, a PSI threshold value directly, or indicates a congestion level and the UE maps the congestion level to a PSI thershold value, when the UL becomes congested.

Proposal 2. Upon the UE receiving the dedicated signaling, any PDU Set subsequently received from upper layer with a PSI value associated with a less importance than the importance associated with the PSI threshold value is subject to early discarding, while any PDU Set subsequently received from upper layer with a PSI value associated with a same importance as or a more importance than the importance associated with the PSI threshold value isn’t subject to the early discarding.

Proposal 3. When the UL is no longer congested, the network indicates UE, via another dedicated signaling, to stop applying the PSI-based discard mechanism.

## On one or two discard timers

One possible way of early discarding is to discard any PDU of the PDU Set that is subject to the early discarding, as determined above, upon the arrival of the PDU at the transmitting PDCP entity, at which time the PSI value associated with the PDU is passed along with the PDU from the GTP-U entity to the SDAP entity and finally to the transmitting PDCP entity. If RAN2 decides that the PSI-based discarded PDUs are discarded upon arrival at the transmitting PDCP entity, there is no need for introducing a second discard timer. Even the legacy discardTimer is not started for these PDUs (PDCP SDUs). This could make the UE implementation easier.

On the other hand, if RAN2 decides that the the PSI-based discarded PDUs can be discarded some time after the  arrival of the PDU at the transmitting PDCP entity but still earlier than the expiry of the legacy discardTimer, then a second discard timer can be started with a non-zero initial timer value, which can be configured by the RRC signaling and PSI-dependent. Meanwhile the legacy discardTimer is still started with the legacy timer value upon the arrival of the PDU and keeps running until it is stoped or expired. When the second discard timer expires, the legacy discardTimer associated with the same PDU Set as the second discard timer is stopped. In this way, the remaining time on the legacy discardTimer continues to reflect the true aging of the associated PDUs and hence can be used by the UE for determining the urgency (i.e., urgent or non-urgent) for transmitting the associated PDU Set. Meanwhile, the remaining time on the second discard timer, which has been distorted by the PSI-dependent initial timer value, cannot be used for determining the urgency for transmitting an associated PDU Set. We notice a special case where if the RRC signaling configures the initial timer value for the second discard timer to be 0 (zero) for all PSI values, it effectively becomes discard-upon-arrival.

Proposal 4. RAN2 discuss and decide whether a PDU Set subject to early discarding should be discarded upon arrival at the transmitting PDCP entity or can be discarded some time after its arrival at the transmitting PDCP entity but still earlier than the expiry of the legacy discardTimer.

Proposal 5. If RAN2 decides that the PSI-based discarded PDUs are discarded upon arrival at the transmitting PDCP entity, there is no need for introducing a second discard timer.

Proposal 6. If RAN2 decides that the the PSI-based discarded PDUs can be discarded some time after their arrival at the transmitting PDCP entity but still earlier than the expiry of the legacy discardTimer, introduce a second discard timer, which is started with a non-zero and potentially PSI-dependent initial timer value configured by the RRC signaling for a PDU Set that is subject to early discarding due to a cogestion condition and its PSI value. Meanwhile, the legacy discardTimer is also started with the legacy timer value configued for the PDCP entity, which timer value is PSI-independent. The expiry of either timer triggers the discarding of the associated PDU Set.

## Potential impact on receive operation due to discard operation

In our Tdoc [3] submitted to RAN2 #122, we have described some potential issues of PDCP reordering and HFN desynchronization due to PDU discarding.

To address these potential issues, the transmitting PDCP entity may inform the receiving PDCP entity about the COUNT value(s) discarded, e.g., by sending a PDCP control PDU to indicate the range of COUNT value(s) being discarded or to indicate the last discarded COUNT value. For the latter, the receiving PDCP entity considers any COUNT values prior to the last discarded COUNT value and having not been received yet upon receiving the PDCP control PDU as being discarded by the transmitting PDCP entity.

Proposal 7. RAN2 consider introducing a new PDCP control PDU, which is sent by the transmitting PDCP entity to inform the receiving PDCP entity about the COUNT value(s) discarded and hence not to be waited for.

# Conclusions

The followings are proposed:

Proposal 1. RAN2 discuss and decide whether the network indicates UE, via the dedicated signaling, a PSI threshold value directly, or indicates a congestion level and the UE maps the congestion level to a PSI thershold value, when the UL becomes congested.

Proposal 2. Upon the UE receiving the dedicated signaling, any PDU Set subsequently received from upper layer with a PSI value associated with a less importance than the importance associated with the PSI threshold value is subject to early discarding, while any PDU Set subsequently received from upper layer with a PSI value associated with a same importance as or a more importance than the importance associated with the PSI threshold value isn’t subject to the early discarding.

Proposal 3. When the UL is no longer congested, the network indicates UE, via another dedicated signaling, to stop applying the PSI-based discard mechanism.

Proposal 4. RAN2 discuss and decide whether a PDU Set subject to early discarding should be discarded upon arrival at the transmitting PDCP entity or can be discarded some time after its arrival at the transmitting PDCP entity but still earlier than the expiry of the legacy discardTimer.

Proposal 5. If RAN2 decides that the PSI-based discarded PDUs are discarded upon arrival at the transmitting PDCP entity, there is no need for introducing a second discard timer.

Proposal 6. If RAN2 decides that the the PSI-based discarded PDUs can be discarded some time after their arrival at the transmitting PDCP entity but still earlier than the expiry of the legacy discardTimer, introduce a second discard timer, which is started with a non-zero and potentially PSI-dependent initial timer value configured by the RRC signaling for a PDU Set that is subject to early discarding due to a cogestion condition and its PSI value. Meanwhile, the legacy discardTimer is also started with the legacy timer value configued for the PDCP entity, which timer value is PSI-independent. The expiry of either timer triggers the discarding of the associated PDU Set.

Proposal 7. RAN2 consider introducing a new PDCP control PDU, which is sent by the transmitting PDCP entity to inform the receiving PDCP entity about the COUNT value(s) discarded and hence not to be waited for.

# References

R2-2304202, RAN2#121-bis Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia).

R2-2306542, RAN2#122 Report on LTE MUSIM QoE XR, Vice Chairman (Nokia).

R2-2306106, Further discussions on discard operation for XR, Futurewei.