3GPP TSG-RAN2#123	R2-2307806

Toulouse, 21th Aug – 25th Aug, 2023

Agenda item:	7.20.3 (NR_MIMO_evo_DL_UL-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on impact of multi-TRP on MAC CE

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made some agreements for unified TCI state activation/deactivation MAC CE for S-DCI based mTRP operation.

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode.

In this contribution, we present our view on the further discussion point for unified TCI state A/D MAC CE.

2.	Discussion

In legacy, for joint TCI mode, one joint TCI state is activated per TCI codepoint, and for separate UL/DL TCI mode, one TCI state for each UL/DL is activated per TCI codepoint. The maximum number of TCI codepoint is 8. Unified TCI State A/D MAC CE is designed based on this.

In RAN1#112bis, for S-DCI based mTRP operation, RAN1 agreed the unified TCI framework as follows.

Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

Based on above RAN1 agreement, following requirements should be considered for MAC CE for TCI framework extension.

- With joint TCI mode, for full-set TCI state, i.e. for both TRP, two joint TCI states are activated for a TCI codepoint, and for sub-set TCI state, i.e. for 1st or 2nd TRP, one or two joint TCI states can be activated for a TCI codepoint, i.e. partially activated.

- With separate TCI mode, for full-set TCI state, i.e. for both TRP, two TCI states for each UL/DL are activated for a TCI codepoint, and for sub-set TCI state, i.e. for 1st or 2nd TRP, up to two TCI states for each UL/DL are partially activated for a TCI codepoint.

Currently, Unified TCI state A/D MAC CE supports up to 2 TCI states per TCI codepoint, while up to 4 TCI states per TCI codepoint is required for separate TCI mode. Thus, RAN2 should consider the new MAC CE supporting up to 4 TCI states per TCI codepoint for separate TCI mode.

Note that regarding joint TCI mode, RAN2 already agreed that the new MAC CE can indicate up to 2 TCI states per TCI codepoint for joint TCI mode.

Proposal 1. For separate TCI mode, the new MAC CE can indicate 4 TCI states per TCI codepoint.

In addition, TCI state can be activated partially, so the new MAC CE needs to indicate that TCI state is partially activated for a TCI codepoint and which sub-set of TCI states are activated for all combinations of sub-set, as in following table. In order to indicate which sub-set of TCI state is activated, 4 bits field is needed per TCI codepoint.

| No. | 1st UL TCI state | 1st DL TCI state | 2nd UL TCI state | 2nd DL TCI state |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 |
| 2 | 1 | 0 | 0 | 0 |
| 3 | 0 | 1 | 0 | 0 |
| 4 | 0 | 0 | 1 | 0 |
| 5 | 0 | 0 | 0 | 1 |
| 6 | 1 | 1 | 0 | 0 |
| 7 | 1 | 0 | 1 | 0 |
| 8 | 1 | 0 | 0 | 1 |
| 9 | 0 | 1 | 1 | 0 |
| 10 | 0 | 1 | 0 | 1 |
| 11 | 0 | 0 | 1 | 1 |
| 12 | 1 | 1 | 1 | 0 |
| 13 | 1 | 1 | 0 | 1 |
| 14 | 1 | 0 | 1 | 1 |
| 15 | 0 | 1 | 1 | 1 |
| 16 | 1 | 1 | 1 | 1 |

Table 1. Example of TCI state A/D in separate TCI mode

Some companies think that it is enough to indicate how many TCI state are activated and 2 bits field is introduced. However, according to RAN1 agreement, any sub set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint by the new MAC CE. Thus, 2 bit field is not enough.

Proposal 2. For separate TCI mode, the new MAC CE can indicate that TCI state is partially activated for a TCI codepoint and which sub-set of TCI state is activated.

Proposal 3. For separate TCI mode, 4 bits field is introduced per a TCI codepoint to indicate which sub-set of TCI state is activated,

Considering agreements for joint TCI mode in the last meeting and our proposal for separate TCI mode, one example is shown as following.

Figure 1. Example of new Unified TCI State A/D MAC CE

Ti,j field indicates which sub-set of TCI state is included.

In the separate TCI mode, i is related to the index of TCI codepoint and j is related to first and second of DL/UL TCI state, e.g. j=1 is for first UL, j=2 is for first DL, j=3 is for second UL and j=4 is for second DL. If this serving cell is configured with the separate TCI state mode and if T1,1 and T2,3 is set to 1, first UL TCI state of TCI codepoint#1 and second UL TCI state codepoint#2 is included.

In the joint TCI mode, i is related to the index of TCI codepoint and j is related to first and second of joint TCI state, e.g. j=1 is for first joint TCI state and j=2 is for second joint TCI state. j=3,4 are ignored in the joint TCI mode.

We propose that the above example considers as a baseline for the new MAC CE.

Proposal 4. RAN2 consider following format as a baseline for new Unified TCI State A/D MAC CE.

3.	Conclusion

In this document, we present our view on impact of multi-TRP on MAC CE, and made observations and proposals as follows.

Proposal 1. For separate TCI mode, the new MAC CE can indicate 4 TCI states per TCI codepoint.

Proposal 2. For separate TCI mode, the new MAC CE can indicate that TCI state is partially activated for a TCI codepoint and which sub-set of TCI state is activated.

Proposal 3. For separate TCI mode, 4 bits field is introduced per a TCI codepoint to indicate which sub-set of TCI state is activated,

Proposal 4. RAN2 consider following format as a baseline for new Unified TCI State A/D MAC CE.

4.	Reference