# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307810
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.3.3

Source:	Apple

Title:	Further discussion on RAN2 work of inter-band SSB-less CA

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on inter-band SSB-less CA is copied below:

In RAN2#122 [2], it was agreed that the RRC signaling and UE capability of intra-band CA can be considered as baseline, and RAN2 will study its specification impacts.

Agreements:

- If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

- If RAN4 concludes it is feasible, RAN2 can further work on at leaest the following specification impacts:

- RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

- UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

- Agreements

  - Continue RAN4 work on the following SSB-less SCell scenarios

    - Scenario 1: SCell without SSB transmission and with TRS transmission

    - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side

      - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective.

  - Deprioritize RAN4 work on the following SSB-less SCell scenario

    - Scenario 2: SCell without SSB transmission and without TRS transmission

  - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Meanwhile in RAN4#107 [3], agreement on scenarios was agreed.

In this contribution, we share our view on below aspects:

- Scenario

- RRC configuration to obtain timing reference

- SCell activation procedure

# 2 Discussion

## 2.1 Scenario

In RAN4 agreement on scenario 2a, there is one Note captured that "No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective."

    - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side

      - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective.

We understand this is a conditional agreement (i.e. if RAN1 conclude any specification change, RAN4 will not consider this scenario). Thus, the correct working procedure is that RAN4 should first send LS to ask RAN1 input. Then, RAN4 will determine whether to define requirement based on RAN1 response. Only if RAN4 has concrete conclusion to specify requirements of Scenario 2a, RAN2 can consider to start work on Scenario 2a.

Observation 1: RAN4 agreement on scenario 2a is an agreement with condition that RAN1 confirm no specification impact. Thus, the correct working procedure is that RAN4 should first send LS to ask RAN1 input. Then, RAN4 will determine whether to define requirement based on RAN1 response. Only if RAN4 has concrete conclusion to specify requirements of Scenario 2a, RAN2 can consider to start work on Scenario 2a.

So, we propose RAN2 focus on Scenario 1 in Rel-18 unless RAN4 has concrete conclusion to specify requirements of Scenario 2a.

Proposal 1: RAN2 focus on Scenario 1 in Rel-18 unless RAN4 has concrete conclusion to specify requirements of Scenario 2a.

## 2.2 RRC configuration to obtain timing reference

Intra-band SSB-less CA was specified in NR Rel-15. RAN1/RAN4 first discussed and concluded its timing issue, and then RAN2 introduced below specification changes on RRC signaling for timing reference:

- TS 38.331: There is no explicit indication on which cell for timing acquisition. Instead, in field description of IE absoluteFrequencySSB, capture RAN1/RAN4 conclusion that timing reference of SSB-less SCell is from the SpCell or an SCell of its serving cell group.

Observation 2: In TS 38.331, there is no explicit indication on which cell for timing acquisition of target SSB-less CA. Instead, the field description of IE absoluteFrequencySSB captures an assumption that timing reference of SSB-less SCell is from the SpCell or an SCell of its serving cell group.

Meanwhile TS 38.133 [4] also captured the below requirements to acquire timing reference for intra-band SSB-less CA:

-	If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

According to RAN2 and RAN4 specifications, we can see the timing assumption implies the UE can always assume co-located deployment for intra-band CA. Thus, it can use any activated serving cell (irrespective of PCell, PSCell or SCell) which is on intra-band contiguous CC to obtain the timing of target SSB-less SCC.

Observation 3: According to RAN2 and RAN4 specifications, the timing assumption implies the UE can always assume co-located deployment for intra-band CA. Thus, it can use any activated serving cell (irrespective of PCell, PSCell or SCell) which is on intra-band contiguous CC to obtain the timing of target SSB-less SCC.

However, in inter-band case, it is possible that there are multiple inter-band CCs to the target SSB-less SCell. In this case, the assumption of always co-located deployment in intra-band CA is not valid anymore. For example, one possible deployment is that PCell and SSB-less cell are inter-band CA while another SCell with SSB and SSB-less cell are intra-band CA. If the PCell and the SCell with SSB have timing difference, the UE may be confused which one is its timing reference.

Observation 4: In inter-band CA, there may be multiple inter-band CCs to the target SSB-less SCell. In this case, the assumption of always co-located deployment in intra-band SSB-less CA is not valid anymore.

Thus, we think an indication needs to be introduced to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. The details of the signaling can be FFS.

Proposal 2: An indication needs to be introduced to indicate which inter-band active serving cell can be used as timing source for the SSB-less SCell. FFS signaling details.

## 2.3 SCell activation procedure

As mentioned in Proposal 1, we only study scenario 1:

    - Scenario 1: SCell without SSB transmission and with TRS transmission

From RAN2 perspective, TRS based fast SCell activation was already introduced in Rel-17. The RAN2 specification impacts include TS 38.331 and TS 38.321:

- TS 38.331: TRS is configured by RRC signaling.

CSI-MeasConfig ::= SEQUENCE { [omitted] [[ sCellActivationRS-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17 OPTIONAL, -- Need N sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL -- Need N ]] } SCellActivationRS-Config-r17 ::= SEQUENCE { scellActivationRS-Id-r17 SCellActivationRS-ConfigId-r17, resourceSet-r17 NZP-CSI-RS-ResourceSetId, gapBetweenBursts-r17 INTEGER (2..31) OPTIONAL, -- Need R qcl-Info-r17 TCI-StateId, ... }

- TS 38.321: fast TRS base SCell activation is triggered by MAC-CE.

The MAC entity shall for each configured SCell: 1> if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell: 2> if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is indicated for this SCell: 3> indicate to lower layers the information regarding the TRS. 2> if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or 2> if the SCell is configured with sCellState set to activated upon SCell configuration: [omitted]

It looks like current RAN2 framework can already support scenario 1. So, if RAN4 conclude scenario 1 is feasible, we think above RRC and MAC spec can be reused to scenario 1, i.e., TRS of SSB-less SCell is configured by RRC signalling as in Rel-17, and the UE behaviour of SCell activation captured in Section 5.9 of TS 38.321 can be reused.

Proposal 3: If RAN4 conclude scenario 1 is feasible, TRS of SSB-less SCell is configured by RRC signalling as in Rel-17, and the UE behavior of SCell activation captured in Section 5.9 of TS 38.321 can be reused.

# 3 Conclusion

In this contribution, we further discuss RAN2 work of inter-band SSB-less CA. Our observations are:

Observation 1: RAN4 agreement on scenario 2a is an agreement with condition that RAN1 confirm no specification impact. Thus, the correct working procedure is that RAN4 should first send LS to ask RAN1 input. Then, RAN4 will determine whether to define requirement based on RAN1 response. Only if RAN4 has concrete conclusion to specify requirements of Scenario 2a, RAN2 can consider to start work on Scenario 2a.

Observation 2: In TS 38.331, there is no explicit indication on which cell for timing acquisition of target SSB-less CA. Instead, the field description of IE absoluteFrequencySSB captures an assumption that timing reference of SSB-less SCell is from the SpCell or an SCell of its serving cell group.

Observation 3: According to RAN2 and RAN4 specifications, the timing assumption implies the UE can always assume co-located deployment for intra-band CA. Thus, it can use any activated serving cell (irrespective of PCell, PSCell or SCell) which is on intra-band contiguous CC to obtain the timing of target SSB-less SCC.

Observation 4: In inter-band CA, there may be multiple inter-band CCs to the target SSB-less SCell. In this case, the assumption of always co-located deployment in intra-band SSB-less CA is not valid anymore.

Based on observations, our proposals can be found below.

Proposal 1: RAN2 focus on Scenario 1 in Rel-18 unless RAN4 has concrete conclusion to specify requirements of Scenario 2a.

Proposal 2: An indication needs to be introduced to indicate which inter-band active serving cell can be used as timing source for the SSB-less SCell. FFS signaling details.

Proposal 3: If RAN4 conclude scenario 1 is feasible, TRS of SSB-less SCell is configured by RRC signalling as in Rel-17, and the UE behavior of SCell activation captured in Section 5.9 of TS 38.321 can be reused.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#122, Chair Notes.

[3] R4-2310086, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon