# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307819
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.15.6

Source:	Apple

Title:	Further discussion on Sidelink CA

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN#99, SL CA was agreed to start WG work with revised scope (RP-230077 [1]). In RAN2#121b-e [2] and RAN2#122 [3], below agreements on SL CA were made:

CA configuration for GC/BC

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

CA configuration for UC

Agreement:

Proposal 1: Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions.

Meanwhile, one LS [4] is sent to SA2 with two questions related to the procedure after unicast PC5 link is established.

CA duplication

1:	Agreement made for GC/BC (RAN2#121bis-e) is also applicable for UC.

2:  SLRB configures whether PDCP duplication is used or not

3:	Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

DTX based SL RLF in SL CA

Agreement on DTX based SL RLF in SL CA

1:	The counting is calculated per carrier.

2:	Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

In this contribution, we further discuss SL CA from below aspects:

- Carrier mapping for UC SL DRB

- CA duplication of SL SRB

- DTX based SL RLF

# 2 Discussion

## 2.1 Carrier mapping for UC SL DRB

In RAN2#121b-e [2], how to provide CA configuration for SL UC was discussed (e.g. whether V2X layer or AS layer can provide the carrier configuration). Finally, one LS [4] is sent to SA2 with two questions related to the procedure after unicast PC5 link is established.

SA2 has provided their response in S2-2307794 [5]:

- Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies?

- Answer 1 with question for clarification:

According to discussion in SA2, most companies' understanding is Question 1 is about how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment procedure. Anyhow some companies think that Question 1 is related to the scenario after the PC5 unicast link is established. Therefore, SA2 would like to ask RAN2 about what the scenario Question 1 is related to.

SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer, however there was no corresponding CR approved yet in SA2.

- Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer?

- Answer 2: Based on the following operations as described in clauses 5.4.1.1.3 and 6.3.3 of TS 23.287, it is considered that the UE can ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer.

i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc.

ii) The V2X layer updates the AS layer about PC5 QoS Flow addition/modification/removal for the established PC5 unicast link. For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)").

iii) The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

We discuss RAN2 impacts for SA2's answer 1 and answer 2, respectively.

### 2.1.1 Discussion on SA2 answer 1

On Question 1, RAN2's original intention is the procedure after PC5-S link establishment procedure. But since there are only two RAN2 meetings in Rel-18, we think RAN2 can also provide RAN2's view in the reply LS. In our understanding, the UE may maintain a mapping between old L2 ID (before PC5 link establishment) and new L2 ID (after PC5 link establishment) by its implementation. And this understanding is aligned with below statement in SA2 reply LS:

i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc.

Thus, we propose to reply SA2 to inform above understanding.

Proposal 1: On Question 1 of S2-2307794, reply SA2 "RAN2's question 1 is intended to discuss after PC5 link establishment. And RAN2 assume that the UE may maintain a mapping between old L2 ID (before PC5 link establishment) and new L2 ID (after PC5 link establishment) by its implementation".

### 2.1.2 Discussion on SA2 answer 2

In SA2's answer to Question 2, a new concept of "mapping between QoS flow(s) and frequencies" (instead of "mapping between services and frequencies" in LTE SC CA) is mentioned by SA2.

- Answer 2: Based on the following operations as described in clauses 5.4.1.1.3 and 6.3.3 of TS 23.287, it is considered that the UE can ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer.

i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc.

ii) The V2X layer updates the AS layer about PC5 QoS Flow addition/modification/removal for the established PC5 unicast link. For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)").

iii) The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

As we know, 5G introduced a new QoS framework based on QoS flow mapping. Thus, we think that this new concept of "mapping between QoS flow(s) and frequencies" is reasonable.

Observation 1: In SA2's answer to Question 2, a new concept of "mapping between QoS flow(s) and frequencies" (instead of "mapping between services and frequencies" in LTE SC CA) is mentioned. This is reasonable because 5G introduced a new QoS framework based on QoS flow mapping.

Thus, we propose RAN2 to implement this new concept for NR UC SL CA.

Proposal 2: For NR UC SL CA, RAN2 implement the mapping from QoS flow(s) to frequencies from V2X layer, and notify SA2.

However, as illustrated in Figure. 1, one issue is that QoS flow(s) are not visible to the UE's MAC layer. Instead, only LCH is visible to MAC layer:

- In current specification, the QoS flow(s) to be used in one LCH is configured via IE SL-SDAP-Config in RRC signaling by NW or in pre-configuration.

- MAC layer has no idea of which QoS flows are feeding the data in one LCH buffer, so it may not be able to determine the allowed frequency subset to be used for a particular LCH.

  - Please note that RAN2 discussed multiple times in NR Rel-15: MAC layer are not supposed to read SDAP header in specified UP procedure, although it can be an UE implementation option.

Observation 2: In current specification, only LCH is visible to MAC layer. MAC layer has no idea of which QoS flows are feeding the data in one LCH buffer, so it may not be able to determine the allowed frequency subset to be used for a particular LCH.

Figure. 1 Illustration of issue for MAC layer to implement QoS flows to frequencies mapping

Thus, we need a solution to ensure the QoS flow(s) are not delivered to a wrong carrier which is not allowed by V2X layer. In other words, for a MAC PDU corresponding to a QoS flow:

- The carrier(s) selected must be part of carriers allowed by V2X layer for this QoS flow

- The carrier(s) selected must be part of carriers allowed by AS layer for SL CA operation.

  - In current specification, SIB is allowed to provide a list of frequency (SL-ConfigCommonNR-r16-> sl-FreqInfoList-r16), but PC5 RRC doesn’t provide the allowed frequency list.

Considering there are two remaining RAN2 meetings for SL CA in Rel-18, we proposed two solutions for RAN2 discussion:

- Solution 1: AS layer generates a subset of carriers among all QoS flows (i.e. “allowed SL carriers”) based on QoS flows to frequencies mapping from V2X layer.

  - It may scarify SL CA performance, but minimize AS spec impacts.

- Solution 2: AS layer rely on LCP restriction to ensure the correct carrier(s) are used for one MAC PDU

  - It fully enjoys SL CA benefit, but will bring significant AS spec impacts

We provide details and comparison of these two solutions in Table 1.

According to Table 1, Solution 1 and Solution 2 have different Pros and Cons. We don't have clear preference, and thereby propose RAN2 to down-select between them.

Proposal 3: On how to implement the mapping from QoS flows to frequencies for UC SL CA, RAN2 down-select between the following 2 solutions with Table 1 into consideration.

- Solution 1: AS layer generates a subset of carriers among all QoS flows (i.e. “allowed SL carriers”) based on all mappings from QoS flows to frequencies from V2X layer.

- Solution 2: AS layer rely on LCP restriction to ensure the correct carrier(s) are used for one MAC PDU.

Meanwhile, notice that both Solution 1 and solution 2 require TX UE to configure AllowedFrequencies for SL DRB via PC5 RRC after capability exchange. To make progress, we propose:

Proposal 4: Irrespective of which solution to adopt, TX UE configures AllowedFrequencies for SL DRB via PC5 RRC after capability exchange.

|  | Solution 1 | Solution 2 |
| --- | --- | --- |
| Solution details | - V2X layer provides QoS flow to carrier mapping to AS layer. - AS layer generates a subset of carriers among all QoS flows (i.e. “allowed SL carriers”). - AS layer assumes all those QoS flows are to be supported simultaneously. - The subset of carriers is the intersection of mapped carrier sets of all possible QoS flow(s) - An example is provided in right figure: - V2X layer provides: Flow1-><f1,f2,f3,f4>, Flow2-><f2,f3, f4>, Flow3-><f3,f4,f5> Flow4-> <f3,f4>. - Then, “allowed SL carrier” is <f3,f4> - “Allowed SL carriers” can be empty if no carrier is within the set. Then, SL CA is not applied. | - V2X layer provides QoS flow to carrier mapping to AS layer. - AS layer derives the mapped frequencies of one SLRB and applies LCP restriction on the mapped frequencies. |
| RAN2 spec impacts | - TS 38.321 and TS 38.300: - Capture the procedure of how TX UE generates "allowed SL carriers". - TS 38.331: - TX UE notifies the RX UE on the “allowed SL carriers” (e.g. AllowedFrequencies) via PC5 RRC after capability exchange. - To avoid misaligned understanding of the "allowed SL carriers" between UE and NW, TX UE in RRC_CONNECTED state notifies NW on the “allowed SL carriers” via SUI. | - TS 38.321 and TS 38.300: - Capture the procedure of how TX UE derives the mapped frequencies for one SLRB. - Capture the new LCP restriction procedure: The TX UE only maps the configured LCH(s) to the configured set of frequencies. - TS 38.331: - Uu RRC: introduce a frequency list (e.g. AllowedFrequencies) in SL-LogicalChannelConfig of one LCH - PC5 RRC: Introduce a frequency list associated with SLRB index in RRCReconfigurationSidelink - To avoid misaligned understanding between UE and NW, TX UE in RRC_CONNECTED state notifies NW on QoS flows to frequencies mapping from V2X layer via SUI. - Special handling on default SLRB is needed. FFS details. |
| Pros / Cons | Pros: minimize spec impacts; Cons: may scarify SL CA performance in some case. | Pros: fully enjoy SL CA benefit; Cons: significant AS spec impacts. |

Table.1 Details and comparison between Solution 1 and Solution 2

## 2.2 CA duplication of SL SRB

### 2.2.1 Before PC5 link establishment

The issue is whether SL CA is applied to initial DCR before PC5 link establishment. It was discussed in RAN2#122 [3], but was not concluded. The related note can be found below:

Proposal 16: If SL CA for unicast is supported, RAN2 to confirm that there is backwards compatible issue for DCR transmission in case of Rel-18 a SL TX UE initiating establishment with a Rel-16/Rel-17 SL RX UE.

[Apple]: We don’t need to wait for SA2 response LS for RAN2 discussion. [Ericsson]: Don’t agree with Apple. Better to wait for SA2 guideline. [OPPO, IDC]: If there is backwards compatible issue for DCR transmission in SL CA manner, we should avoid this problem, i.e. SL CA is not applied to initial DCR before SL link is established. [Qualcomm]: The solution would be related to our previous discussion for GC/BC. [LG]: SA2 already discussed backwards compatible issue for DCR so better to wait for SA2 response LS.

- Wait for SA2 response LS.

Although it is theoretically feasible, we prefer not to apply CA (i.e. only single carrier) to initial PC5-S signaling due to below reasonings:

- If CA for initial PC5-S signaling can be used, it will incur legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- If CA for initial PC5-S signaling can be used, the UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Observation 3: If CA for initial PC5-S signaling can be used, it will incur at least below issues:

- Legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Thus, we propose not to apply CA to initial PC5-S signaling.

Proposal 5: SL CA is not applied to initial PC5-S signaling.

Then, one followed question is how TX and RX UE align with one single carrier for initial PC5-S signaling. Note that it includes two cases:

- Case 1: Rel-18 TX UE establishment with Rel-18 RX UE.

- Case 2: Rel-18 TX UE establishment with Rel-16/Rel-17 RX UE (backward combability issue).

For case 2, legacy Rel-16/Rel-17 UE use sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR to determine the frequency for initial PC5-S signaling. And the frequency list sl-FreqInfoList-r16 is limited to a single carrier by Rel-17.

SL-ConfigCommonNR-r16 ::=        SEQUENCE {

sl-FreqInfoList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16      OPTIONAL,    -- Need R

[…]

}

sl-FreqInfoList This field indicates the NR sidelink communication configuration on some carrier frequency (ies). In this release, only one entry can be configured in the list.

SidelinkPreconfigNR-r16 ::=                 SEQUENCE {

sl-PreconfigFreqInfoList-r16                SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16     OPTIONAL,

[…]

}

sl-PreconfigFreqInfoList This field indicates the NR sidelink communication and/ or NR sidelink discovery configuration some carrier frequency(ies). In this release, only one SL-FreqConfig can be configured in the list.

Observation 4: Legacy Rel-16/Rel-17 UE use sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR to determine the frequency for initial PC5-S signaling. And the frequency list sl-FreqInfoList-r16 is limited to a single carrier by Rel-17.

Thus, the simplest solution is to continue to restricting only single carrier can be configured in sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR, which is used by Rel-18 UEs to determine the single frequency for initial PC5-S signaling. This solution can address issues in both case 1 and case 2, and resolve the backward compatibility issue.

Proposal 6: To align carrier for initial PC5-S signaling with Rel-16/Rel-17/Rel-18 RX UEs, Rel-18 TX UEs use sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR which continues to be restricted to single carrier in Rel-18.

### 2.2.2 After PC5 link establishment

RAN2#122 discussed this issue. One WA and FFS were captured in Chair Notes [3].

1:	Agreement made for GC/BC (RAN2#121bis-e) is also applicable for UC.

2:  SLRB configures whether PDCP duplication is used or not

3:	Working assumption: SL CA/PDCP duplication is applied to PC5-RRC after SL link is established. FFS on exact time when it can be started.

We prefer to confirm this WA. With regarding to the FFS, we think the same solutions as SL DRB in Proposal 3 can be reused, i.e. TX UE configures AllowedFrequencies for SL SRB3 via PC5 RRC after capability exchange.

Proposal 7: Confirm the WA that CA duplication is applied to PC5-RRC after SL is established. TX UE configures AllowedFrequencies via PC5 RRC after capability exchange, and starts duplication after reception of RRCReconfigurationCompleteSidelink message.

## 2.3 DTX based SL RLF

In RAN2#122 [3], per-carrier DTX counting was agreed. However, if counting in one carrier is reached to threshold, the followed UE behavior is not clear. We discuss this issue in this section.

Agreement on DTX based SL RLF in SL CA

1:	The counting is calculated per carrier.

2:	Legacy SL RLF is not declared when the counting is reached to sl-MaxnumConsecutiveDTX) for carrier(s) and the UE has other available SL carrier(s) for SL CA.

First, we think there are the following 2 cases for one SL carrier:

- Case 1: Only SL DRB(s) are configured in the carrier

- Case 2: SL SRB3 is configured in the carrier

We think Case 1 and Case 2 should be treated differently because the TX UE can reconfigure the SL DRB via PC5 RRC reconfiguration procedure performed in available carrier with SL SRB3 (e.g. release the concerned carrier).

Observation 5: When counting DTX in one carrier is reached to threshold, if only SL DRB(s) are configured in this carrier, the TX UE can reconfigure the SL DRB via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3 (e.g. release the concerned carrier or replace it with a new carrier).

On Case 2, if CA duplication is applied according to our proposal 7, it can be further divided into below two sub-cases:

- Case 2a: Counting DTX is reached to threshold in all carriers configured with SL SRB3

- Case 2b: Counting DTX is reached to threshold only in a subset of carriers configured with SL SRB3

For Case 2b, because there are still carrier(s) available for SL SRB3 exchange, we think the same treatment as Case 1 can be reused, i.e. the TX UE can reconfigure duplication of SL SRB3 via PC5 RRC reconfiguration procedure performed in available carrier with SL SRB3 (e.g. release the concerned carrier or replace it with a new carrier).

Observation 6: When counting DTX in one carrier with SL SRB 3 is reached to threshold, if there are still carrier(s) available for SL SRB3 exchange, the TX UE can reconfigure duplication of SL SRB3 via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3 (e.g. release the concerned carrier or replace it with a new carrier).

Finally, for case 2a, we think SL RLF will be declared and legacy procedure is followed (i.e. TX UE triggers upper layer to run a keep-alive check for the failed carrier).

Based on above discussion, we propose:

Proposal 8: When counting DTX in one carrier is reached to threshold (sl-MaxnumConsecutiveDTX), the TX UE takes different actions for below 3 cases:

- Case 1: Only SL DRB(s) are configured in the carrier.

  - The TX UE may reconfigure the SL DRB via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3.

- Case 2a: SL SRB3 is configured in the carrier but there are other carrier(s) with SL SRB3 available.

  - The TX UE may reconfigure duplication of SL SRB3 via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3.

- Case 2b: All carrier(s) configured with SL SRB3 are reaching to threshold

  - SL RLF is declared and legacy procedure is followed (i.e. TX UE triggers upper layer to run a keep-alive check for the failed carrier.

# 3 Conclusion

In this contribution, we further discuss SL CA. Our observations are:

Observation 1: In SA2's answer to Question 2, a new concept of "mapping between QoS flow(s) and frequencies" (instead of "mapping between services and frequencies" in LTE SC CA) is mentioned. This is reasonable because 5G introduced a new QoS framework based on QoS flow mapping.

Observation 2: In current specification, only LCH is visible to MAC layer. MAC layer has no idea of which QoS flows are feeding the data in one LCH buffer, so it may not be able to determine the allowed frequency subset to be used for a particular LCH.

Observation 3: If CA for initial PC5-S signaling can be used, it will incur at least below issues:

- Legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Observation 4: Legacy Rel-16/Rel-17 UE use sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR to determine the frequency for initial PC5-S signaling. And the frequency list sl-FreqInfoList-r16 is limited to a single carrier by Rel-17.

Observation 5: When counting DTX in one carrier is reached to threshold, if only SL DRB(s) are configured in this carrier, the TX UE can reconfigure the SL DRB via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3 (e.g. release the concerned carrier or replace it with a new carrier).

Observation 6: When counting DTX in one carrier with SL SRB 3 is reached to threshold, if there are still carrier(s) available for SL SRB3 exchange, the TX UE can reconfigure duplication of SL SRB3 via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3 (e.g. release the concerned carrier or replace it with a new carrier).

Based on observations, our proposals are:

CA configuration for UC SL DRB

Proposal 1: On Question 1 of S2-2307794, reply SA2 "RAN2's question 1 is intended to discuss after PC5 link establishment. And RAN2 assume that the UE may maintain a mapping between old L2 ID (before PC5 link establishment) and new L2 ID (after PC5 link establishment) by its implementation".

Proposal 2: For NR UC SL CA, RAN2 implement the mapping from QoS flow(s) to frequencies from V2X layer, and notify SA2.

Proposal 3: On how to implement the mapping from QoS flows to frequencies for UC SL CA, RAN2 down-select between the following 2 solutions with Table 1 into consideration.

- Solution 1: AS layer generates a subset of carriers among all QoS flows (i.e. “allowed SL carriers”), based on all mappings from QoS flows to frequencies from V2X layer.

- Solution 2: AS layer rely on LCP restriction to ensure the correct carrier(s) are used for one MAC PDU.

Proposal 4: Irrespective of which solution to adopt, TX UE configures AllowedFrequencies for SL DRB via PC5 RRC after capability exchange.

CA duplication for SL SRB

Proposal 5: SL CA is not applied to initial PC5-S signaling.

Proposal 6: To align carrier for initial PC5-S signaling with Rel-16/Rel-17/Rel-18 RX UEs, Rel-18 TX UEs use sl-FreqInfoList-r16 configured in SIB12 or SL- PreconfigurationNR which continues to be restricted to single carrier in Rel-18.

Proposal 7: Confirm the WA that CA duplication is applied to PC5-RRC after SL is established. TX UE configures AllowedFrequencies via PC5 RRC after capability exchange, and starts duplication after reception of RRCReconfigurationCompleteSidelink message.

DTX based SL RLF

Proposal 8: When counting DTX in one carrier is reached to threshold (sl-MaxnumConsecutiveDTX), the TX UE takes different actions for below 3 cases:

- Case 1: Only SL DRB(s) are configured in the carrier.

  - The TX UE may reconfigure the SL DRB via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3.

- Case 2a: SL SRB3 is configured in the carrier but there are other carrier(s) with SL SRB3 available.

  - The TX UE may reconfigure duplication of SL SRB3 via PC5 RRC reconfiguration procedure performed in another available carrier with SL SRB3.

- Case 2b: All carrier(s) configured with SL SRB3 are reaching to threshold

  - SL RLF is declared and legacy procedure is followed (i.e. TX UE triggers upper layer to run a keep-alive check for the failed carrier.

# 4 References

[1] 3GPP RP-230077, WID revision: NR sidelink evolution, OPPO

[2] RAN2#121b-e, Chair Notes.

[3] RAN2#122, Chair Notes.

[4] R2-2304236, LS on carrier mapping for unicast SL CA, Apple.

[5] S2-2307794, Reply LS on carrier mapping for unicast SL CA, LG.