# 3GPP TSG RAN WG2 Meeting #123      	                           R2-2307821
Toulouse, France, Aug 21- Aug 25

Agenda item:	7.12.2.2

Source:	Apple

Title:	Discussion on IDLE/INACTIVE UE mobility enhancement

WID/SID:	NR_mobile_IAB-Core – Release 18

Document for:	Discussion and Decision

# 1 Introduction

WID of mobile IAB (RP-213601) was agreed in RAN#94e [1]. The related WID objectives on mobility enhancement are summarized below.

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

The following principles should be respected:

- Mobile IAB-nodes should be able to serve legacy UEs.

- Solutions providing optimization for Mobile IAB may entail Rel-18 UE enhancements, provided that such enhancements are backwards compatible

For IDLE / INACTIVE UE mobility enhancement, cell reselection enhancement and IDLE / INACTIVE UE handling during full migration were discussed in RAN2#121 [2], RAN2#121b-e [3] and RAN2#122 [4].

Cell (re)selection

Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection.

FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

IDLE / INACTIVE UE handling during full migration

R2-2303945	Enhancements for IAB-node mobility and onboard UEs	AT&T	discussion

noted, Barring or similar issue postponed

In this contribution, we discuss the remaining issues of below aspects on IDLE / INACTIVE UE mobility:

- Cell reselection enhancement

- IDLE / INACTIVE UE handling during full migration

Note that we discuss CONNECTED UE mobility in our companion contribution [5].

# 2 Discussion

## 2.1 Cell reselection enhancement

In RAN2#119b-e [6], cell reselection enhancement was discussed, and below agreements were made:

RAN2 observes that a UE could potentially consider itself on-board of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e. the UE then need to know that this is such a cell). FFS the time. FFS if this is needed.

RAN2 assumption: For the mobile IAB cell broadcasting info:

- 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known)

- FFS how this is used (might be implementation specific).

Then, in RAN2#121 [2], some progress was made on inter-frequency cell reselection. Below WA and FFS were agreed:

Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection.

FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

Finally, in RAN2#122 [4], some more agreements were achieved. Although the detailed solution is still not clear, it can effectively preclude some directions.

R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state.

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

We think the key aspect is whether to confirm the WA made in RAN2#119b-e, and the other FFSs are related details. Thus, we first share our view on the WA in inter-frequency cell reselection and intra-frequency cell reselection, respectively. And then the FFS on autonomous search was discussed. Finally, the assistance information from NW is discussed.

### 2.1.1 Inter-frequency cell reselection

According to TS 38.304 [7], the UE performs inter-frequency cell reselection based on absolute frequency priority configured by the NW. During online discussion of RAN2#121 [2], some company thought that prioritization of mobile IAB cell in case of inter-frequency cell reselection can rely on legacy mechanism, i.e. the NW can configure a higher priority value (cellReselectionPriority/cellReselectionSubPriority in SIB4) for the frequency with mobile IAB cell(s). However, this solution can't work well because surrounding UE will also prioritize that frequency.

Observation 1: Some company thought on-board UEs' prioritization of mobile IAB cell(s) in inter-frequency cell reselection can rely on legacy mechanism, i.e. NW configures a higher priority value for the frequency with mobile IAB cell(s) in SIB4. However, it can't work because surrounding UE will also prioritize that frequency.

Actually, RAN2 has discussed the similar issues for LTE/NR MBMS and V2X. And the solution is that if the UE is configured to perform V2X/MBMS, the UE may consider the frequency providing V2X/MBMS to be the highest priority. We think the similar mechanism can be reused for mobile IAB.

Observation 2: In existing TS 36.304 / TS 38.304, if the UE is configured to perform V2X/MBMS, the UE may consider the frequency providing V2X/MBMS to be the highest priority.

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

Finally, this mechanism is aligned with below agreed direction in RAN2#122 [4], especially the highlighted part:

Thus, we propose to Confirm the WA for inter-frequency cell reselection in agreed two scenarios:

Proposal 1: Confirm the WA for inter-frequency cell reselection in agreed two scenarios: For a UE that is physically on a moving vehicle, irrespective whether it is camped on the mobile IAB cell or a stationary cell, it may prioritize a mobile IAB cell with best rank in another frequency.

### 2.1.2 Intra-frequency cell reselection

In agreements of RAN2#122 [4], it is clearly stated that the prioritized cell is a highest ranked cell, and the assistance information is restricted to only inter-frequency cell list (i.e. SIB4).

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

According to TS 38.304 [7], it is legacy UE behaviour to select the best ranked cell in intra-frequency/equal-priority cell reselection:

The reason behind the specification is that best cell principle should be respected in intra-frequency cell reselection (i.e. based on radio condition). Otherwise there may be coverage issue. It is aligned with the agreement made in RAN2#122 [4].

Observation 3: In existing TS 38.304, the UE performs intra-frequency / equal-priority cell reselection to the best ranked cell to respect best cell principle (i.e. based on radio condition). It is aligned with agreement made in RAN2#122.

During the online discussion, some company had concern that the on-board UE may prioritize static cell if no enhancement for intra-frequency cell reselection. We think it can be resolved by legacy intra-frequency Qoffset solution (i.e. NW can configure different intra-frequency Qoffset in SIB3 for mobile IAB cells).

SIB3 ::=                            SEQUENCE {

intraFreqNeighCellList              IntraFreqNeighCellList                                   OPTIONAL,   -- Need R

intraFreqBlackCellList              IntraFreqBlackCellList                                   OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                               OPTIONAL,

...,

]]

}

...

IntraFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                  OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                  OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                  OPTIONAL,   -- Need R

...

}

Observation 4: Some company had concern that the on-board UE may prioritize static cell if no enhancement for intra-frequency cell reselection. However, it can be resolved by legacy solution (i.e. NW can configure different intra-frequency Qoffset in SIB3 for mobile IAB cells).

Thus, we propose to confirm that no enhancement is needed for intra-frequency / equal-priority cell reselection.

Proposal 2: No enhancement is needed for intra-frequency and equal-priority cell reselection.

### 2.1.3 Procedure of mobile cell search

In RAN2#121 [4], the below FFS is captured on whether to specify procedure of searching mobile IAB cells:

FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

This FFS was captured because below 2 alternatives were discussed during online discussion:

- Alt-1: UE performs autonomous search (based on previous visit information or acquisition of system information or other side information)

- Alt-2: SIB is extended to indicate which frequency has mobile cell(s) and possibly which cell is a mobile cell (i.e. introduce mobile IAB dedicated inter-frequency and intra-frequency cell list in SIB3/SIB4).

Please note that Alt-2 is aligned with below highlighted part of RAN2#122 agreement (i.e. inter-frequency cell list) [4]:

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

In our understanding, the concern of Alt-2 is that mobile IAB cells need to be upgraded to get the neighbour mobile cells' information and to signal the new information. However, we think it can be up to NW implementation, and no need to preclude this option from standard perspective. Thus, we think such mobile IAB dedicated inter-frequency cell list can be optional provided in SIB4.

Proposal 3: The procedure that UE searches and measure for mIAB cells on different frequencies is unspecified. As assistance information, the NW can optionally provide inter-frequency mIAB cell list in SIB4.

### 2.1.4 Misc. issues

Finally, we address the below misc. issues on whether to specify condition of mobile IAB cell prioritization and cell dwelling condition.

R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state.

First, from history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Observation 5: From history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Correspondingly, we think a solution to determine whether the UE is in a moving vehicle or not is out of 3GPP scope. Thus, it should be left to UE implementation.

Proposal 4: It is left to UE implementation to determine whether the UE is physically on a moving vehicle and when it applies mobile IAB cell prioritization for agreed scenarios.

## 2.2 IDLE / INACTIVE UE handling during full migration

In RAN2#119b-e [6], dual-DU model during full migration was agreed:

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

This dual-DU modelling implies that the source DU will be released by donor CU after completion of full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Observation 6: Dual-DU modelling implies that the source DU will be released by donor CU after full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Thus, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

Observation 7: After full migration is triggered, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

R2-2303945	Enhancements for IAB-node mobility and onboard UEs	AT&T	discussion

-	Chair think that for Idle inactive UEs, cell barring can prevent access to source cell during the procedure. AT&T think there is a risk that cell barring will bar the UE for a too long time from the freq and the UE will not find his way back to the new mIAB cell. LGE think this may not be an issue. Chair think we can discuss this next meeting, and e.g. by first collecting comments offline)

noted, Barring or similar issue postponed

This issue was already discussed in RAN2#121b-e [3], and no conclusion was made:

We think this issue is similar to the discussion on CHO enhancement. And similar solutions can be considered. To be specific, RAN2 can discuss below 3 alternatives:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

- Alt-3: via cell reselection enhancement (i.e. prioritize reselection to target DU)

Because Alt-3 doesn't support legacy UEs, we think it can be precluded. For the other two solutions, we think Alt-1 is simpler but it may bar the UEs from accessing the frequency for up to 300 seconds [7], so that the UE may have risk to access target DU if source and target DU are in the same frequency. However, this issue can be resolved by either Alt-2 (i.e. decrease transmit power of source DU) or configuration of different frequency for source DU and target DU.

Observation 8: There is some concern that Alt-1 may bar the UEs from accessing the frequency for up to 300 seconds so that the UE may have risk to access target DU in the same frequency. However, this issue can be resolved by either Alt-2 or configuration of different frequency for source DU and target DU.

Thus, we propose RAN2 confirm this issue can be resolved, and it is up to NW implementation whether to use Alt-1 or Alt-2.

Proposal 5: On the issue of how to avoid IDLE / INACTIVE UEs camping in source DU during full migration, it is up to NW implementation which below two solutions to utilize:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

# 3 Conclusion

In this contribution, we discuss IDLE/INACTIVE UE mobility enhancement in mobile IAB. Our observations are:

Observation 1: Some company thought on-board UEs' prioritization of mobile IAB cell(s) in inter-frequency cell reselection can rely on legacy mechanism, i.e. NW configures a higher priority value for the frequency with mobile IAB cell(s) in SIB4. However, it can't work because surrounding UE will also prioritize that frequency.

Observation 2: In existing TS 36.304 / TS 38.304, if the UE is configured to perform V2X/MBMS, the UE may consider the frequency providing V2X/MBMS to be the highest priority.

Observation 3: In existing TS 38.304, the UE performs intra-frequency / equal-priority cell reselection to the best ranked cell to respect best cell principle (i.e. based on radio condition). It is aligned with agreement made in RAN2#122.

Observation 4: Some company had concern that the on-board UE may prioritize static cell if no enhancement for intra-frequency cell reselection. However, it can be resolved by legacy solution (i.e. NW can configure different intra-frequency Qoffset in SIB3 for mobile IAB cells).

Observation 5: From history of 3GPP similar discussions, it was always hard to converge to a specified speed estimation solution because 3GPP can only specify air interface related solution but UE may rely on other techniques (e.g. its sensor info), which is out of 3GPP scope.

Observation 6: Dual-DU modelling implies that the source DU will be released by donor CU after full migration. Thus, the served UEs is expected to leave the source DU after full migration is triggered. However, unlike CONNECTED UEs, IDLE / INACTIVE UEs can't be indicated by NW that full migration is triggered.

Observation 7: After full migration is triggered, the IDLE / INACTIVE UEs camping in source DU should perform cell reselection, and source DU should avoid new IDLE / INACTIVE UEs camping.

Observation 8: There is some concern that Alt-1 may bar the UEs from accessing the frequency for up to 300 seconds so that the UE may have risk to access target DU in the same frequency. However, this issue can be resolved by either Alt-2 or configuration of different frequency for source DU and target DU.

Based on observations, our proposals are:

Proposal 1: Confirm the WA for inter-frequency cell reselection in agreed two scenarios: For a UE that is physically on a moving vehicle, irrespective whether it is camped on the mobile IAB cell or a stationary cell, it may prioritize a mobile IAB cell with best rank in another frequency.

Proposal 2: No enhancement is needed for intra-frequency and equal-priority cell reselection.

Proposal 3: The procedure that UE searches and measure for mIAB cells on different frequencies is unspecified. As assistance information, the NW can optionally provide inter-frequency mIAB cell list in SIB4.

Proposal 4: It is left to UE implementation to determine whether the UE is physically on a moving vehicle and when it applies mobile IAB cell prioritization for agreed scenarios.

Proposal 5: On the issue of how to avoid IDLE / INACTIVE UEs camping in source DU after inter-donor full migration is triggered, it is up to NW implementation which below two solutions to utilize:

- Alt-1: via setting cellBarred in MIB (applicable to legacy UE and Rel-18 UE)

- Alt-2: via decreasing transmit power of source DU (applicable to legacy UE and Rel-18 UE)

# 4 References

[1] RP-213601, New WID on mobile IAB.

[2] RAN2#121, Chair Notes.

[3] RAN2#121b-e, Chair Notes.

[4] RAN2#122, Chair Notes.

[5] R2-2307820, CONNECTED mobility enhancement in mobile IAB, Apple.

[6] RAN2#119b-e, Chair Notes.

[7] TS 38.304-v17.1.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state, 2022-6.