3GPP TSG-RAN WG2 Meeting #123	R2-2307837

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.5

Source:	Apple

Title:	Views on UE Capabilities for Rel-18 QoE

WID/SID:	NR_QoE_enh-Core

Document for:	Discussion / Decision

# Introduction

As we are approaching the end of Rel-18 QoE WI, it is an appropriate time to start the discussions on the UE capabilities for the support of this feature. In this paper, we present some of our views on UE capabilities for Rel-18 QoE.

# Discussions

Akin to many other features introduced by 3GPP in the earlier releases, a specific set of functionalities and features defined in a work item typically target at certain services and deployments. Apparently, different sub-features introduced in Rel-18 QoE targets to handle QoE measurements/reporting in different situations. For instance, QoE measurements in RRC-IDLE/INACTIVE states are mainly for the UEs that may receive MBS, while QoE reporting over SRB5 is only for NR-DC deployment scenarios. Hence, we do not think any of UE capabilities to be introduced in Rel-18 QoE WI needs to be mandatory. Furthermore, the backward compatibility has to be ensured. Thus, in our view, all new UE capabilities to be introduced by Rel-18 QoE WI (including QoE measurements in RRC-IDLE/INACTIVE, QoE reporting in NR-DC, and any feature to be introduced to handle Rel-17 leftover issues) should be optional.

Proposal 1: All the new UE capabilities to be introduced for different features in Rel-18 QoE are optional.

During RAN2 #122, we have touched upon some discussions relating to the AS memory size for QoE measurements in RRC-IDLE/INACTIVE states. The following agreement has been made:

Introduce AS layer minimum memory requirement for storing Rel-18 QoE reports measured in RRC_IDLE/RRC_INACTIVE. Could have larger values than in Rel-17. FFS what is the minimum size requirement capability. FFS what is the value range of the capability.

From our point of view, this is quite straightforward that we simply reuse the Rel-17 minimum memory requirement for QoE as the baseline, which means the minimum memory requirement for storing QoE measurements in the RRC-IDLE/INACTIVE state is 64KB. However, some companies think this is too small, since it is uncertain how long the UE will be staying in the RRC-IDLE/INACTIVE state, before it can return to RRC-CONNECTED state for QoE reporting, and so many QoE reports may be discarded before they can be reported to the network.

This represents a trade-off between UE implementation and minimization of QoE measurement discarding. As a compromise, we think 64KB can be kept as a baseline AS memory requirement, while we can introduce separate UE capabilities for one or more larger memory sizes such as 128KB,  256KB and so on. This allows a more flexible implementation for a UE, depending on the use cases this UE is targeting at.

Proposal 2: The minimum AS layer memory for QoE measurements storage in Rel-18 is 64Kb (same as Rel-17). Introduce new separate UE capabilities for larger AS memory sizes such as 128KB and 256KB if needed.

# Conclusions

In this contribution, we have presented some of our opinions on UE capabilities for Rel-18 QoE, with the following proposals:

Proposal 1: All the new UE capabilities to be introduced for different features in Rel-18 QoE are optional.

Proposal 2: The minimum AS layer memory for QoE measurements storage in Rel-18 is 64Kb (same as Rel-17). Introduce new separate UE capabilities for larger AS memory sizes such as 128KB and 256KB if needed.