3GPP TSG-RAN WG2 Meeting #123	R2-2307884

Toulouse, France, 21-25 August 2023

Source: 			Apple Inc

Title: 	CFRA and CG configuration aspects in LTM

Document for: 	Decision

Agenda Item: 	7.4.2.3

# Introduction

In the pre-sync call on progressing Rel-18 FeMob[1], there was an open issue on the aspect of a “common” CFRA (RACH etc) configuration for the RACH/CG resources to be used in pre-sync and in other aspects before/during the LTM switch. In this short paper, we show a rather simple location where this configuration can be given.

# Discussion

  - CFRA/CG Pre-Configuration

We paste below the way forward that was chosen for the CFRA configuration topic.

Table 2-1 Way-forward on CFRA config

While there was (and likely will be in RAN2-123) a discussion on the aspect of a “common” RACH config that would be given to the UE and would be considered as the common pool of RACH resources which include resources from all candidate LTM cells, we think there are a couple of issues with this:

- The configuring cell now needs to post-process after collecting the RACH resources from all the candidate LTM cells.

- The configuring cell also (very likely) needs to provide the indices info to all the candidate LTM cells – after compending a pool of resources.  This is needed as the candidate LTM cells need to provide at runtime, which resource is to be actually used. This can result in a hand-shake that can be avoided

- Any change (for eg., one candidate LTM cell deciding to change the RACH resources of it’s cell) can result in re-creating the list again (we acknowledge that this can be avoided by having that resource not being selected by candidate cell, but this is not fool-proof!)

On the other hand, the current running RRC CR already has a place holder which can hold the RACH and CG resources for every candidate LTM cell – this configuration is per candidate LTM cell!

We think the better place for this would in the IE EarlyUL-SyncConfig  and this is given per candidate LTM cell but outside the actual candidate configuration that UE would unpack during LTM switch - ltm-CandidateConfig-r18.

We also think that this IE can hold information about UL resources for the UE before and during LTM switch, and so not strictly for Early UL syncing purposes. So a better name would be EarlyUL-ResourceConfig .

Therefore we propose the below change as a TP:

#### – LTM-Config The IE LTM-Config is used to provide LTM candidate cell configuration. LTM-Config information element -- ASN1START -- TAG-LTM-CONFIG-START LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 OCTET STRING (CONTAINING RRCReconfiguration), OPTIONAL, -- Cond FirstLTM-Candidate ltm-CandidateToReleaseList-r18 LTM-CandidateToReleaseList-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 LTM-CandidateToAddModList-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1.. maxNrofCellsLTM-r18-plus-1) OPTIONAL, -- Cond FirstLTM-Only ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF LTM-CSI-ResourceConfig OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF LTM-CSI-ResourceConfigId OPTIONAL, -- Need N ... } Editor’s Note: FFS on whether the LTM-CandidateNoResetL2-List field should include separate reset flags for RLC, and PDCP recovery. LTM-CandidateToReleaseList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-CandidateId-r18 OPTIONAL -- Need N -- TAG-LTM-CONFIG-STOP -- ASN1STOP LTM-Config field descriptions. ltm-CandidateToAddModList List of LTM candidate cell configurations to add and/or modify.. ltm-CandidateToReleaseList List of LTM candidate cell configurations to remove.. ltm-ServingCellNoResetID This field is used by the UE to determine on whether L2 reset should be performed for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in an LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during the LTM cell switch procedure.. ltm-ReferenceConfiguration This field includes an RRCReconfiguration message used to configure a reference configuration for LTM.. Conditional Presence; Explanation. FirstLTM-Candidate; This field is mandatory present upon the first configuration of LTM-Config which includes at least one LTM candidate cell configuration where ltm-ConfigComplete is not present. Otherwise, the field is optionally present, Need M.. FirstLTM-Only; This field is mandatory present upon the first configuration of LTM-Config which includes at least one LTM candidate cell configuration. Otherwise, the field is absent, Need N.. #### – LTM-CandidateId The IE LTM-CandidateId is used to identify an LTM candidate cell configuration. LTM-CandidateId information element -- ASN1START -- TAG-LTM-CANDIDATEID-START LTM-CandidateId-r18 ::= INTEGER (1.. maxNrofCellsLTM-r18) -- TAG-LTM-CANDIDATEID-STOP -- ASN1STOP #### – LTM-CandidateToAddModList The IE LTM-CandidateToAddModList concerns a list of LTM candidate cell configurations to add or modify. LTM-CandidateToAddModList information element -- ASN1START -- TAG-LTM-CANDIDATETOADDMODLIST-START LTM-CandidateToAddModList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-Candidate-r18 LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration), OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 SetupRelease { EarlyUL-ResourceSyncConfig-r18 } OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofCellsLTM-r18) OPTIONAL, -- Need M ltm-CandidateTCI-States-ToAddModList-r18 CandidateTCI-StatesToAddModList-r18 OPTIONAL, -- Need N ltm-CandidateTCI-States-ToReleaseList-r18 CandidateTCI-StatesToReleaseList-r18 OPTIONAL, -- Need N ... } CandidateTCI-StatesToReleaseList-r18 ::= SEQUENCE (SIZE (1..FFS)) OF CandidateTCI-StatesId-r18 OPTIONAL -- Need N Editors’s Note: FFS how to indicate to the UE that RACH should be skipped when doing an LTM cell switch. -- TAG-LTM-CANDIDATETOADDMODLIST-STOP -- ASN1STOP LTM-CandidateToAddModList field descriptions. ltm-CandidateId This field indicates an LTM candidate cell configuration.. ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate cell.. ltm-ConfigComplete This field indicates whether the LTM candidate cell configuration within ltm-CandidateConfig is a complete configuration and thus the UE shall not use the LTM reference configuration within the field ltm-ReferenceConfiguration.. ltm-NoResetID This field indicate whether the UE should perform no L2 reset for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in the LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during an LTM cell switch procedure.. #### – EarlyUL-ResourceSyncConfig The IE EarlyUL-ResourceSyncConfig is used to uplink configure random access resources to be used during LTM cell switch and for the early UL synchronization procedure EarlyUL-ResourceSyncConfig information element --ASN1START --TAG-EARLYUL-SYNCCONFIG-START EarlyUL-ResourceSyncConfig ::= SEQUENCE { ra-Config FFS OPTIONAL, -- Need M cg-Config FFS OPTIONAL, -- Need M ... } Editor’s Note: FFS on what the configuration of EarlyUlSync-Config actually is (e.g., RACH-Dedicated, CFRA, or something else). Wait for more RAN1 progresses. -- TAG-EARLYUL-SYNCCONFIG-STOP -- ASN1STOP EarlyUL-ResourceSyncConfig field descriptions. ra-Config Configuration used by the UE to perform the early UL synchronization procedure.. cg-Config Configuration used by the UE to during the time of LTM switch on the target LTM cell..

Proposal 1: CFRA and CG “pre” configured resource for each LTM candidate cell, are provided per candidate LTM cell in the IE EarlyUL-SyncConfig.

Proposal 2: Change the IE name EarlyUL-SyncConfig to EarlyUL-ResourceConfig  to better reflect the purpose of the IE.

# Conclusion

Proposal 1: CFRA and CG “pre” configured resource for each LTM candidate cell, are provided per candidate LTM cell in the IE EarlyUL-SyncConfig.

Proposal 2: Change the IE name EarlyUL-SyncConfig to EarlyUL-ResourceConfig  to better reflect the purpose of the IE.

# References

[1]	   R2-23xxxx Rel-18 Mobility WI: Pre-RAN2#123 Small Group Discussion