3GPP TSG RAN WG2#123	R2-2307894

Toulouse, France, August 21-25, 2023

Agenda item:	7.7.4.2

Source: 	ITRI

Title: 	Discussion on common information of group handover

Document for:	Discussion

1	Introduction

Providing target cell parameters that are common to UE by broadcast could reduce signalling overhead for HO command. In RAN2 #122 meeting the servingCellConfigCommon is agreed as a target cell-specific parameter common to UE for HO, and a LS to RAN3 is agree to be sent to RAN3 [1].

Agreements:

- Come back to the proposal to broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling) after feedback from RAN3

- Send al LS to RAN3 asking whether, in case target cell’s servingCellConfigCommon is broadcast in the source cell (as common (C)HO signalling), the target cell’s servingCellConfigCommon can be transferred to the source cell in the inter-gNB HO case in R18

- Group handover related to P1~P4 from R2-2304736 is not supported in Rel-18..

In this contribution we propose to provide ntn-Config of the target cell as a HO parameter by broadcast.

2	Discussion

During NTN handover procedure, UE needs to perform timing advance (TA) adjustment before performing UL transmission. The information for a UE to perform TA adjustment information including the common TA parameters, ephemeris and epoch time associated with the target cell, and the information is encapsulated in target cell ntn-Config.

Observation 1: The ntn-Config of needed for UE to perform TA adjustment before initiating the first UL transmission during handover procedure.

The ntn-Config of a cell is cell-specific and common to all UE regardless of UE location and other configurations. The ntn-Config of a neighbour cell could be provided in SIB19 by broadcast for neighbour cell measurements.

Observation 2: The ntn-Config is cell specific and is common for all UE and can be broadcast by system information.

However, the target cell may not be included in the neighbour cell configured for UE to perform neighbour cell measurements. In that case, the target cell ntn-Config needs to be included in the handover configuration for the UE to perform handover to the target cell. For group handover, it would be resource efficient to configure ntn-Config of target cell by broadcast rather than by dedicated signalling.

Observation 3: It is resource efficient to provide the ntn-Config of target cell by broadcast for group handover.

While the ntn-Config of target cell is necessary for UE to perform TA adjustment during handover procedure, and it possessed the characteristic of cell-specific and common to UE, it is proposed to provide ntn-Config of the target cell by broadcast as a parameter of common handover configuration.

Proposal 1: It is proposed to provide ntn-Config of the target cell by broadcast as a common handover configuration for group handover.

3	Conclusion

The observations are:

Observation 1: The ntn-Config of target cell is needed for UE to perform TA adjustment before initiating the first UL transmission during handover procedure.

Observation 2: The ntn-Config is cell specific and is common for all UE and can be broadcast by system information.

Observation 3: It is resource efficient to provide the ntn-Config of target cell by broadcast for group handover.

The proposal from this contribution:

Proposal 1: It is proposed to provide ntn-Config of the target cell by broadcast as a common handover configuration for group handover.

4	References

[1] R2-23xxxxx	Draft report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea.