3GPP TSG RAN WG2 Meeting #123                                	R2-2307896

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	7.7.4.2

Source: 	FGI

Title: 	Discussion on soft satellite switching with PCI unchanged

Document for:	Discussion

Introduction

RAN2 sent an LS to RAN1 regarding NTN-NTN mobility in RRC_CONNECTED in Rel-18 NR NTN [1] in RAN2#121bis-e (RAN1#112bis-e).

To RAN1 ACTION: RAN2 kindly requests RAN1 to take into account the above agreement on hard satellite switching without PCI change and provide feedback if RAN1 identifies any major technical issues, and also provide feedback on the feasibility to support soft satellite switching without PCI change.

RAN1 has replied that there are not any major technical issues for hard satellite switching with PCI unchanged in RAN1#113 [2]. However, RAN1 discussed about soft satellite switching with PCI unchanged but couldn’t reach consensus on how to reply to the LS to RAN2.

Despite of the progress in RAN1, we will provide our view on the design of soft satellite switching with PCI unchanged in this contribution.

Discussions

Soft satellite switching with PCI unchanged

In soft satellite switching, there is a duration where two satellites (the source satellite SAT#1 and the target satellite SAT#2) provide coverage to the same area as illustrated in Figure 1.

Figure 1: An illustration of soft satellite switching

Frequency location of the SSBs transmitted from the target satellite

In RAN2#121bis-e, the following agreement was reached:

Agreements: 1. In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

We note that the above agreement was targeted on scenarios of hard satellite switching. For soft satellite switching, whether the SSB frequency of the source satellite and the SSB frequency of the target satellite are the same or different is discussed below.

When the coverage area of the source satellite and the coverage area of the target satellite overlap, since the PCI of the SSB of the source satellite and the PCI of the SSB of the target satellite should be the same, if the SSB frequencies are different, there will be two frequencies on which the PCIs associated with the respective SSBs are different. This case should be avoided as this would have impact to the measurement framework.

In quasi-earth-fixed cell cases, for soft satellite switching in the same SSB frequency and same gNB, satellite switching without PCI changed is supported.

Necessary information of the target satellite

When two satellites, i.e., the source satellite and the target satellite, provides coverage to (almost) the same area, given that the SSB frequencies are the same among the source satellite and the target satellite, the NW needs to multiplex the SSBs from different satellites in the time domain, but a legacy UE cannot know which satellite the SSBs are transmitted from. If the UE cannot differentiate from which satellite an SSB is being transmitted, the different delays and different Doppler shifts of the two satellite links may make the UE hard to perform measurements. It may be difficult for a UE to be aware of SSBs from another satellite given that the SSBs transmitted from different satellites may not be frame-aligned and GSCN-aligned. The UE needs to be informed by the NW about the time-domain information of the SSBs transmitted from the target satellite, e.g., in form of SSB indices.

Considering that the time-domain information of the SSBs transmitted from the target satellite is a common information for all UEs served by the source satellite (or by the source cell), we think this information can be signal via system information (e.g., SIB19) to reduce signaling overhead.

For soft satellite switching with PCI unchanged in quasi-earth-fixed cell scenario, the NW shall signal the time-domain information of the SSBs transmitted from the target satellite to the UE via system information after the target satellite starts providing coverage to the UE.

When to perform soft satellite switch

After receiving the time-domain information of the SSBs transmitted from the target satellite, the UE can consider that there is another satellite (the target satellite) providing coverage to it. The UE may also need to know the satellite assistance information, e.g., in type of NTN-Config, of the target satellite in order to perform smooth satellite switching. If UE complexity allows, the UE may maintain DL timing and frequency synchronization to the SSBs transmitted from the target satellite after receiving the time-domain information of the SSBs transmitted.

The next aspect to consider is when to switch to the target satellite. We think this can be a NW-signalled-based, a time-based mechanism:

- NW-signalled-based mechanism: the NW can signal to each UE to switch the serving satellite from the source satellite to the target satellite. Since the signaling is a switch command, which is similar to a handover command, the reliability of the signaling should be guaranteed, we think the signaling can be a MAC CE and it can be sent to the NW via the target satellite. The ACK corresponding to the PDSCH carrying the MAC CE can be regarded as RRCReconfigurationComplete message in legacy handover procedure. After receiving the ACK, the NW can consider the serving satellite switch is successful.

- Time-based mechanism: the NW can indicate to each UE a timestamp, e.g., a combination of {(hyper-SFN, if necessary), SFN, subframe number}, at/from which the UE can change its timing and frequency to the target satellite, i.e., perform soft satellite switching. After switching to the target satellite, the UE can transmit an acknowledgement to let the NW know it has switched the serving satellite to the target satellite. The acknowledgement can be a MAC CE.

For soft satellite switching with PCI unchanged in quasi-earth-fixed cell scenario, a serving satellite switch shall be explicitly indicated to the UE by the NW, e.g., via a MAC CE (like a switch command) or via a timestamp.

Conclusions

Based on the discussion, we have the following proposals:

- In quasi-earth-fixed cell cases, for soft satellite switching in the same SSB frequency and same gNB, satellite switching without PCI changed is supported.

For soft satellite switching with PCI unchanged in quasi-earth-fixed cell scenario, the NW shall signal the time-domain information of the SSBs transmitted from the target satellite to the UE via system information after the target satellite starts providing coverage to the UE.

For soft satellite switching with PCI unchanged in quasi-earth-fixed cell scenario, a serving satellite switch shall be explicitly indicated to the UE by the NW, e.g., via a MAC CE (like a switch command) or via a timestamp.

Reference

R2-2306553, Chair’s notes RAN2#121bis-e, April 17th – April 26th, 2023.

R2-2304273, LS on unchanged PCI, 3GPP RAN2 #112bis-e.