3GPP TSG-RAN WG2 #123 meeting	R2- 2307901

21st Aug–25th Aug 2023, Toulouse

Source:		NEC

Agenda item:		7.5.3	XR-specific power saving

Title:		Discussion on C-DRX enhancement for XR

Document for:		Discussion and decision

# 1. Introduction

In RAN2#121bis-e meeting, agreements [1] on DRX enhancement were made as below:

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

In RAN2#122 meeting, agreements [2] on DRX enhancement were made as below:

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications. Not use broadcast signalling for counter and reference SFN in XR.

In this contribution, we will provide our views on DRX enhancement for XR.

# 2. Discussion

2.1 Non-integer XR periodicity

DRX formula:

In RAN2#122 meeting, it was agreed that DRX cycles with rational numbers were introduced to resolve the issue of non-integer XR periodicity. Furthermore, the DRX formula should be enhanced to support both integer and non-integer DRX cycles. In [3], the following formulas (based on equivalence A modulo B = A – [ B × floor (A / B) ])) have been proposed to address the non-integer XR periodicity and SFN wraparound issue.

For long DRX cycle,

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset	(Eq1)

where n = [(H-SFN × 10240) + (SFN × 10) + subframe number]

For short DRX cycle,

floor [n – [drx-ShortCycle × floor (n / drx-ShortCycle) ] ] =

floor [drx-StartOffset – [drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle) ] ]	(Eq2)

where n = [(H-SFN × 10240) + (SFN × 10) + subframe number]

Figure 1 shows an example of DRX cycle pattern for XR traffic with 60 fps. Based on the formulas, the pattern of the DRX cycles is {17, 17, 16} ms, which will realign with XR traffic every 50ms.

Figure 1: Alignment between XR traffic and DRX cycle by the new formulas

Proposal 1: New DRX formulas (i.e., Eq1 and Eq2) can be introduced to address the non-integer XR periodicity issues.

2.2 SFN wraparound issue

SFN wraparound:

To address the SFN wraparound issue, it was proposed in RAN2#121bis-e meeting to adopt an option with a counter in DRX formula that increments at every SFN wraparound and a DRX reference SFN signalled by the network.

Although a new hyper SFN counter with a specific range (e.g., 0~99 or 0~999) can address the issue, we think it’s better to introduce hyper SFN without wraparound, that is, the hyper SFN counter keeps increasing unless DRX configuration is (re-)configured. The solution is similar to the value of Nth in CG type 1 formula, which does not cause any H-SFN wraparound issue.

Proposal 2: To address the potential hyper SFN wraparound issue, hyper SFN counter can keep increasing (i.e., without wraparound) unless DRX configuration is (re-)configured.

Reference hyper frame:

Some companies are concerned that a potential hyper SFN boundary ambiguity between the network and UE exists when RRC Reconfiguration message is re-/transmitted across hyper frames. An example is illustrated in Figure 2. If the network sends an RRC message at the end boundary of a hyper frame (e.g., H-SFN n), and due to HARQ/ARQ retransmission, UE successfully receives the RRC message in the next hyper frame (e.g., H-SFN n+1). Then the network shall use H-SFN n as the reference hyper frame, while UE will use H-SFN n+1 as the reference hyper frame, which would lead to a different understanding of start occasions of on Duration.

Figure 2: Time reference ambiguity caused by RRC signalling retransmission across hyper frame

Observation 1: Hyper SFN boundary ambiguity between the network and UE may exist when RRC Reconfiguration message is (re-)transmitted across hyper frames.

With regard to the potential hyper SFN boundary ambiguity issue, it was agreed to introduce a DRX reference SFN for the determination of reference hyper frame. For this, a new IE drx-timeReferenceSFN can be signaled via RRC message, which is similar to timeReferenceSFN in CG Type 1 formula.

If DRX configuration is (re-)configured by RRC signalling,

- A new parameter drx-timeReferenceSFN ∈ {0,512} is configured. drx-timeReferenceSFN indicates SFN used for the determination of the reference hyper frame in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the DRX configuration.

- If the RRC message with DRX configuration is sent after SFN 512 but before the next SFN 0 boundary, drx-timeReferenceSFN = 512 shall be indicated to UE. If the RRC message with DRX configuration is sent after SFN 0 but before SFN 512, then drx-timeReferenceSFN = 0 shall be indicated to UE.

- Based on drx-timeReferenceSFN, UE can determine which hyper frame the reference SFN is in, and consider the hyper frame as reference hyper frame.

Figure 3: Reference hyper frame determination without ambiguity

Figure 3 shows an example of determining the reference hyper frame, where drx-timeReferenceSFN is set to 512. The DRX configuration is successfully received by UE in H-SFN n+1, which is one hyper frame later than the initial transmission of the RRC signalling. Based on the value of drx-timeReferenceSFN, UE finds that the closest SFN with the value indicated by drx-timeReferenceSFN that precedes the reception of the DRX configuration is in H-SFN n. Hence, both the network and UE know where the reference hyper frame is (e.g., H-SFN n), and there shall be no H-SFN boundary ambiguity.

Proposal 3: drx-timeReferenceSFN with value {0, 512} can be introduced to determine the reference hyper frame of DRX configuration without hyper SFN boundary ambiguity.

After determining the reference hyper frame, MAC layer can be responsible for maintaining the hyper SFN counter value, and its initial counter value is determined as below.

- If DRX configuration with drx-timeReferenceSFN is received by UE, the hyper SFN of reference hyper frame shall be (re-)set to 0.

After the determination of reference hyper frame with hyper SFN = 0, the hyper SFN counter shall keep increasing without wraparound as we mentioned before. Figure 4 shows an example of determining the reference hyper frame with value 0 (i.e., H-SFN = 0), where drx-timeReferenceSFN is set to 512.

Figure 4: Reference hyper frame with H-SFN = 0

Proposal 4: Hyper SFN counter can be maintained by MAC layer only, and the hyper SFN of the reference hyper frame (i.e., initial counter value) is set to 0 when DRX configuration with drx-timeReferenceSFN is (re-)configured.

Solution summary:

According to the discussion above, to address the non-integer XR periodicity and SFN wraparound issues, only limited impact on the specification shall be introduced for the solution:

- Introduce new DRX formulas Eq1 and Eq2.

- New hyper frame counter without wraparound is maintained by the MAC layer, and the initial value of the counter is set to 0 when DRX configuration with drx-timeReferenceSFN is (re-)configured.

- Non-integer DRX cycle with rational number and drx-timeReferenceSFN shall be configured by RRC signalling.

# Summary

This contribution provided our analysis on candidate solutions of DRX enhancement, and we propose to list all these possible solutions and the issues each solution can solve:

Non-integer DRX cycle:

Proposal 1: New DRX formulas (i.e., Eq1 and Eq2) can be introduced to address the non-integer XR periodicity issues.

SFN wraparound:

Proposal 2: To address the potential hyper SFN wraparound issue, hyper SFN counter can keep increasing (i.e., without wraparound) unless DRX configuration is (re-)configured.

Reference hyper frame:

Observation 1: Hyper SFN boundary ambiguity between the network and UE may exist when RRC Reconfiguration message is (re-)transmitted across hyper frames.

Proposal 3: drx-timeReferenceSFN with value {0, 512} can be introduced to determine the reference hyper frame of DRX configuration without hyper SFN boundary ambiguity.

Proposal 4: Hyper SFN counter can be maintained by MAC layer only, and the hyper SFN of the reference hyper frame (i.e., initial counter value) is set to 0 when DRX configuration with drx-timeReferenceSFN is (re-)configured.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #121bis-e

[2] Report of 3GPP TSG RAN WG2 meeting #122

[3] R2-2301372, C-DRX enhancements for XR, MediaTek Inc.

[4] TS 38.321, Medium Access Control (MAC) protocol specification, V17.5.0