3GPP TSG-RAN WG2 Meeting #123	R2-2307942

Toulouse, France, May 21 – 25, 2023

Agenda item:		7.5.4.1

Source:		Lenovo

Title:		delay information reporting for XR

Document for:		Discussion and Decision

# Introduction

- RAN#98-e has approved the XR WID in RP-223502 including BSR and delay report enhancement for XR capacity:

- Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2);

In this contribution we further discuss the delay reporting details.

# Discussion

In RAN2#121bis-e and RAN2#122 meeting following agreements for BSR enhancements/delay information reporting have been reached:

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs. - As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). - Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error. - UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this. - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

### Delay Report enhancements

The gNB can take knowledge of PDU set delay into account in scheduling transmissions, e.g., by giving priority to transmissions close to their delay budget limit, and by not scheduling (e.g., UL) transmissions exceeding a PDU set delay budget. The UE can also take advantage of such knowledge to save UE’s power by determining if an UL transmission (e.g., PUCCH in response to PDSCH, UL pose, or PUSCH) corresponding to a transmission that exceeds its delay budget can be dropped. Additionally, UE does not need to wait for re-transmission of a PDSCH that will never occur (e.g., DRX retransmission timers can be stopped). For DL transmissions it is assumed that gNB is aware of the Remaining delay budget of the data pending for transmission, e.g., based on information provided by the SMF, and takes such knowledge into account in scheduling decisions.

For UL resource allocation, it would be necessary that UE provides some assistance information regarding the remaining delay budget of the data pending in its buffer to the gNB. We think that UE should provide information on the delay budget of the data for which UL resources are requested when sending a buffer status report. We think that the PSDB information provided to the RAN is not sufficient. Since NW is not aware of the exact arrival time of UL data in the buffer and hence can also not be sure about the remaining (valid) time of data being pending in the buffer for transmission, UE should provide this information within some assistance information. In our view the delay information should be reported as part of a BSR MAC CE. A new BSR MAC CE format should be introduced for XR-specific BSR reporting which includes the amount of data available for transmission and some delay information associated with the data reported.

Proposal 1: UE should include delay information, e.g. remaining delay budget, associated with data being available for transmission within a new XR-specific BSR.

In the last meeting following agreements have been reached

- UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

According to RAN2#122 meeting UE calculates the remaining time based on the PDCP discard timer value. As agreed before UE maintains a PDCP discard timer per PDCP SDU/PDU. For cases that the PDCP PDUs of a PDU set don’t arrive at the exact same time in the PDCP buffer, the PDCP discard timer values of the PDUs associated with the same PDU set may be different at the time of delay information reporting, e.g. when BSR including delay information is reported. We think that UE should only report one remaining time for a PDU set. Essentially the remaining time of a PDU set is determined based on the lowest PDCP discard timer value of the PDCP SDUs belonging to the PDU set. Generally, the discard timer value of the first PDCP SDU/PDU of a PDU set determines the remaining PSDB of the whole PDU set.

Proposal 2: UE reports the shortest PDCP discard timer value of the PDUs belonging to a PDU set as the remaining time info for the PDU set.

The delay report is associated with the reported buffer status, e.g. delay information is associated with a PDU set. In case more than one LCH is assigned to a LCG, there may be more than one PDU sets reported in a BSR and correspondingly multiple delay information reported within a LCG. How to report should be discussed in RAN2. To follow the BSR report principle, we prefer to report the shortest PDCP discard timer value among the LCHs in an LCG as gNB needs to allocate UL resources most urgently for the data with the shortest remaining delay. Optionally, UE may further consider the LCH priority when UE selects the shortest PDCP discard timer value of an LCH.

Proposal 3: UE reports the shortest delay information, e.g. shortest remaining delay, per LCG.

According to TS38.321, the current BSR triggers do not take the remaining time into account. Therefore, new BSR conditions should be introduced for the delay information reporting. In our opinion, UE triggers the delay report when the remaining time associated with a PDU set is less than a preconfigured threshold to make the network schedule an uplink grant timely to the UE, so that the PDU set can be transmitted to the network within its required latency.  UE should report the amount of data of all the PDCP SDU/PDUs of a PDU set being available for transmission even if the remaining time (PDCP discard timer value) of only one of the PDCP PDUs/SDUs belonging to the PDU set is below the predefined threshold.

According to TS38.321, a BSR is triggered based on data becoming available (also considering the priority of the data). Here we think a new trigger event could be when data for LCH(s) configured to report the associated remaining delay becomes available to a MAC entity. I.e., even if there’s data available for transmission with higher or equal to the LCH priority of the newly arrived data and without delay information for transmission, the delay report should be triggered due to the data arrival for the LCHs configured to report the delay information. Correspondingly, the report can be cancelled if the remaining delay value becomes zero.

Proposal 4: UE triggers a BSR including remaining timing information of the available PDU set:

- if the remaining time associated with a PDU set, e.g.PDCP discard timer value, is less than a threshold.

- UL data for an LCH configured to report the delay information becomes available to a MAC entity and none of the LCHs configured to report the delay information contains any available UL data.

Similarly, gNB would benefit from a notification by UE about UL data - which were reported in a BSR previously - for which the delay budget is exceeded. gNB can take such knowledge into account for an efficient resource allocation, e.g. avoiding to issue further UL grants for data pending in UE’s buffer which is of no use for the user.

Proposal 5: UE should notify gNB when the delay budget is exceeded for data which has been previously reported in a BSR.

The delay information reported in a MAC control element, e.g. BSR MAC CE, should reflect the status of the delay information at the time of the transmission of the MAC CE which contains the delay information on the PUSCH resource. UE determines/calculates the delay information which is reported in a MAC CE for the slot/symbols, e.g. first symbol of the slot, where the MAC CE is transmitted on a PUSCH resource. Essentially the delay information associated with a PDU set reflects the remaining time/ PSDB at the time where the MAC CE is transmitted on PUSCH. 
For cases when the BSR MAC CE could not be transmitted due to e.g. inter-UE prioritization or LBT failure, there might be some misunderstanding between UE and gNB about the reference time for the calculation of the delay information. For NR-U a similar issue exists for the PHR reporting. During Rel-16 discussions it was finally agreed that UE can – as an implementation option – update the PHR content when initial transmission could be carried out due to LBT failure. Just below is the corresponding NOTE from TS38.321

NOTE 3: If a HARQ process is configured with cg-RetransmissionTimer and if the PHR is already included in a MAC PDU for transmission on configured grant by this HARQ process, but not yet transmitted by lower layers, it is up to UE implementation how to handle the PHR content.

For the delay information reporting, we think that there is on one hand the UTO-UCI “unused” indication reported on the CG-UCI, which helps gNB to detect situations where UE couldn’t carry out an uplink (CG PUSCH) due to e.g. intra-UE prioritization. In that sense the situation is a bit different compared to the PHR reporting issue in NR-U, e.g. here gNB couldn’t distinguish between UE not performing an CG PUSCH due to LBT failure or due to no data being in the buffer.

Therefore, there is in our understanding no immediate need that UE updates the delay information at the time of the PUSCH transmission when a BSR MAC CE has been already included in the MAC PDU for transmission. However, we would be also OK when majority of companies is in favour of UE updating the delay information delay information at the time when the MAC CE is (re)transmitted, e.g. for cases when due to intra-UE prioritization a PUSCH transmission has not been performed.

Proposal 6: RAN2 to discuss whether UE should update the delay information reported in a MAC CE at the time of MAC CE transmission, e.g. for cases when previous transmission couldn’t be carried our due to intra-UE prioritization. Given that UE indicates unused resources, e.g. UTO-UCI, gNB should be generally aware of an intra-UE prioritization situation occurring.

# Conclusion

In this contribution, we discuss UE feedback enhancements for XR communications, in particular the details of the delay information reporting procedure. We have the following proposals:

Proposal 1: UE should include delay information, e.g. remaining delay budget, associated with data being available for transmission within a new XR-specific BSR.

Proposal 2: UE reports the shortest PDCP discard timer value of the PDUs belonging to a PDU set as the remaining time info for the PDU set.

Proposal 3: UE reports the shortest delay information, e.g. shortest remaining delay, per LCG.

Proposal 4: UE triggers a BSR including remaining timing information of the available PDU set:

- if the remaining time associated with a PDU set, e.g.PDCP discard timer value, is less than a threshold.

- UL data for an LCH configured to report the delay information becomes available to a MAC entity and none of the LCHs configured to report the delay information contains any available UL data.

Proposal 5: UE should notify gNB when the delay budget is exceeded for data which has been previously reported in a BSR.

Proposal 6: RAN2 to discuss whether UE should update the delay information reported in a MAC CE at the time of MAC CE transmission, e.g. for cases when previous transmission couldn’t be carried our due to intra-UE prioritization. Given that UE indicates unused resources, e.g. UTO-UCI, gNB should be generally aware of an intra-UE prioritization situation occurring.

# References

RAN2#121bis chairman note.

R2-2303203, Discussion on UE Feedback enhancements, Lenovo