3GPP TSG-RAN2 WG2 Meeting #123	R2-2307946

Toulouse, France, August 21-25, 2023

Agenda item:	7.9.4

Source: 	China Telecom

Title:	Discussion on remaining issues of multi-path relaying in scenario 1

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, multi-path relaying related issues have been widely discussed and much progress has been made. However, there are still some open issues and stage-3 details left for further discussion, for both scenarios 1 and 2.

In this contribution, we would like to share our opinions on the following remaining issues in scenario 1 based on the agreements reached in previous RAN2 meetings.

- Whether to allow non-split SRB1/2 configuration over indirect path in scenario 1

- RLF handling in scenario 1

- PC5-RRC message to bring idle/inactive relay UE to connected

- Discussion

Non-split SRB1/2 configuration over indirect path in scenario 1

In previous RAN2 meeting, the following agreements were reached on SRB configuration in scenario 1 and 2. Whether to allow non-split SRBs over indirect path in scenario 1 remains FFS.

Agreements (for scenario 2):

Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.

Agreements (for scenario 1):

The bearer type configuration is provided per SRB.  It is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types (within the bearer types that are supported).

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful.

During the study phase of multi-path relaying, it has agreed that for scenario 1 SRB1 and SRB2 can be configured on either the direct or the indirect path, or on both at least with duplication in RAN2#119bis meeting. In RAN2#120 meeting, RAN2 agreed to support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2. Considering PCell location is somehow related to the SRB configuration, some companies suggest revising the previous agreement on SRB configuration made at the initial stage for scenario 1.

Technically, it is a bit hard to judge which path is always or most of time reliable in multi-path scenario, since the radio link quality may change dynamically. On the other hand, for scenario 2, it has agreed that non-split SRB1 and 2 over indirect path is not supported and primary path of the split SRB1 and 2 is always on direct path. To minimalize spec impact and make progress for multi-path relaying, we are fine to have same restriction on SRB configuration in scenario1 to make the procedure align in both scenarios.

Proposal 1: For Scenario 1, non-split SRB1 and 2 over indirect path is not supported.

RLF handling in multi-path relaying

Regarding RLF handing in multi-path relaying, several agreements were reached in previous RAN2 meeting, but there remain some open issues that may need further discussion.

- Direct path failure(i.e. Uu-RLF) handling

Regarding how to handle direct path failure in multi-path relaying, the following agreement and assumption were reached in last RAN2 meeting.

Agreement:

For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery.

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

Whether addition IE is needed or not could be left for stage-3 CR discussion. One of the FFS that may need to be addressed is the primary path location issue. Basically, since the PCell is always configured on direct path. We understand it is also reasonable to restrict the primary path of the split SRB1 and SRB2 always configured on direct path. However, when direct path failure is detected, the primary path location may need further discussion. If the UE cannot switch the primary path of split SRB1 to the indirect path for reporting after direct path failure, then the actual gain of direct-path failure reporting mechanism maybe reduced. Since whether non-split SRB1 and SRB2 can be configured on indirect path is still pending in RAN2, the only valid case for direct-path failure reporting currently might be split SRB1 with duplication. Considering direct-path failure reporting is introduced to avoid unnecessary RRC re-establishment, we think to allow switching primary path to indirect path for reporting after direct path failure is useful. Thus, we suggest RAN2 confirm the WA.

Proposal 2: For Scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. And the UE could switch the primary path to the indirect path for reporting after direct path failure.

- In case of indirect path failure, which message to be used

Regarding indirect path failure, RAN2 has no consensus on which message to be used for failure reporting upon indirect path failure. We think at least for Scenario1 the SidelinkUEInformationNR message can be used since it has already been used to report a sidelink radio link failure or detection of sidelink RRC reconfiguration failure. The detail description could be updated in stage-3 CR drafting.

Proposal 3: For multi-path relaying in Scenario 1, upon detection of PC5-RLF on the indirect path or upon reception of Uu RLF indication from the relay UE, SidelinkUEInformationNR message can be used to report the indirect path failure to the gNB via direct path if available.

PC5-RRC message to bring idle/inactive relay UE to connected

In previous RAN2 meeting, the issue on how to trigger idle/inactive relay UE into connected in indirect path addition or change case has been discussed and the following agreements were reached.

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

Based on the above agreements, the legacy Rel-17 behaviour of sending the RRCReconfigurationComplete message on the indirect path and/or a new PC5-RRC trigger could be used. Since it is the gNB that select the target relay UE and the gNB is aware of the RRC state of the relay UE, whether to use the legacy way or PC5-RRC trigger is clear from gNB side. However, it remains unclear in which case or on what condition for the remote UE to use PC5-RRC trigger method. There might be the following two options to make the remote UE behaviour clear.

- Option 1: rely on the configuration of SRB1

- Option 2: introduce additional indication for PC5-RRC trigger

For option 1, whether to send the PC5-RRC message for bringing idle/inactive relay UE to connected mode is up to the configuration of SRB1. If no SRB1 (including non-split and split SRB1) is configured on indirect path, the remote UE could not send the RRCReconfigurationComplete message on the indirect path. Then the remote UE shall send PC5-RRC message in order to bring the relay UE to connected mode. Based on option 1, the following cases are listed in Table.1.

Table.1 SRB1 configuration and remote UE behaviour in Option 1

|  | Relay UE in RRC_IDLE/RRC_INACTIVE |  | Relay UE in RRC_CONNECTED |  |
| --- | --- | --- | --- | --- |
| SRB1 configuration on direct/indirect path | Direct + indirect (or indirect only if non-split SRB1 is supported) | Direct path | Direct + indirect (or indirect only if non-split SRB1 is supported) | Direct path |
| Remote UE behaviour (whether to send PC5-RRC message) | No | Yes | No | Yes |

As listed above, for the relay UE in RRC connected mode case, if the gNB only configures SRB1 on the direct path, the remote UE will send the PC5-RRC message to the relay UE even if it is actually not needed, which also brings unnecessary signalling overhead and sidelink resource waste. Otherwise, the gNB shall always configure SRB1 on indirect path if the relay UE is in RRC_CONNECTED mode, which also restricts the gNB implementation. To solve this issue, we can introduce additional indication for PC5-RRC trigger in RRCReconfiguration message, which can explicitly indicate whether the remote UE shall send the PC5-RRC message to the relay UE for bringing it to connected mode.

Proposal 4: Introduce an additional indication for PC5-RRC trigger to indicate whether the remote UE shall send the PC5-RRC message to the relay UE for bringing it to RRC_CONNECTED.

- Conclusion

In this contribution, we discuss several remaining issues on multi-path relaying based on the agreements reached in previous RAN2 meetings, including SRB1/2 configuration, RLF handling and PC5-RRC trigger for idle/inactive relay UE to connected mode in scenario 1. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For Scenario 1, non-split SRB1 and 2 over indirect path is not supported.

Proposal 2: For Scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. And the UE could switch the primary path to the indirect path for reporting after direct path failure.

Proposal 3: For multi-path relaying in Scenario 1, upon detection of PC5-RLF on the indirect path or upon reception of Uu RLF indication from the relay UE, SidelinkUEInformationNR message can be used to report the indirect path failure to the gNB via direct path if available.

Proposal 4: Introduce an additional indication for PC5-RRC trigger to indicate whether the remote UE shall send the PC5-RRC message to the relay UE for bringing it to RRC_CONNECTED.

- Reference

RP-221262	Revised WID on NR sidelink relay enhancements	LG Electronics

3GPP TS 38.331 17.0.0: "NR; Radio Resource Control (RRC) protocol specification".

Report from session on positioning and sidelink relay (RAN2#119e, 119bis-e, 120, 121, 122)