3GPP TSG RAN WG2 #123          								   R2-2307951

Toulouse, France, August 21-25, 2023

Agenda Item:	7.20.2

Source: 	NEC

Title:	Discussion on TAG management for Multi-TRP

Document for:	Discussion

- Introduction

This contribution provides our considerations on multi-DCI based multi-TRP operation with two TAGs from RAN2 perspective.

- Discussion

If SpCell has 2 TAGs, from RAN2 perspective, the question is a problem on how to define the PTAG for this case. The following solutions can be considered to define the PTAG for this case:

- Solution 1: Only one TAG of the SpCell is treated as PTAG. Thus, if the TAT of PTAG is timeout, legacy UE behaviors as in TS 38.321[2] can be adopted to manage the TAT and related resources;

- Solution 2: Two TAGs of SpCell are treated as two separate PTAGs. Then extra specific effort is required to manage the TAT and the corresponding resources when TATs expire, for example how to manage the resources of one cell, one TAG or all cells when the TAT of one PTAG expires or when TATs of both PTAGs expire.

Given the complexity, we prefer Solution 1. Then we propose:

RAN2 to update the concept of PTAG and treat one TAG of the SpCell as PTAG when it is configured with 2 TAGs.

- Conclusion

We have the following proposal:

Proposal 1:	RAN2 to update the concept of PTAG and treat one TAG of the SpCell as PTAG when it is configured with 2 TAGs.

- References

- R2-2300021 LS on Multi-DCI Multi-TRP with two TAs

[2] TS 38.321 V17.5.0