3GPP TSG-RAN WG2 #123	Tdoc R2-2307952

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.18.2

Source: 	Ericsson

Title:  	Further MT-SDT discussion

Document for:	Discussion and Decision

# Introduction

In the WID [1] for MT-SDT it is stated that the objective is

“Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure based UL response;

MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.”

This contribution discusses several issues which have been left as FFS and that must be addressed before a complete specification of MT-SDT can be done. The contribution discusses open issues with regards to - among other things – SIB content, SDT-RSRP threshold for MT-SDT and the introduction of Release Assistance Information (RAI) and use of TA for 2-step RA.

# Discussion

## MT-SDT RSRP Threshold

In RAN2#122, the following agreement was made:

A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

For MO-SDT the threshold is mainly to ensure that UL TBs of a desired size can be transmitted reliably. That is, the larger data volume is expected in the UL and the RSRP threshold is to ensure a certain data volume in the first UL transmission at cell edge. For MT-SDT the main traffic direction is the opposite, meaning that the RSRP threshold is set to enable transmissions of DL TBs of a desired size. Also, for MT-SDT, Msg3/MsgA or the initial CG transmission will not contain any UL data, only the legacy RRCResumeRequest is needed. Given the different objectives of the RSRP threshold for MO-SDT and MT-SDT it may lead to conflicting thresholds and it would be more efficient to configure them separately.

The objective of RSRP thresholds for MO-SDT and MT-SDT are different. The RSRP threshold for MO-SDT is mainly for the first UL data transmission while the RSRP threshold for MT-SDT is mainly for DL traffic.

The current wording in the running CR is shown below:

SIB1-v1800-IEs ::=               SEQUENCE {

mt-SDT-ConfigCommonSIB-r18   SDT-ConfigCommonSIB-r17                                                OPTIONAL,  -- Cond MT-SDT-Only

nonCriticalExtension         SEQUENCE {}                                                            OPTIONAL

}

Based on this, we give the following proposal:

The RSRP threshold for MT-SDT is configured separately from the sdt-RSRP-Threshold in Rel-17.

Remove the text “Cond MT-SDT only” from the running CR, to reflect that the IE can be configured in addition to an MO-SDT configuration.

Define a new information element for MT-SDT, which is always present if MT-SDT is activated in the cell, containing RSRP threshold, t319a and sdt-LogicalChannelSR-DelayTimer.

## SIB Configuration

In RAN2#122, the following agreement was made:

Allow support of only MT-SDT in a cell. A separate SIB configuration will be introduced.  FFS what is put in there.

There are two cases that need to be considered. First the case where only MT-SDT is configured in the cell and secondly the case where both MO-SDT and MT-SDT are configured in the cell. For MO-SDT, the following parameters are configured in SIB:

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

Of these, sdt-LogicalChannelSR-DelayTimer-r17 and t319a-r17 are common to both MO-SDT and MT-SDT. For the other parameters, we believe that sdt-RSRP-Threshold-r17 and sdt-DataVolumeThreshold-r17, are specific to MO-SDT and should not be configured for MT-SDT. Instead, a new RSRP threshold for MT-SDT should be included. In addition to this, we believe that CQI reporting in Msg3 or MsgA should be configurable as we explain in our contribution R2-2307934 (ref [4]) and that an indication for this is needed in SIB for MT-SDT.

When only MO, or MO together with MT-SDT is configured in a cell, parameters common to MO and MT-SDT, could be indicated in the release 17 IE.

Parameters explicitly used for MT-SDT, could be signalled in the release 18 IE.

The new IE could also contain information fields for when only MT-SDT is used in a cell, e.g. t319a and sdt-LogicalChannelSR-DelayTimer.

Table 1; Suggestion for where to read the information

|  | T319a | RSRP threshold | DVT | Logical Channel Delay timer | CQI indication |
| --- | --- | --- | --- | --- | --- |
| MO-SDT only | Rel-17 IE | Rel-17 IE | Rel-17 IE | Rel-17 IE | N/A |
| MO-SDT & MT-SDT | Rel-17 IE | Rel-17 IE & Rel-18 IE | Rel-17 IE | Rel-17 IE | Rel-18 IE |
| MT-SDT only | Rel-18 IE | Rel-18 IE | N/A | Rel-18 IE | Rel-18 IE |

When MO-SDT and MT-SDT is configured in a cell, the legacy release 17 IE (SDT-ConfigCommonSIB-r17) carries information related to both MO-SDT and MT-SDT, except for MT-SDT RSRP threshold.

RSRP threshold for MT-SDT and indication to apply CQI report in Msg3/MsgA are configured in a new information element (SDT-MT-ConfigCommonSIB-r18). The information element also contains general SDT information that is only used when MO-SDT is not configured in the cell.

The new RRC IE could be coded as follows in RRC:

SDT-MT-ConfigCommonSIB-r18 ::=          SEQUENCE {

sdt-RSRP-Threshold-r18               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-CQI-Reporting-r18               ENUMERATED { true, false}                                    OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r18  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

## Release Assistance Indication (RAI)

The applications supporting MT-SDT can include use-cases where no UL response is generated. Since the time between DL traffic and the eventual UL response is hard to predict, the SDT procedure might be ongoing a long time waiting for an UL response that is never transmitted. Typically, the UE will be released using a timeout (internal timer) as shown in Figure 2. However, this results in a high energy expenditure.

Figure 2; A case when no UL response is generated.

In these cases, it is beneficial to release the UE immediately after sending the DL data and initiate a new MO-SDT procedure to transmit a possible UL response to keep energy consumption low.

For MT-SDT applications where the UL response has a long delay, or for cases without UL response, it is beneficial to release the UE immediately after sending DL data.

Based on this observation, it seems beneficial to predict the expected UL response based on the application or service. This could be done by categorizing the applications based on the DRB or QoS service flow on the RAN-level. In many cases it is expected that the UE has better knowledge of the response pattern upon receiving the DL Data. An indication from the UE could assist the gNB of possible recommended actions, e.g. release, delayed release, etc. This can be done either using RRC procedures, e.g, the network can request for UEAssistanceInformation, or alternately, this indication can be conveyed as a MAC CE. Since the latter option involves reduced signalling overhead, we make the following proposal.

Introduce a release assistance information MAC CE that the UE can send to the gNB when it has information about the UL response during an MT-SDT procedure.

Going forward with the above proposal means the UE can send an early indication of whether a UL reply is expected once it receives DL data from the gNB, thereby enabling more efficient RRC procedures to save energy and resources and enhance end user performance. It should be noted that a DCQR and AS RAI MAC control element is defined for LTE. A similar MAC CE could easily be designed also for SDT in NR.

## Reusing TA for 2-step RA for UL transmissions.

The legacy approach for handling an upcoming UL response in MT-SDT is to trigger a RACH procedure when an UL response arrives at the UE’s buffer. By definition, Timing Alignment Timer (TAT) validity ensures that Timing Advance (TA) remains valid. In other words, this mean that any UL response from the UE should be able to use the same TA as long as TAT is running.

As long as the TAT continues to be running, any UL response from the UE should be able to use the same TA.

This means that if TAT continues to be running, the same TA could be used during a 2-step RACH procedure triggered as part of UE’s UL response as shown in the figure below. In current specifications, 2-step RA is always using TA=0.

Figure 3- If TAT is running, a 2-step RA-based UL response from UE should be able to use the TA obtained from Step 5.

Reusing the timing advance based on the valid TAT offers significant benefits as listed below.

- Re-using TA improves the coverage for using 2-step RA for UL response since the legacy assumption of TA=0 is not used. When the actual TA is used, the 2-step RA can be applied for UL response even for cell-edge UEs in large cells. This consequently reduces the signalling overhead compared to doing 4-step RA.

- Improves decodability of the PUSCH since the slots are aligned. This improves the energy efficiency by reducing the retransmissions.

- Reusing TA to enable usage of 2-step RA improves the resource efficiency as the gNB is not pre-allocating any UL resources.

Legacy approach for handling UL response in MT-SDT by triggering a RACH procedure can significantly benefit by reusing the Timing Advance (TA) if Timing Alignment Timer (TAT) continues to be running. Re-using TA improves the coverage of 2-step RA for UL response since the 2-step RA can work with the actual TA rather than with the assumption of TA=0. In addition, reusing TA can improve the decodability of PUSCH slots.

Reuse TA obtained from MT-SDT initiation during 2-step RACH procedure triggered as part of UL response in MT-SDT

# Conclusion

In the previous sections we made the following observations:

Observation 1	The objective of RSRP thresholds for MO-SDT and MT-SDT are different. The RSRP threshold for MO-SDT is mainly for the first UL data transmission while the RSRP threshold for MT-SDT is mainly for DL traffic.

Observation 2	When only MO, or MO together with MT-SDT is configured in a cell, parameters common to MO and MT-SDT, could be indicated in the release 17 IE.

Observation 3	Parameters explicitly used for MT-SDT, could be signalled in the a release 18 IE.

Observation 4	The new IE could also contain information fields for when only MT-SDT is used in a cell, e.g. t319a and sdt-LogicalChannelSR-DelayTimer.

Observation 5	For MT-SDT applications where the UL response has a long delay, or for cases without UL response, it is beneficial to release the UE immediately after sending DL data.

Observation 6	As long as the TAT continues to be running, any UL response from the UE should be able to use the same TA.

Observation 7	Legacy approach for handling UL response in MT-SDT by triggering a RACH procedure can significantly benefit by reusing the Timing Advance (TA) if Timing Alignment Timer (TAT) continues to be running. Re-using TA improves the coverage of 2-step RA for UL response since the 2-step RA can work with the actual TA rather than with the assumption of TA=0. In addition, reusing TA can improve the decodability of PUSCH slots.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The RSRP threshold for MT-SDT is configured separately from the sdt-RSRP-Threshold in Rel-17.

Proposal 2	Remove the text “Cond MT-SDT only” from the running CR, to reflect that the IE can be configured in addition to an MO-SDT configuration.

Proposal 3	Define a new information element for MT-SDT, which is always present if MT-SDT is activated in the cell, containing RSRP threshold, t319a and sdt-LogicalChannelSR-DelayTimer.

Proposal 4	When MO-SDT and MT-SDT is configured in a cell, the legacy release 17 IE (SDT-ConfigCommonSIB-r17) carries information related to both MO-SDT and MT-SDT, except for MT-SDT RSRP threshold.

Proposal 5	RSRP threshold for MT-SDT and indication to apply CQI report in Msg3/MsgA are configured in a new information element (SDT-MT-ConfigCommonSIB-r18). The information element also contains general SDT information that is only used when MO-SDT is not configured in the cell.

Proposal 6	Introduce a release assistance information in MAC that the UE can send to the gNB when it has information about the UL response during an MT-SDT procedure.

Proposal 7	Reuse TA obtained from MT-SDT initiation during 2-step RACH procedure triggered as part of UL response in MT-SDT

# References

3GPP RP-222993. “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR”

3GPP 38.331, 17.4.0, NR; Radio Resource Control (RRC); Protocol specification

- [3]	3GPP 38.212, 17.4.0, NR; Multiplexing and channel coding

- [4]	3GPP Tdoc R2-2307934