3GPP TSG-RAN WG2 Meeting #123	R2-2307960

Toulouse, France, August 21-25, 2023

Agenda item:		7.18.3

Source:		Lenovo

Title:		Discussion on remaining UP issues for MT-SDT

Document for:		Discussion and Decision

# Introduction

In RAN2#122 meeting, the following agreements were achieved [1].

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

In this contribution, we discuss remaining UP issues related with the MT-SDT procedure.

# Discussion

SDT selection procedure

Following the legacy SDT resource selection procedure as defined for Rel-17, UE would determine based on a data volume threshold and some RSRP thresholds whether to select the CG-SDT procedure or RACH-SDT procedure or even select the legacy RRCResume procedure (transit to RRC_CONNECTED and performing data transmission in CONNECTED state).
For Rel-17 the RSRP thresholds, e.g. RSRP threshold to determine whether to perform SDT procedure or legacy RRCResume procedure, were introduced in order to ensure that there is a high likelihood that the UL transmissions are successfully transmitted by the UE, i.e. only UEs in good geometry/channel conditions are allowed to perform SDT procedure where larger TB sizes are transmitted in the UL. 
We see some benefit  that UE applies different RSRP thresholds for MT-SDT (DL-SDT). RAN2#122 already agreed the possibility to define separate RSRP thresholds for MO-SDT and MT-SDT. The reason for defining separate RSRP thresholds for MO-SDT and MT-SDT is that they are targeting different performance aspects.. For MO-SDT the threshold ensurey that UL TBs of a desired size to be transmitted reliably in the first UL transmission at cell edge as already mentioned. For MT-SDT the target is downlink traffic which means that the RSRP threshold is set for DL transmissions. Since UL and DL link budget is different, this could indeed result in different threshold values. Furthermore, if the configuration enables separate thresholds, it is fully possible to use the same values for companies not agreeing to the benefit of separate threshold values. RSRP threshold(s) which are applied by the UE for a SDT procedure triggered by NW can be either signalled within the RRCRelease message or broadcast in the cell.

Proposal 1: Confirm the RAN2#122 agreement that RSRP thresholds for MT-SDT and MO-SDT can be configured separately.

Proposal 2: The RSRP thresholds applied for MT-SDT are either signalled within the RRCRelease message or broadcast in the cell.

Carrier selection for MT-SDT

It may not be beneficial that UE performs carrier selection for cases when the SDT procedure was triggered by the NW, e.g. paging. UE should only consider the NUL carrier for the SDT resource selection, since the SDT procedure was triggered for purpose of DL (MT) data transmission. Therefore, any potential SDT related configurations on a supplemental UL (SUL) carrier don’t need to be considered for the SDT resource selection step/procedure.

Proposal 3: It is not necessary to perform carrier selection for MT-SDT and only the SDT resources on NUL carrier should be used to respond to MT-SDT.

SSB selection for CG resources

UE should select the CG-SDT resources which are mapped to the beam/SSB on which the SDT triggering message was received. In Rel-17 SDT procedure, an association between CG resources (used for CG-SDT) and SSBs has been introduced. According to the procedure UE checks during the SDT initiation stage (SDT resource selection procedure), if at least one of the SSB’s RSRP is above a configured threshold in order to determine if at least one of the SSB is qualified to be selected for CG transmission. The association between SSB and CG resource is configured in order to indicate to gNB on what SSB/beam to schedule the UE for further transmissions, i.e. similar to the SSB to RO mapping. In Rel-17 in the SDT initiation stage, if none of SSB is qualified to be selected for CG transmission, UE is not allowed to transmit any small data on the CG resource. If CG-SDT criteria are not met, UE should further check the RA-SDT criteria to evaluate whether the criteria of RA-SDT could be met or not. We think that for DL SDT UE doesn’t need to perform the SSB’s RSRP comparison (CG-SDT criteria) against the preconfigured threshold, but instead selects the CG resources (when CG-SDT is selected by UE based on TA validity for example) which are associated with the SSB/beam on which the SDT triggering message, e.g. paging message, was received.

Proposal 4:The CG resources associated with the SSB/beam on which the SDT triggering message was received, are selected in response to the MT-SDT.

PUCCH resources for HARQ feedback for DL data transmission

HARQ feedback is needed to respond to the DL data transmission. PUCCH resources in CONNECTED state may be used or the SDT specific PUCCH resources can be configured when the UE is released to INACTIVE state. It can be up to RAN1 to decide the detailed configuration of PUCCH resources for HARQ feedback for DL data transmission.

Proposal 5: RAN1 to discuss the detailed configuration of PUCCH resources for HARQ feedback for DL data transmission.

SDT resources are too far

Currently, the offset between the initiation of the SDT procedure and the next CG-SDT occasion is used to determine whether the CG resource is too far away. So far, no corresponding agreement for RA-SDT resources, e.g. whether RA-SDT resources are too far away, has been achieved. Since the too far issue origins from the transmission resource for SDT, we think RA-SDT resource shall also be determined whether it is too far for MO-SDT. Further, to simplify the determination, the PRACH resource can be used to determine whether the RA-SDT resource is too far for MO-SDT. If the RA-SDT is also too far, a legacy RACH is initiated for MO-SDT. on the other hand, if the RA-SDT resources can be frequently configured, the RA-SDT is not too far for any MO-SDT procedure. Then, the UE will not need to determine whether RA-SDT resource is too far.

Proposal 6: The UE shall determine whether the next available RA-SDT resource is too far away.

Once the CG-SDT is determined as too far, UE falls back to the legacy RACH. Then the cg-SDT-TimeAlignmentTimer is considered as expired. However, the CG resource may be too far for only one SDT. For the next SDT transmission, it may be not too far and the CG resource can be considered as valid.

Proposal 7: The cg-SDT-TimeAlignmentTimer keeps running for the case that UE falls back to legacy RACH because of the CG-SDT resource being too far away.

# Conclusion

In this contribution, we discuss the procedure related with MT-SDT and provide the proposals:

Proposal 1: Confirm the RAN2#122 agreement that RSRP thresholds for MT-SDT and MO-SDT can be configured separately.

Proposal 2: The RSRP thresholds applied for MT-SDT are either signalled within the RRCRelease message or broadcast in the cell.

Proposal 3: It is not necessary to perform carrier selection for MT-SDT and only the SDT resources on NUL carrier should be used to respond to MT-SDT.

Proposal 4:The CG resources associated with the SSB/beam on which the SDT triggering message was received, are selected in response to the MT-SDT.

Proposal 5: RAN1 to discuss the detailed configuration of PUCCH resources for HARQ feedback for DL data transmission.

Proposal 6: The UE shall determine whether the next available RA-SDT resource is too far away.

Proposal 7: The cg-SDT-TimeAlignmentTimer keeps running for the case that UE falls back to legacy RACH because of the CG-SDT resource being too far away.

# References

RAN2#122 chairman notes.