3GPP TSG-RAN WG2 #123								R2-2307967
Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.14.2 QoE measurements in RRC_IDLE INACTIVE

Source:	Ericsson

Title:	QoE measurements in RRC_INACTIVE and RRC_IDLE

Document for:	Discussion

# 1	Introduction

Multicast and Broadcast Service (MBS) was specified for NR, in RP-221803 [1], while Quality of Experience (QoE) was specified for Virtual Reality (VR) and Dynamic Adaptive Streaming over HTTP (DASH) in [2]. The WID [3] for Release 18, entitled “Enhancement on NR QoE management and optimizations for diverse services”, aims at introducing enhanced QoE support for diverse services, such as MBS. This contribution addresses the following objective defined in [3]:

“Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]”

In the previous RAN2 meetings, storing of QoE reports in RRC_INACTIVE and IDLE states were extensively discussed. In this contribution, we therefore discuss QoE report storing related issues, such as UE AS minimum memory size, a new UE capability and possible QoE report discarding mechanisms in more detail.

# 2	Discussion

## 2.1 	Storing QoE measurement reports and AS layer buffer size

In the RAN2#121 meeting, it was decided that RAN2 will discuss where the QoE reports buffered while the UE was in RRC_INACTIVE or RRC_IDLE state could be stored, i.e., in UE AS or UE application layer. Additionally, it was left for further discussions, what should be the minimum memory size requirement. In the contribution R2-2212938 it was discussed that in the case the reports stored in RRC INACTIVE/RRC IDLE states would exceed 64 kB, then both the UE AS and application layers would be responsible for storing the reports until the UE re-enters the RRC CONNECTED state. In rel-17, when QoE reports were agreed to be stored during pause of QoE reporting, it was decided to store the reports in UE AS in order not to reveal to the application layer when the network is overloaded. Information related to the UE being in RRC_IDLE or RRC_INACTIVE may not be as sensitive information and the application layer could possibly be used for storing of QoE reports. However, storing QoE reports at the application layer would prevent the AS from having a full overview of the number of stored reports and the amount of data they contain, and this could potentially restrict the AS’ possibilities to signal detailed information about the stored QoE reports when returning to RRC_CONNECTED state. Following the discussion on the minimum memory size for reporting QoE reports in RRC_INACTIVE and RRC_IDLE states, the following was agreed in the RAN2#122 meeting:

Introduce AS layer minimum memory requirement for storing Rel-18 QoE reports measured in RRC_IDLE/RRC_INACTIVE. Could have larger values than in Rel-17. FFS what is the minimum size requirement capability. FFS what is the value range of the capability.

There were some proposals in the RAN2#122 meeting to introduce new UE capabilities either to indicate whether the measurements are dedicated or suitable for RRC_IDLE or RRC_INACTIVE states (R2-2305606, R2-2306107 ZTE) and/or whether other than 64 kB UE SA buffer size(s) is(are) available (R2-2305310 Samsung, R2-2305606 CMCC, R2-2305766 Qualcomm, R2-2305809 Huawei). There were proposals to introduce a separate UE capability with multiple values, e.g. {64KB, 128KB, 256KB, 512KB}, indicated as a separate Rel-18 UE AS layer minimum memory requirement., where UE indicates itself one of those values as a UE capability (R2-2305310).

In the RAN2#122 it was agreed that an AS layer minimum memory requirement for storing QoE reports collected in RRC_IDLE/RRC_INACTIVE states could have larger value(s) than in that in Rel-17.

Additionally, it was agreed that a UE could store the reports for an extended period, i.e., 48 h: UE is allowed to release stored reports and configuration after 48h (similar to logged MDT). No timer is configured by the network.

Considering observations 1 and 2, the UE AS minimum memory size for storage of QoE reports could be increased to at least 512 kB, to accommodate for a longer storage time and enabling more QoE reports to be collected. This would also enable storing a larger amount of the buffered reports in the UE AS layer before needing to possibly start storing in the UE Application layer. Storing the QoE reports in the UE AS layer enables more control over the report storing and reporting, as well as enables a rather easy modification of the UE AS memory layer in comparison to an option of storing buffered reports in the UE application layer, which would require to assess how to store the reports in the UE application layer and standardize additionally what is a suitable memory size available in the UE application layer for the report storing.

Increase the UE AS minimum memory for storage of QoE reports to at least 512 kB to accommodate larger amounts of buffered QoE reports in the RRC_INACTIVE/RRC_IDLE states.

Introduce a new UE capability to indicate available UE AS memory size for QoE measurements in RRC_INACTIVE and RRC_IDLE states.

However, if the reports are collected and stored for a long time, they might become no longer relevant for the OAM which provided the QoE configuration to the UE, or for the gNB which configured the UE for those measurements. Therefore, to enable storing newer and/or more relevant QoE reports, a mechanism to discard certain QoE reports could also be introduced. One principle for discarding the QoE reports collected while in RRC_INACTIVE/RRC_IDLE states could be replacing old reports with new reports. That means if the UE AS memory is already full, the UE should discard oldest report to make space for the new incoming report.

When the UE AS memory becomes full, discard the oldest accumulated QoE reports when new reports are received.

## 2.2	PLMN identity list checking upon returning to RRC_CONNECTED

To ensure the QoE measurement continuity upon RRC state transition from RRC CONNECTED to RRC IDLE state, the network version of the QoE measurement configuration should not be deleted but stored somewhere else, FFS for the exact storage location. Independently of where this QoE measurement configuration is stored, the key principle should be that this configuration is provided to the new gNB, only if the new gNB belongs to the same network as configured the UE with QoE. One important aspect that needs to be addressed to enable this is the check of the PLMN identities. If PLMN identity list for an area scope is not the same for the new gNB as for the old gNB, the QoE measurements may not be applicable for the new gNB. Therefore, when returning to RRC_CONNECTED, the UE should check that the new PLMN matches the PLMN in which the UE received the QoE configuration, or an equivalent PLMN, before indicating to the network that it has QoE reports to transmit. This is to ensure that the QoE reports are sent to the network that configured the UE with the QoE measurements.

The UE checks the PLMN of the target gNB when reconnecting from RRC_IDLE to RRC_CONNECTED and indicates to the target gNB that it has QoE reports to transmit, only if the PLMN ID of the target gNB is the same as the PLMN ID where the UE received the QoE configuration, or an equivalent PLMN.

# Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Observation 1	In the RAN2#122 it was agreed that an AS layer minimum memory requirement for storing QoE reports collected in RRC_IDLE/RRC_INACTIVE states could have larger value(s) than in that in Rel-17.

Observation 2	Additionally, it was agreed that a UE could store the reports for an extended period, i.e., 48 h: UE is allowed to release stored reports and configuration after 48h (similar to logged MDT). No timer is configured by the network.

Proposal 1	Increase the UE AS minimum memory for storage of QoE reports to at least 512 kB to accommodate larger amounts of buffered QoE reports in the RRC_INACTIVE/RRC_IDLE states.

Proposal 2	Introduce a new UE capability to indicate available UE AS memory size for QoE measurements in RRC_INACTIVE and RRC_IDLE states.

Proposal 3	When the UE AS memory becomes full, discard the oldest accumulated QoE reports when new reports are received.

Proposal 4	The UE checks the PLMN of the target gNB when reconnecting from RRC_IDLE to RRC_CONNECTED and indicates to the target gNB that it has QoE reports to transmit, only if the PLMN ID of the target gNB is the same as the PLMN ID where the UE received the QoE configuration, or an equivalent PLMN.

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

R2-2213054, LS on QoE measurements in RRC IDLE/INACTIVE states, Huawei

R2-2301940, Reply LS on QoE measurements in RRC IDLE/INACTIVE states, Huawei