3GPP TSG-RAN WG2 Meeting #123	R2-2307974

Toulouse, France, 21– 25 August 2023

Agenda item:	7.24.2

Source:	Nokia, Nokia Shanghai Bell, AT&T, Qualcomm

Title:	PTM retransmission reception by UEs without HARQ feedback

WID/SID:	NR_MBS-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In Rel. 17 MBS multicast, the PTM retransmissions requested by other UEs cannot be received by those UEs whose HARQ feedback had been disabled. As discussed, and realized within the RAN2 group during the end of Rel-17, it would be beneficial both for the UEs and the network to allow UEs with HARQ feedback disabled to receive HARQ retransmissions requested by other UEs. Due to timing constraints, Rel-17 agreement was not achieved  on  this enhancement, and requested that this would be handled in Rel-18.

# 2	Discussion

When there are many UEs receiving MBS multicast in a cell, the network may want to disable the HARQ feedback for some UEs, without  moving those UEs to the RRC_INACTIVE state, as the UEs may have some other unicast ongoing that forces gNB to keep this UE in RRC_CONNECTED. Since other UEs are sending HARQ feedback, the network may be sending PTM retransmissions to those UEs and it would be beneficial if also UEs without HARQ feedback could receive those retransmissions. Currently, UEs without/disabled HARQ feedback configuration cannot be configured with drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM. Therefore, the UEs without/disabled HARQ feedback configuration and configured with DRX, cannot receive the HARQ PTM retransmissions efficiently (i.e. unless the drx-onDurationTimerPTM or drx-InactivityTimerPTM are configured so large that HARQ retransmission happens when at least one of them is running or UE monitors all the time irrespective of the possibility of sleep during DRX).

Observation 1: UEs with HARQ feedback disabled cannot be configured with drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM and therefore, cannot efficiently receive PTM retransmissions requested by other UEs.

By letting drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM also to be configurable for UEs that have HARQ feedback disabled, both UE and network performance and reliability can be improved. Since drx-RetransmissionTimerDL-PTM is started only if the reception of the transport block has not been successful, this change would not increase the UE power consumption unnecessarily. Therefore, we propose:

Proposal 1: Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM also when HARQ feedback is disabled. (38.331)

On the other hand, considerations are needed for ACK/NACK and NACK-only HARQ feedback mechanisms on how to realize the above proposals.

If a UE is configured "disabled feedback", but it still knows the PUCCH configuration for multicast ACK/NACK feedback, e.g., feedback is temporarily disabled for this UE, then the UE can understand still when such feedback would have been transmitted, as DCI includes fields defining the timing and frequency resources for PUCCH. With those, it is possible that UE understands where it would send the PUCCH normally, if it were not disabled.

If a UE is configured with NACK-only, but  with "disabled feedback", then the same applies as above for only 1-bit NACK-only (same fields of DCI are used). For multiple NACK-only feedback bits, there are two options:

- Mode-1: UE converting NACK-only to ACK/NACK. Again, DCI indicates which PUCCH resource to use in this case (same as for ACK/NACK feedback explained above).

- Mode-2: UE picking a PUCCH resource based on configuration on mapping of NACK-only sequences (for this multiple bits) to PUCCH resources. If the UE has disabled feedback, it may be that UE is unaware of such mapping. Therefore, it cannot understand when to transmit feedback.

If configured by “enabled” HARQ feedback, UE starts drx-HARQ-RTT-TimerDL-PTM after sending the feedback, i.e., in the symbol where utilized PUCCH for HARQ ends. Therefore, the UE should only start the drx-HARQ-RTT-TimerDL-PTM if it knows when it would transmit the HARQ feedback if it had feedback enabled. We propose two options for starting the PTM DRX timers at the UE:

Proposal 2: RAN2 chooses one of the below options for reception of PTM retransmissions by the UEs with disabled HARQ feedback:

Option 1: UEs indicate its capabilities to support PTM retransmission reception when HARQ is disabled and accordingly network optionally configures the drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM also when HARQ feedback is disabled. In this case,  network expects the UEs to start timers drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM.

Note: The changes to capture option 1 will be captured as normative text.

Or

Option 2: Network optionally configures the drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM  when HARQ feedback is disabled. The UEs can optionally start timers drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM if HARQ is disabled for the UEs.

Note: The changes required to capture option 2 will be added as a NOTE.

The corresponding text proposals are provided in the Annex for both Option 1 and Option 2. The co-sourcing companies prefer option 1. If option 2 of the  above proposal is acceptable, we propose to discuss how would Rel. 17 UEs not implementing the Rel. 18 CR handle if they receive the parameter i.e. would we need a capability bit to avoid NW sending the configuration if UE has not implemented the proposed change.

Proposal 3: Discuss whether implementing the change with option 2 would require UE capability, especially if we allow earlier release UEs to implement this change.

# 3	Conclusion

This document has made the following observations:

Observation 1: UEs with HARQ feedback disabled cannot be configured with drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM and therefore, cannot efficiently receive PTM retransmissions requested by other UEs.

And proposed the following:

Proposal 1: Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM also when HARQ feedback is disabled. (38.331)

Proposal 2: RAN2 chooses one of the below options for reception of PTM retransmissions by the UEs with disabled HARQ feedback:

Option 1: UEs indicate its capabilities to support PTM retransmission reception when HARQ is disabled and accordingly network optionally configures the drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM also when HARQ feedback is disabled. In this case, if the UEs indicate it’s capability to support PTM retransmission reception when HARQ is disabled and the network configures the timers, then network expect the UEs to start timers drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM. The changes to capture option 1 will be captured as normative text.

Or

Option 2: Network optionally configures the drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM  when HARQ feedback is disabled. The UEs can optionally start timers drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM if HARQ is disabled for the UEs. The changes required to capture option 2 will be added as a NOTE.

Proposal 3: Discuss whether implementing the change with option 2 would require UE capability, especially if we allow earlier release UEs to implement this change.

# Text proposal for MAC: Option 1

START OF THE CHANGE

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	if HARQ feedback is enabled:

3>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]; and

3>	if CS-RNTI is configured:

4>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else if drx-HARQ-RTT-TimerDL-PTM is configured and UE knows when the corresponding DL HARQ feedback would be transmitted if enabled:

3>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding HARQ feedback.

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL-PTM expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL-PTM.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received:

2>	stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI;

2>	stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI.

1>	if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM:

2>	start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe.

1>	if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI:

2>	monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL multicast transmission:

3>	if HARQ feedback is enabled:

4>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

4>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]:

5>	if the PDCCH addressed to G-RNTI indicates a DL multicast transmission; or

5>	if the PDCCH addressed to G-CS-RNTI indicates a DL multicast transmission and CS-RNTI is configured:

6>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else if drx-HARQ-RTT-TimerDL-PTM is configured and UE knows when the corresponding DL HARQ feedback would be transmitted if enabled:

4>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding HARQ feedback.

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a new multicast transmission for this G-RNTI or G-CS-RNTI:

3>	start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception.

NOTE 1:	A PDCCH indicating activation of multicast SPS is considered to indicate a new transmission.

NOTE 2:	The UE may start the drx-HARQ-RTT-TimerDL after receiving a PTM transmission only if ptp-Retx-Multicast or ptp-Retx-SPS-Multicast was included in the UECapabilityInformation message to network.

The MAC entity needs not to monitor the PDCCH for a G-RNTI or a G-CS-RNTI if it is not a complete PDCCH occasion (e.g. the Active Time for a G-RNTI or a G-CS-RNTI starts or ends in the middle of a PDCCH occasion).

END OF THE CHANGE

# Text proposal for MAC: Option 2

START OF THE CHANGE

When multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall for this G-RNTI or G-CS-RNTI:

1>	if a MAC PDU is received in a configured downlink multicast assignment:

2>	if HARQ feedback is enabled:

3>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]; and

3>	if CS-RNTI is configured:

4>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL-PTM expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL-PTM.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to a G-RNTI or G-CS-RNTI, or by a configured downlink multicast assignment is received:

2>	stop drx-onDurationTimerPTM of the DRX for this G-RNTI or G-CS-RNTI;

2>	stop drx-InactivityTimerPTM of the DRX for this G-RNTI or G-CS-RNTI.

1>	if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM:

2>	start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe.

1>	if the MAC entity is in Active Time for this G-RNTI or G-CS-RNTI:

2>	monitor the PDCCH for this G-RNTI or G-CS-RNTI as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL multicast transmission:

3>	if HARQ feedback is enabled:

4>	start the drx-HARQ-RTT-TimerDL-PTM for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

4>	if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured as specified in TS 38.213 [6]:

5>	if the PDCCH addressed to G-RNTI indicates a DL multicast transmission; or

5>	if the PDCCH addressed to G-CS-RNTI indicates a DL multicast transmission and CS-RNTI is configured:

6>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a new multicast transmission for this G-RNTI or G-CS-RNTI:

3>	start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception.

NOTE 1:	A PDCCH indicating activation of multicast SPS is considered to indicate a new transmission.

NOTE 2:	The UE may start the drx-HARQ-RTT-TimerDL after receiving a PTM transmission only if ptp-Retx-Multicast or ptp-Retx-SPS-Multicast was included in the UECapabilityInformation message to network.

NOTE 3: If drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM is configured, the UEs can optionally start timers drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM if HARQ is disabled, and if UE knows when the corresponding DL HARQ feedback would be transmitted if enabled for the corresponding HARQ process in the first symbol after the end of the corresponding HARQ feedback.

The MAC entity needs not to monitor the PDCCH for a G-RNTI or a G-CS-RNTI if it is not a complete PDCCH occasion (e.g. the Active Time for a G-RNTI or a G-CS-RNTI starts or ends in the middle of a PDCCH occasion).

END OF THE CHANGE

# Text proposal for RRC – For option 1 and option 2

### 6.3.6	MBS information elements

****************************** Unchanged parts are omitted ***************************************

#### –	DRX-ConfigPTM

The IE DRX-Config-PTM is used to configure DRX related parameters for PTM transmission as specified in TS 38.321 [3].

DRX-Config-PTM information element

-- ASN1START

-- TAG-DRX-CONFIGPTM-START

DRX-ConfigPTM-r17 ::=             SEQUENCE {

drx-onDurationTimerPTM-r17        CHOICE {

subMilliSeconds                   INTEGER (1..31),

milliSeconds                      ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1

}

},

drx-InactivityTimerPTM-r17        ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1

},

drx-HARQ-RTT-TimerDL-PTM-r17      INTEGER (0..56)                      OPTIONAL,  -- Cond HARQFeedback

drx-RetransmissionTimerDL-PTM-r17 ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1

}                                                                      OPTIONAL,   -- Cond HARQFeedback

drx-LongCycleStartOffsetPTM-r17   CHOICE {

ms10                              INTEGER(0..9),

ms20                              INTEGER(0..19),

ms32                              INTEGER(0..31),

ms40                              INTEGER(0..39),

ms60                              INTEGER(0..59),

ms64                              INTEGER(0..63),

ms70                              INTEGER(0..69),

ms80                              INTEGER(0..79),

ms128                             INTEGER(0..127),

ms160                             INTEGER(0..159),

ms256                             INTEGER(0..255),

ms320                             INTEGER(0..319),

ms512                             INTEGER(0..511),

ms640                             INTEGER(0..639),

ms1024                            INTEGER(0..1023),

ms1280                            INTEGER(0..1279),

ms2048                            INTEGER(0..2047),

ms2560                            INTEGER(0..2559),

ms5120                            INTEGER(0..5119),

ms10240                           INTEGER(0..10239)

},

drx-SlotOffsetPTM-r17                 INTEGER (0..31)

}

-- TAG-DRX-CONFIGPTM-STOP

-- ASN1STOP

DRX-Config-PTM field descriptions

drx-HARQ-RTT-Timer-DL-PTM Value in number of symbols of the CFR where the transport block was received.

drx-InactivityTimerPTM Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffsetPTM drx-LongCycle-PTM in ms and drx-StartOffset-PTM in multiples of 1 ms.

drx-onDurationTimerPTM Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimer-DL-PTM Value in number of slot lengths of the CFR where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-SlotOffsetPTM Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

| Conditional Presence | Explanation |
| --- | --- |
| HARQFeedback | The field is mandatory present if HARQ feedback is enabled for a G-RNTI/G-CS-RNTI associated with this DRX configuration. Otherwise, it is optionally present if UE is expected to receive HARQ PTM retransmissions requested by other UEs. It is absent otherwise. |

End of change