3GPP TSG-RAN WG2 Meeting #123	R2-2307984

Toulouse,  21 – 25  August 2023

Agenda item:	7.11.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	User plane aspects of multicast reception in RRC_INACTIVE state

WID/SID:	NR_MBS_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The Rel-18 work item on Enhancements of NR Multicast and Broadcast services has been agreed in [1] with the objectives including:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

Rel-17 work specified multicast reception for UEs in RRC_CONNECTED state, whereas Rel-18 WID targets especially cells with a large number of UEs. In addition to the varied limitations on the network side for keeping all UEs receiving a multicast session in RRC_CONNECTED state, receiving multicast transmission in RRC_INACTIVE state by the UEs is assumed to be more power efficient [1].

In this paper, we provide an analysis of the user plane aspects to operate the feature of multicast reception in RRC_INACTIVE state including SPS, DRX and CFR aspects. Note that relevant aspects on configuration, mobility, RRC state changes and notifications are provided under our companion paper [2].

# 2	Multicast Reception in RRC_INACTIVE State via SPS

Semi-Persistent Scheduling (SPS) is supported for MBS multicast in Rel-17 for RRC_CONNECTED mode allowing the network to send some periodic MBS multicast services without PDCCH. In Rel-18, MBS multicast is supported also in RRC_INACTIVE and the intention is to use same PTM transmission for UEs in RRC_CONNECTED and UEs in RRC_INACTIVE.

In the first RAN2 meeting on Rel-18 MBS, the following agreements were made [3]:

It is supported that gNB transmit one multicast session to both UEs in CONNECTED and INACTIVE in the same cell. FFS how the gNB configures this.

The following is taken as baseline: we assume the same PDCCH/PDSCH resources (e.g. resources used for MTCH) can be used for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session. Different configuration/resources are not precluded as well. FFS what exactly can be common and what not (e.g. HARQ, SPS etc.) and what is needed in addition (to legacy PTM config).

If SPS is not supported for RRC_INACTIVE, then it would imply that SPS cannot be used in RRC_CONNECTED either in case the gNB allows MBS multicast reception in RRC_INACTIVE. This would create problems for backwards compatibility.

Observation 1: If SPS is not supported for UEs in RRC_INACTIVE state, it would imply that SPS cannot be used for UEs in RRC_CONNECTED state, which would create backwards compatibility issues with Rel-17 UEs.

Proposal 1: Support SPS for MBS multicast also in RRC_INACTIVE.

In Rel-17, a UE in RRC_CONNECTED state provides HARQ feedback when SPS activation is received. That way, the gNB can understand whether SPS activation had been successfully received by all UEs. Even in case NACK-only feedback is configured to the UE, UE replies with ACK/NACK feedback against SPS activation.

UEs in RRC_INACTIVE cannot send feedback, therefore, is it not possible for UEs in RRC_INACTIVE to acknowledge SPS activation/deactivation DCI. Thus, the network does not know whether a UE in RRC_INACTIVE is receiving the SPS transmissions sent without PDCCH. In order to increase the probability of receiving an SPS activation/deactivation, the network can repeat it.

Observation 2: Network can repeat SPS activation/deactivation DCI to increase the probability that all UEs in RRC_INACTIVE receive it.

When a UE in RRC_CONNECTED state moves and is handed over to a cell where SPS has already been activated, the network is aware of that and can repeat the SPS activation.

If a UE in RRC_INACTIVE is moving and reselects a cell where SPS reception has been activated, the UE cannot receive the SPS transmissions since the UE does not know the time/frequency resources, MCS, etc., and the network does not know that a new UE entered the cell and thus cannot repeat the SPS activation. The UE would be monitoring the configured G-CS-RNTI trying to receive an SPS activation. The UE can then receive PDCCH/DCI indicating SPS retransmission and can thus notice that SPS for that G-CS-RNTI has been activated in the cell. However, NW does not have knowledge that UE is missing SPS activation. Therefore, we propose RAN2 to study how to enable NW to know that UE receiving multicast in RRC_INACTIVE is missing SPS activation, e.g., UEs in RRC_INACTIVE could request reactivation of the SPS.

Observation 3: When UE in RRC_CONNECTED is handed over to a cell where SPS reception has been activated, the gNB can repeat the SPS activation. When UE in RRC_INACTIVE reselects a cell where SPS reception has been activated, the gNB does not know it and therefore, cannot repeat the SPS activation.

Observation 4: UE reselecting a new cell in RRC_INACTIVE may receive SPS retransmissions and thus notice the SPS has been activated in that cell, but the UE cannot receive the SPS transmissions without activation.

Proposal 2: Study how one would enable NW to know UE receiving multicast in RRC_INACTIVE is missing SPS activation.

# 3	DRX with Multicast Reception in RRC_INACTIVE State

DRX is supported for MBS multicast reception in RRC_CONNECTED state and for MBS broadcast in all RRC states. There are proposals that DRX for MBS multicast in RRC_INACTIVE should follow DRX for MBS broadcast since UEs in RRC_INACTIVE cannot send HARQ feedback. DRX for MBS broadcast does not support drx-HARQ-RTT-TimerDL-PTM or drx-RetransmissionTimerDL-PTM since HARQ retransmissions are not supported.

In Rel. 17 MBS multicast, UEs with HARQ feedback disabled (disabling the UE feedback is up to network implementation) cannot be configured with drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM and therefore, cannot receive PTM retransmissions requested by other UEs. The issue was discussed at the end of Rel. 17 [4], and although companies acknowledged the problem and clear advantages, the solution was left to Rel. 18 to be specified due to lack of time left in Rel. 17.  In Rel. 18 MBS multicast is received by the UEs in both RRC_CONNECTED and RRC_INACTIVE and UEs in RRC_CONNECTED state send HARQ feedback and the network may retransmit the packets using PTM transmission. In that case, especially if the PTM configuration is the same, i.e., same PTM transmission is received in both RRC states, then, it is beneficial for the UEs in RRC_INACTIVE state to be able to receive the potential HARQ retransmissions requested by UEs in RRC_CONNECTED state (please note that the RRC_INACTIVE UEs cannot send HARQ feedback).

Proposal 3: RAN2 enables RRC_INACTIVE UE receiving multicast to also receive possible PTM retransmissions initiated by UEs receiving multicast in RRC_CONNECTED.

Proposal 4: DRX for MBS multicast in RRC_INACTIVE should be based on DRX for MBS multicast in RRC_CONNECTED.

Since PTP transmissions are not supported in RRC_INACTIVE, drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL are not really required to be started. However, if DRX is configured for multicast, DRX retransmission timers should also be configured for the INACTIVE UE as already proposed for connected UEs whose HARQ feedback is disabled in Rel. 17 and in our TEI-18 contribution [5].

Proposal 5: Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for INACTIVE UEs (38.331).

Even if the UE is configured with the DRX retransmission timers, it may not be clear when to start the DRX HARQ RTT timer since the UE in inactive state does not send HARQ feedback, and the HARQ RTT timer is by definition started after sending HARQ feedback.

Since the INACTIVE UE is not configured with PUCCH resources and, , the UE may not understand HARQ feedback related parameters in DCI. Those parameters  are K1 (PDSCH-to-HARQ_feedback timing indicator) in the DCI pointing to PUCCH resource's slot where the HARQ feedback is supposed to be sent (if enabled) and PRI (PUCCH resource indicator) which then selects which PUCCH resources (symbol(s) and frequency resources) to use in that slot. The interpretation of K1 and PRI are configured to connected mode UE but not to inactive UE. Therefore, UE in INACTIVE does not know when the HARQ feedback would be transmitted if enabled and therefore does not know when to start the DRX HARQ RTT timer.

Observation 5: The UE in INACTIVE does not know when the HARQ feedback would be transmitted and therefore does not know when to start the DRX HARQ RTT timer.

Therefore, we propose:

Proposal 6 : dl-DataToUL-ACK-MulticastDCI-Format4-1 or dl-DataToUL-ACK to be configured also for UE in inactive, i.e., it should be provided by MCCH or via RRCRelease message to the UE, which could be per TMGI (MBS service) or a general one for all multicast services.

Proposal 7: The UE in RRC_INACTIVE state can use the existing rules for interpreting K1 (PDSCH-to-HARQ_feedback timing indicator): For DCI format 4_1, the PDSCH-to-HARQ_feedback timing indicator field (K1) values are provided by dl-DataToUL-ACK-MulticastDCI-Format4-1 or, if dl-DataToUL-ACK-MulticastDCI-Format4-1 is not provided, by {1, 2, 3, 4, 5, 6, 7, 8}

If Proposal 6, and Proposal 7 are accepted, the UE in INACTIVE would know when the HARQ feedback would be transmitted if enabled and therefore the UE knows when to start the DRX HARQ RTT timer. Hence, the UEs in INACTIVE state could start the multicast DRX timers when reception of the transport block has not been successful.Proposal 8: UE receiving MBS multicast in RRC_INACTIVE should start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM as specified in 38.321 when reception of the transport block has not been successful, but need not start drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL.

# 3	CFR Aspects

Specifically on CFR for multicast in RRC_INACTIVE, RAN2 made the following agreements in RAN2#121bis-e:

Broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE.

Case B and case D are not supported for multicast CFR in RRC_INACTIVE;

Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed.

The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs.

Based on these agreements, similar operation to broadcast CFR was agreed. There is only one CFR for multicast that is used both for MCCH and MTCH. In addition, it has been agreed that the multicast CFR for UEs in RRC_CONNECTED and RRC_INACTIVE can be configured differently depending on gNB implementation. This is logical, as not all the services in the cell are to be received by all the UEs.

However, in some scenarios, using the same DCI format 4_1/4_2 scheduling the same PDSCH/MTCH data for a specific multicast service can be problematic. Consider the following case:

Figure 1:   CFR coordination between INACTIVE and RRC Connected UEs

In the scenario shown in Fig. 1, UE-1 is receiving a multicast service 1 in RRC_CONNECTED state. Similarly, UE-2 receives another multicast service 2 in RRC_CONNECTED state. The CFRs configured for such services are slightly overlapping, but mostly separated, e.g., for the gNB to be able to schedule both services in the same slot in a more flexible way. Note that if there are some UEs interested in both services in RRC_CONNECTED, the gNB would configure a CFR that is the union of dark blue and dark green for both groups. Therefore, the assumption herein is that there are no UEs interested in both services in RRC_CONNECTED state. This is a possible assumption, considering the number of different use cases of MBS, e.g., one service could be for public safety and the other could be V2X.

So, for the use case illustrated by Figure 1, UEs-1/2 would compute the frequency domaing resource allocation (FDRA) field size of the multicast DCI using their own CFR configurations (size and starting point of CFR). In addition, when a DCI is received, the UE-1/2 would interpret the DCI fields (including the FDRA field) again using their own CFR configuration. The annex provides the text from TS38.212 and TS38.214 on how CFR information is used.

Now, assuming that the same services would be available to the UEs in RRC_INACTIVE state, the gNB needs to configure one CFR for UEs in RRC_INACTIVE state (note that RAN2 agreed to have one CFR for MCCH and MTCH as in Rel-17 broadcast, therefore, all the UEs should have the same CFR configuration to be able to receive at least the common MCCH). However, the RRC_INACTIVE CFR should now include both CFR for service 1 and 2.

Observation 6: 	In some scenarios, the size of the CFR for UEs in RRC_CONNECTED and RRC_INACTIVE interested to receive the same service cannot be aligned.

Due to misalignment in the CFRs, the UEs in RRC_INACTIVE state will interpret both the DCI (field) size and certain contents of the DCI differently, in particular any field defined relative to the RRC_CONNECTED mode CFR.

Observation 7: 	RRC_CONNECTED and RRC_INACTIVE UEs that are interested in the same multicast service cannot have the same DCI understanding of certain DCI fields, when their CFRs are not the same.

One way to overcome the problem could be providing the UE in RRC_INACTIVE state with the needed RRC_CONNECTED CFR relevant values for DCI field/size computation per multicast service, so that the UE computes those using the provided values, rather than making computations based on the RRC_INACTIVE CFR. The other option could be indicating the CFR to be used for such computations for DCI per multicast service (e.g., full CFR configuration of RRC_CONNECTED UEs; or starting point and size of the RRC_CONNECTED CFR) .

Proposal 9: 	RAN2 works on solutions for FDRA size and interpretation alignment, for the case where the same DCI and PDSCH are to be received by UEs in RRC_INACTIVE and RRC_CONNECTED states for the same multicast service when the CFRs for the UEs in different RRC states are not aligned.

# 4	Conclusion

This document has made the following observations:

Observation 1: If SPS is not supported for UEs in RRC_INACTIVE state, it would imply that SPS cannot be used for UEs in RRC_CONNECTED state, which would create backwards compatibility issues with Rel-17 UEs.

Observation 2: Network can repeat SPS activation/deactivation DCI to increase the probability that all UEs in RRC_INACTIVE receive it.

Observation 3: When UE in RRC_CONNECTED is handed over to a cell where SPS reception has been activated, the gNB can repeat the SPS activation. When UE in RRC_INACTIVE reselects a cell where SPS reception has been activated, the gNB does not know it and therefore, cannot repeat the SPS activation.

Observation 4: UE reselecting a new cell in RRC_INACTIVE may receive SPS retransmissions and thus notice the SPS has been activated in that cell but the UE cannot receive the SPS transmissions without activation.

Observation 5: The UE in INACTIVE does not know when the HARQ feedback would be transmitted if enabled and therefore does not know when to start the DRX HARQ RTT timer.

Observation 6: In some scenarios, the size of the CFR for UEs in RRC_CONNECTED and RRC_INACTIVE interested to receive the same service cannot be aligned.

Observation 7: RRC_CONNECTED and RRC_INACTIVE UEs that are interested in the same multicast service cannot have the same DCI understanding of certain DCI fields, when their CFRs are not the same.

And proposed the following:

Proposal 1: Support SPS for MBS multicast also in RRC_INACTIVE.

Proposal 2: Study how one would enable NW to know UE receiving multicast in RRC_INACTIVE is missing SPS activation.

Proposal 3: RAN2 enables RRC_INACTIVE UE receiving multicast to also receive possible PTM retransmissions initiated by UEs receiving multicast in RRC_CONNECTED.

Proposal 4: DRX for MBS multicast in RRC_INACTIVE should be based on DRX for MBS multicast in RRC_CONNECTED.

Proposal 5: Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM  for INACTIVE UEs (38.331).

Proposal 6 : dl-DataToUL-ACK-MulticastDCI-Format4-1 or dl-DataToUL-ACK to be configured also for UE in inactive, i.e., it should be provided by MCCH or via RRCRelease message to the UE, which could be per TMGI (MBS service) or a general one for all multicast services,

Proposal 7: The UE in RRC_INACTIVE state can use the existing rules for interpreting K1 (PDSCH-to-HARQ_feedback timing indicator): For DCI format 4_1, the PDSCH-to-HARQ_feedback timing indicator field (K1) values are provided by dl-DataToUL-ACK-MulticastDCI-Format4-1 or, if dl-DataToUL-ACK-MulticastDCI-Format4-1 is not provided, by {1, 2, 3, 4, 5, 6, 7, 8}

Proposal 8: UE receiving MBS multicast in RRC_INACTIVE should start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM as specified in 38.321 when reception of the transport block has not been successful, but need not start drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL.

Proposal 9: 	RAN2 works on solutions for FDRA size and interpretation alignment, for the case where the same DCI and PDSCH are to be received by UEs in RRC_INACTIVE and RRC_CONNECTED states for the same multicast service when the CFRs for the UEs in different RRC states are not aligned.

# Annex How CFR information is Used for Calculations of DCI Relevant Fields in TS38.212 and TS38.214

##### 7.3.1.5.3	Format 4_2

The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI:

-	Frequency domain resource assignment – number of bits determined by the following, where NRBDL,CFR is the size of the common frequency resource as configured by higher layer parameter locationAndBandwidth-Multicast:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	log2(NRBDL,CFR(NRBDL,CFR+1)2 bits if only resource allocation type 1 is configured, or

-	maxlog2(NRBDL,CFR(NRBDL,CFR+1)2,NRBG+1 bits if resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch'.

-	If resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2(NRBDL,CFR(NRBDL,CFR+1)2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

##### 5.1.2.2.1	Downlink resource allocation type 0

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured by PDSCH-Config and the size of the bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs (NRBG) for a downlink bandwidth part i of size NBWP,isize PRBs is given by NRBG=NBWP,isize+NBWP,istartmodP/P, where

-	the size of the first RBG is RBG0size=P-NBWP,istartmodP,

-	the size of last RBG is RBGlastsize=NBWP,istart+NBWP,isizemodP if NBWP,istart+NBWP,isizemodP>0 and P otherwise,

-	the size of all other RBGs is P.

In downlink resource allocation of type 0 scheduled using a DCI with CRC scrambled by G-RNTI for multicast or G-CS-RNTI, the resource block assignment information bitmap is calculated based on the description above with the following changes: the parameter NBWP,istart is the starting PRB of the CFR, NBWP,isize is the size of the common frequency resource (CFR) and the value of the higher layer parameter rbg-Size is configured by pdsch-ConfigMulticast.

##### 5.1.2.2.3	Downlink resource allocation type 1 for multicast/broadcast

In downlink resource allocation of type 1 scheduled using DCI format 4_0 or DCI format 4_1 with CRC scrambled by G-RNTI, G-CS-RNTI or MCCH-RNTI, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks.

A downlink type 1 resource block assignment field in the DCI format 4_0 or DCI format 4_1 consists of a RIV corresponding to a starting resource block in reference to the lowest RB of the CFR and a length in terms of virtually contiguously allocated resource blocks LRBs, where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell;

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

The resource indication value is defined by:

if  then

else

where, and where shall not exceed .

If NCFR>NBWPinitial, K is the maximum value from set {1, 2, 4, 6, 8, 10, 12} which satisfies K≤NCFR/NBWPinitial; otherwise K = 1.

In downlink resource allocation of type 1 scheduled using DCI format 4_2 with CRC scrambled by G-RNTI for multicast or G-CS-RNTI, the description in clause 5.1.2.2.2 with the following changes:  corresponds to a starting resource block in reference to the lowest RB of the CFR and  is the size of the CFR.

# References

- RP-221458, “Revised WID: Enhancements of NR Multicast and Broadcast Services”.

- R2- 2307768, “Control plane aspects of multicast reception in RRC_INACTIVE state”, RAN2#123.

- R2-2208707, “Report from MBS breakout session”, RAN2#119-e.

- R2-2306392, “PTM retransmission reception by UEs without HARQ feedback”, RAN2#122.

- R2-2307974, ‘PTM retransmission reception by UEs without HARQ feedback’, RAN2#123