3GPP TSG-RAN WG2 Meeting #123	R2-2308011

Toulouse, France, August 21-25, 2023

Agenda item:		7.7.4.1.2

Source:		Lenovo

Title:		Feeder link switch time and reference location of NTN moving cells

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding IDLE/INACTIVE mobility for moving cells in NR NTN:

RAN2#121 agreements - In R18, for earth-moving system, satellite with steerable beam is not considered as part of mobility enhancement in NTN. - A serving cell reference location and a distance threshold/radius will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). - For cell selection/reselection, location-based measurement initiation is supported in earth-moving cell - For earth-moving cell, the location-based cell measurement rules of quasi-fixed cell is reused, i.e., for cell reselection in earth-moving cell, UE initiates measurements when its location to serving cell reference location is larger than the configured distance threshold. RAN2#121bis agreements - RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements. - For earth-moving cell, new IE is introduced to indicate the reference location of serving cell. - For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19. - For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case. - Time-based cell reselection criteria is not pursued in R18. RAN2#122 agreements - Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location. - Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one. - Location-based cell reselection criteria are not pursued in R18.

In this contribution we further discuss potential solutions for neighbour cell measurement triggering in moving cells based on above agreements.

- Discussion

Indication of feeder link switch time

Indication of the feeder link switch time to UE has been agreed in previous meetings reusing the format of t-Service, and the only remaining issue is whether to define a new IE name or reuse t-Service by only updating the field description.

The situations are a bit different for quasi-earth-fixed and earth-moving cells. In quasi-earth-fixed cells, cell stop serving time and feeder link switch time are both cell-specific (i.e. common for all UEs in the same serving cell), and only one parameter is sufficient as network can configure the value of t-Service as the earlier one between cell stop serving time and feeder link switch time. For earth-moving cells, cell stop serving time is UE/location-specific while feeder link switch is cell-specific, and the network may not determine which one is earlier for a UE/location. From specification perspective, the following 2 options are workable:

- Option A): Feeder link switch time of an earth-moving cell is indicated by a new IE (e.g., t-FeederLinkSwitch), while t-Service is still used to indicate cell stop serving time at specific locations or absent.

- Option B): Feeder link switch time of an earth-moving cell is indicated by t-Service, while cell stop serving time is derived by UE.

Observation 1: In earth-moving cells wherein cell stop serving time is UE/location-specific while feeder link switch is cell-specific, it could be unclear to use one parameter to indicate two possible variables.

To make the descriptions clear and avoid possible confusion in future (e.g., in case cell stop serving time and feeder link switch time need to be treated separately), Option A) is preferred. In this case for quasi-earth-fixed cells, network can determine which one to be provided; and for earth-moving cells, network provides feeder link switch time as one of the reference time before which neighbour cell measurement shall be initiated.

Proposal 1: A new optional IE is introduced to indicate the feeder link switch time for NTN cells.

In Rel-17 NR NTN, reference location referenceLocation of serving cell is broadcast in SIB19, which is fixed for a quasi-earth-fixed cell during before its stop serving time. For an earth-moving cell with varying referenceLocation over time, frequent UE re-acquiring (e.g., triggered by SI change) will cause more power consumption. It could likely be similar to the ephemeris handling in Rel-17 NTN wherein no SI change is triggered by ephemeris change. Therefore an issue arises as how to indicate the moving referenceLocation of an earth-moving cell to the UE.

In the discussion one agreement is that the trajectory of serving cell reference location can be derived by UE with rough accuracy solely based on satellite’s ephemeris and epoch time. In our understanding, such accuracy is uncertain and meanwhile more calculation needs to be done at UE, see in Figure 1. Firstly, the trajectory of satellite represented by ephemeris is an ellipse which is not parallel to the earth, meaning that the velocity of satellite does not equal to the velocity of its projection (e.g., cell reference location) on earth. Secondly, even the projection is derived, it only represents the nadir point of the satellite which could be the reference location of one cell, while the reference locations of the other cells generated by the same satellite could not be represented or derived, e.g., the elevation angle between the reference location of a cell and the satellite, which is not part of ephemeris, also affects the velocity of the cell reference location.

Figure 1. The trajectory of satellite and cell reference location

Observation 2: Deriving the reference location of an Earth-moving cell only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

Meanwhile, it has also been agreed that a new IE is introduced to indicate the reference location of serving cell, which allows more space in content design to improve the accuracy and ease UE’s power consumption for calculation. Therefore additional change information, e.g., drift or velocity vector of the reference location can be considered to help UE estimate real-time reference location accurately.

Proposal 2: For earth-moving cells, change information (e.g., drift or velocity vector) of reference Location can be provided.

- Conclusion

In this contribution, we provide further issue analysis of IDLE/INACTIVE mobility in NTN considering moving cells. The following observation are given:

Observation 1: In earth-moving cells wherein cell stop serving time is UE/location-specific while feeder link switch is cell-specific, it could be unclear to use one parameter to indicate two possible variables.

Observation 2: Deriving the reference location of an Earth-moving cell only by the ephemeris data at epoch time has uncertainty in accuracy and introduces more power consumption for UE’s calculation.

And we propose:

Proposal 1: A new optional IE is introduced to indicate the feeder link switch time for NTN cells.

Proposal 2: For earth-moving cells, change information (e.g., drift or velocity vector) of reference Location can be provided.