3GPP TSG-RAN WG2 Meeting #123	R2-2308015

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 7.13.4

Source:		Lenovo

Title:		Discussion on inter-RAT SHR from NR to LTE

Document for:		Discussion and Decision

1	Introduction

In RAN2#120 meeting [1], agreements on SON enhancements for inter-RAT SHR were achieved:

Agreements:

1	For Q5 in R2-2211160, RAN2 confirms the support for the parameters for inter-RAT SHR from NR to LTE when T310 and T312 are configured as triggering condition.

2	T304 trigger for inter-RAT SHR from NR to LTE is not supported.

RAN2#120 meeting also agreed to prioritise inter-RAT HO from NR to LTE first, inter-RAT HO from LTE to NR can be considered after that.

In RAN2#121 meeting [2], further agreements were achieved:

Agreement:

1: For Q1 in the LS R2-2211160, RAN2 agrees to reduce/avoid the impact on LTE specification to support inter-RAT SHR.

2: For handover from NR to LTE, UE generates the NR SHR when SHR for inter-RAT mobility is triggered due to T310 or T312 trigger threshold is fulfilled.

3: For HO from NR to LTE, UE records the SHR for inter-RAT mobility in the VarSuccessHO-Report.

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

5:  A new EUTRA target cell CGI is introduced in inter-RAT SHR.

6: For HO from NR to LTE, the T310 and T312 threshold is provided to the UE by source gNB in the otherConfig.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

RAN2#122 meeting [3] agreed that intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

In this contribution, we would focus on inter-RAT SHR from NR to LTE.

2	Discussion

For inter-RAT SHR from NR to LTE, only T310 and T312 related triggers are to be considered. Based on RAN3#119 meeting’s agreements, the receiving node forwards the inter-RAT SHR to source NR node when T310/T312 triggers the inter-RAT SHR. The source NR node needs to identify UE context when receiving the inter-RAT SHR. RAN3#120 meeting discussed this issue, and sent an LS [4] to RAN2, mentioning that:

RAN3 discussed how to retrieve mobility information related to a UE in the nodes involved in the mobility procedure when Successful Handover Report (SHR) and Successful PSCell Change Report (SPR) related optimizations are to be performed in the network nodes involved in the mobility procedure. One solution discussed was whether UE can report the source C-RNTI and time since receiving HO/PSCell change/addition command and retrieving SHR/SPR to assist in the network node receiving the SHR/SPR identifying information related to a UE (e.g. UE related configuration of interest, mobility strategies).

But since the possibility to retrieve information related to a specific UE depends on gNB's implementation. e.g. whether it can store the UE related configuration of interest during the wanted time (up to 48 hours) after a successful HO or PSCell change/addition, RAN3 discussed an alternative solution to enable the retrieval of UE related information without requiring this to be stored on a per UE basis.

RAN3 is therefore studying whether another solution can be defined for newly defined scenarios since Rel-18 as well as intra-RAT SHR, whenever the configuration used by the UE needs to be retrieved in order to perform SON optimizations.

One potential solution RAN3 discussed is as follows:

- The node involved in mobility procedure (via implementation) create references to a configuration or a set of parameters used by one or a group of UEs and can send this “Configuration Information” to the UE in order to assist in the analysis of SHR or SPR or other SON reports (if needed).

- this “Configuration Information” can be optionally sent to the UE in dedicated signaling (e.g., together with the SHR/SPR configuration or in any other RRCReconfiguration). If received, UE should then store this “Configuration Information” together with the SON report(s) and UE should report it back to the gNB along with the SON report(s) (e.g., SHR/SPR).

- How to encode this “Configuration Information” is up to RAN2, taking into account RAN3 guidance related to the size of this information. One example to do this would be to encode this as an OCTET STRING (e.g., 32 bits) as is done for Mobility Information in XnAP (i.e., TS 38.423).

- This solution shall cover all existing mobility scenarios, including dual-connectivity scenarios.

We can find that based on RAN3’s LS, there are two options on table,

- Option 1: the UE includes source C-RNTI and time since receiving HO command and retrieving SHR in the SHR, based on these two parameters, source NR node identifies UE context.

- Option 2: the source RAN node sends “Configuration Information” (e.g. a configuration or a set of parameters used by one or a group of UEs) to the UE, then the UE include this “Configuration Information” in SHR.

In the LS, RAN3 asked several questions to RAN2, including:

- Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above?

- Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command?

- Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

Currently, last serving PCell ID, C-RNTI and time information (e.g. timeSinceFailure) are included in the RLF report, based on these information reported by the UE, network can retrieve the UE context. We can find that using C-RNTI and time related information to identify UE context is a usual way.

This principle for RLF report can be applied to SHR without any problem, i.e. in Option 1, the UE includes source C-RNTI and time since receiving HO command and retrieving SHR in the SHR, it is a simple and straightforward solution to enable source RAN node to retrieve the UE context or the configuration used for the UE, e.g. by mapping the source C-RNTI to a specific UE context in the specific time period.

From RAN2 point of view, Option 2 i.e. defining a solution for “Configuration Information” seems complex and unnecessary, and the benefit of using “Configuration Information” is not clear. In Option 2, RAN2 needs to encode and design “Configuration Information”, the time left for Rel.18 SON WI is limited, but Option 2 needs more technical discussion and specification impacts.

Proposal 1: For retrieval of UE context at source RAN node during inter-RAT HO from NR to LTE, the UE can include source C-RNTI, and time between HO command and SHR retrieval in the inter-RAT SHR.

In R17, target C-RNTI is introduced in intra-NR SHR, obviously it is used to correlate NR SHR (which is triggered due to T304 trigger condition is fulfilled) and NR RLF Report in case that there is a RLF shortly after a successful intra-NR HO, i.e. when T304 triggers intra-NR SHR, the receiving node forwards the intra-NR SHR to the target gNB, based on the target C-RNTI which is included in both intra-NR SHR and NR RLF Report, the target gNB can performs correlation without any spec impact.

Observation 1: When T304 triggers intra-NR SHR, the target gNB can correlate the intra-NR SHR and NR RLF report based on target C-RNTI.

For inter-RAT SHR, another issue is in case there is a RLF shortly after a successful inter-RAT HO from NR to LTE, whether to support correlation of NR RLF report and LTE SHR, and if yes whether any UE assistance is needed to support this correlation. Simailar as intra-NR SHR, we think correlation of NR RLF report and LTE SHR is needed since it is beneficail for MRO analysis and optimisation at network side to avoid mistakes. The solutions for network correlation are as below:

- Option A: source gNB performs correlation based on target C-RNTI. In Option A, target C-RNTI is needed in inter-RAT SHR even source gNB can’t understand target C-RNTI, target C-RNTI is just used as a reference to enable correlation, i.e. source gNB can identify that the two reports are for a same handover procedure for a same UE when a same target C-RNTI is included in both inter-RAT SHR and LTE RLF Report.

- Option B: source gNB performs correlation based on source C-RNTI. In Option B, source C-RNTI is needed in inter-RAT SHR. Since target node would explicitly transfer source C-RNIT to source node when forwarding RLF report to source node, source gNB can correlate inter-RAT SHR and LTE RLF Report, based on the source C-RNTI which is received in the inter-RAT SHR and explicitly received from target eNB.

The spec impact of Option A is to include target C-RNTI in inter-RAT SHR, and the spec impact of Option B is to include source C-RNTI in inter-RAT SHR, since source C-RNTI can also be used by the source gNB for UE context retrieval besides correlation, Option B is preferred.

Observation 2: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF report in case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

Proposal 2: Source C-RNTI can be included in the inter-RAT SHR from NR to LTE for correlating inter-RAT SHR and LTE RLF report, in case that there is a RLF shortly after a successful HO.

3	Conclusion

In this contribution, the details of iner-RAT SHR from NR to LTE are discussed. We have the following observations and proposals:

Observation 1: When T304 triggers intra-NR SHR, the target gNB can correlate the intra-NR SHR and NR RLF report based on target C-RNTI.

Observation 2: When T310/T312 triggers inter-RAT SHR from NR to LTE, source gNB correlates inter-RAT SHR and LTE RLF report in case that there is a RLF shortly after a successful inter-RAT HO from NR to LTE.

Proposal 1: For retrieval of UE context at source RAN node during inter-RAT HO from NR to LTE, the UE can include source C-RNTI, and time between HO command and SHR retrieval in the inter-RAT SHR.

Proposal 2: Source C-RNTI can be included in the inter-RAT SHR from NR to LTE for correlating inter-RAT SHR and LTE RLF report, in case that there is a RLF shortly after a successful HO.

Reference

[1] Chairman notes of RAN2#120 meeting

[2] Chairman notes of RAN2#121 meeting

[3] Chairman notes of RAN2#122 meeting

[4] R3-233380, LS on SHR and SPR, Samsung