3GPP TSG-RAN WG2 Meeting #123	R2- 2308030

Toulouse, France, August 21-25, 2023

Agenda item:		7.20.2

Source:		Lenovo

Title:		Consideration on RLF in multi-TRP operation

Document for:		Discussion and Decision

# Introduction

The multi-TRP related discussion has been kicked off and some agreements have been achieved in RAN2#122 [1].

Agreement

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

RAN2 confirm the following working assumption as an agreement:

- Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles:

    - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs).

- The following information can be indicated by the MAC CE (for joint DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if  the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

FFS for the separate DL/UL TCI mode.

In this contribution, we discuss the potential RLF for multi-DCI multi-TRP operation in New Radio (NR).

# Discussion

Random Access problem

For multi-TRP operation with 2TAs, RAN1 is discussing PDCCH ordered RACH and RAN2 is discussing UE-initiated RACH. Referring to the email discussion [2], the RACH procedure corresponds to one TAG. While, as we known, it is concluded in RAN2#121bis that per TRP UE-initiated RACH procedure is not supported. Then it is possible that the RACH associated with TAG#1 is triggered for SR and the UL associated with TAG#2 is synchronized.

If the PREAMBLE transmission reaches the configured max number, what shall be the UE behavior? In legacy, the RLF is detected by the indication of the Random Access problem to upper layers. Considering the synchronized UL associated with TAG#2, it seems not necessary to declare RLF. And there is another NTA, offset for PRACH preamble transmission or RACH resource is applied for PRACH. It seems there is potential chance for another RACH.

Proposal 1: RAN2 to discuss whether to declare RLF if the random access problem is indicated while there is synchronized UL TRP.

Proposal 2: RAN2 to discuss how to model the problem indication for the Random Access problem e.g. per TAG.

RLC reTX

A logical channel could be associated with a configured grant by the parameter allowedCG-List. The configured grant can be associated with one of the multiple TRPs corresponding to SSBs in multi-TRP operation. Then, the logical channel can be associated to the TRP. The max number of RLC retransmission of the logical channel can be considered as reached on the TRP. If following the legacy behaviors, the RLF is detected by the indication of max retransmission being reached to upper layers. It may be not necessary to trigger RLF since there is one UL on the other TRP can work.

Proposal 3: RAN2 to discuss whether to declare RLF if the RLC max retransmission is reached while there is synchronized UL TRP.

# Conclusion

In this contribution, we discuss the potential RLF for multi-DCI multi-TRP operation in NR and provide the following proposals:

Proposal 1: RAN2 to discuss whether to declare RLF if the random access problem is indicated while there is synchronized UL TRP.

Proposal 2: RAN2 to discuss how to model the problem indication for the Random Access problem e.g. per TAG.

Proposal 3: RAN2 to discuss whether to declare RLF if the RLC max retransmission is reached while there is synchronized UL TRP.

# References

RAN2#122 chairman notes

RAN2#122 email discussion [Post122][852][MIMOevo] RAN2 impacts of 2TAs for multi-DCI multi-TRP (Samsung)