3GPP TSG-RAN WG2 Meeting #123	R2-2308032

Toulouse, France, 21 - 25 August, 2023

Title: 	Remaining issues on RACH-less HO in NTN

Source: 	Huawei, Turkcell, HiSilicon

Agenda item:	7.7.4.2

Document Type:	Discussion and Decision

- Introduction

In RAN2 #121bis-e, an LS was sent to RAN1 asking about the beam related aspects and power control aspects for initial UL transmission during RACH-less HO [1]. In the reply LS, RAN1 indicated that they will further discuss power control for initial UL transmission, and the responses related to beam selection did not provide RAN2 with a clear conlusion [2].

Furthermore, there is an open issue on the confirmation of RACH-less HO completion in RAN2 #122 [3]:

Agreements: - In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0 - From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO - Release pre-allocated UL grant after RACH-less HO completion - LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI - Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

In this paper, we will continue discussing the remaining issues of beam selection and HO completion.

- Discussion

  - Beam selection during RACH-less

- Pre-allocated grant

The first issue is whether the pre-allocated grant is provided with association to SSB(s) and whether an RSRP threshold is needed for SSB selection for initial UL transmission.

The issue was raised to RAN1 and RAN1 response is as follows [2]:

1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission? RAN1 response: One company thinks that when the network knows the suitable DL beam for RACH-less handover, the pre-allocated grant can be associated with a SSB index of the target cell, and when the network does not know the suitable DL beam, RACH-based HO can be used instead of introducing beam-sweeped pre-allocated grants associated with multiple SSB indexes. Other companies think that the association between the pre-allocated grant for initial transmission and SSB index should be supported without any condition(s), and think that RSRP threshold may be helpful.

During RAN1 discussion [4], one company thinks that RACH-based HO should be considered instead of RACH-less HO if gNB has no idea of the preferred beam direction because the association mechanism would require more candidate PUSCH resources for beam sweeping in case of RACH-less HO. Majority of companies consider it beneficial to provide the association between pre-allocated grant and SSB index, and the RSRP threshold.

From our perspective, the association and RSRP threshold are needed:

- Whether gNB is always able to know the preferred beam for a UE during RACH-less handover?

We think in most cases, the gNB can know the suitable beam based on L3 measurement report (with beam-level results) or the UE location report. However, there are also cases where the gNB has no such information, e.g. blind HO or no location reporting. Also, the suitable beam may be changed before or during the RACH-less HO process, since the grant is pre-allocated (especially in case of CHO combined with RACH-less where the timepoint to trigger HO is uncertain). Therefore, the beam selection by the UE is needed. By configuring the association between pre-allocated grant and SSB, the gNB can know the beam selected by the UE and use the corresponding beam to schedule subsequent transmissions. During the beam selection process, the SSB RSRP threshold is important to ensure reliable intial UL transmission.

- UE behavior if no SSB is above a threshold

If at least one SSB with SS-RSRP above the RSRP threshold is available, UE can select the PUSCH resource associated with the SSB for the initial UL transmission. If there are multiple qualified SSBs whose SS-RSRP are above the threshold, it’s up to UE implementation to decide which SSB is used for initial UL transmission. UE can inform target cell the selected beam implicitly by the selected PUSCH resources.

However, if no measured SSB has an SS-RSRP above the RSRP threshold, one solution is that UE selects the PUSCH resource associated with any measured SSB to perform initial UL transmission, and the alternative is that network also configures RACH resources (CFRA or CBRA) in the HO command and in this case UE performs random access procedure. We slightly prefer the latter because 1) UE can perform power ramping during random access procedure; 2) it is unclear for the moment whether the association between pre-allocated grant and SSB includes all the SSBs or only partial of them, while in RACH-based procedure all SSBs can be considered.

Proposal 1: NW provides the association between the pre-allocated grant and SSBs and an RSRP threshold.

Proposal 2: When configuring the UE to perform RACH-less HO, the NW also configures RACH resources in the HO command. If no SSB has an SS-RSRP above the threhsold, UE performs RACH-based handover.

- Dynamic grant

If the uplink grant is acquired by monitoring target cell’s PDCCH, beam selection is also needed. There are four  options as follows:

- Option 1: UE and NW select beam separately

On the UE side: During DL sync with the target cell, UE determines an appropriate SSB based on the signal quality and uses this beam for monitoring PDCCH of the target cell.

On the NW side: Based on the measurement results including beam results forwarded by the source cell, the target cell can determine one best or several good beams for transmitting dynamic grant.

The advantange is less spec impact. The disadvantage is that UE and network may choose different beams, but this can be solved by network transmitting PDCCH on multiple beams.

- Option 2: NW selects beam and signals to UE

Target cell determines the beam based on the measurement results including beam result forwarded by source cell, and indicates the selected beam index to the UE in HO command. If no beam indication is provided in the HO command, the target cell can transmit PDCCH on all the beams and UE monitors on all the beams. The advantage of the option 2 is the guaranteed beam alignment between the network and UE. And the disadvantage is that if UE is not under the coverage of the indicated beam due to UE or satellite movement, UE can’t receive the PDCCH from the network. Option 2 introduces more spec impact than Option1.

- Option 3:  Blind transmit and monitor

Regardless of Option1 or Option2, if the target cell doesn’t obtain the measurement result from the source cell (e.g. in case of blind HO where the source cell has not configured the UE to measure target cell), the target cell can’t determine the suitable beam. Hence, the target cell can only transmit the PDCCH on all the beams. UE can monitor PDCCH on the appropriate beam, e.g., the beam of best RSRP which is selected during DL synchronization. This approach has no spec impact, but leads to a waste of resources at the NW side.

- Option 4: Associating SRs with SSBs

The NW sends scheduling request configuration to the UE (generated by the target cell and included in the HO command), where SR resources are in a one-to-one correspondence with SSBs. UE selects an appropriate SSB beam based on the quality of beam, then the UE sends a scheduling request associated with the selected SSB to the target cell. The target cell can determine the SSB selected by the UE by the association between the SSB and the SR, and use the corresponding beam for transmitting dynamic grant to the UE. UE can release SR configuration after handover.

Proposal 3: Regarding beam selection for monitoring dynamic beam, downselect from the following options:

- Option1: UE and NW select beam separately

- Option2: NW selects beam and signals to UE

- Option3: Blind transmit and monitor

- Option4: Associating SRs with SSBs

- Hybrid UL resource allocation

The two ways of assigning UL grant, i.e., via pre-allocation or via PDCCH monitoring, both have pros and cons. If the target gNB pre-allocates the UL grant in each beam, it will waste more UL resources; if the target cell allocates UL grant by PDCCH, the handover delay maybe longer than pre-allocated resource method. RACH-based also has its own demerits, e.g., RACH collision, longer access delay etc.

Hence we think RAN2 can consider a hybrid configuration, e.g., some beams are associated with pre-allocated grants and other beams are used for monitoring dynamic grants.

Proposal 4: RAN2 considers a hybrid configuration of UL resource allocation.

  - Confirmation of RACH-less completion

In last RAN2 meeting, RAN2 has agreed that LTE approach of confirming the HO completion is reused for both pre-allocated grant and dynamic grant, and FFS any enhancement to the confirmation of RACH-less HO completion.

Agreement: - LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

The intention of such enhancement is to save the signalling overhead of the MAC CE and directly schedules DL data for the UE. If the UE receives PDCCH addressed to the C-RNTI scheduling PDSCH, the UE can know the HO is successfully completed. However, there are also cases where the PDCCH is used to schedule PUSCH for MSG5 (re) transmission, and under this circumstance the UE cannot declare completion of the HO. Therefore, additional UE complexity is added in terms of determining whether the PDCCH is for DL assignment or UL grant. Furthermore, if there is no immediate DL data, the NW still needs to use the MAC CE to confirm the HO completion. The gain of saving a MAC CE (only when there is immediate DL data) does not justify the additional complexity of the UE.

Proposal 5: No need to introduce any enhancement to the confirmation of RACH-less HO completion.

- Conclusion

In this paper, we discuss RACH-less HO, and provide the following proposals:

Proposal 1: NW provides the association between the pre-allocated grant and SSBs and an RSRP threshold.

Proposal 2: When configuring the UE to perform RACH-less HO, the NW also configures RACH resources in the HO command. If no SSB has an SS-RSRP above the threhsold, UE performs RACH-based handover.

Proposal 3: Regarding beam selection for monitoring dynamic beam, downselect from the following options:

- Option1: UE and NW select beam separately

- Option2: NW selects beam and signals to UE

- Option3: Blind transmit and monitor

- Option4: Associating SRs with SSBs

Proposal 4: RAN2 considers a hybrid configuration of UL resource allocation.

Proposal 5: No need to introduce any enhancement to the confirmation of RACH-less HO completion.

- Reference

- R2-2304271, LS on RACH-less Handover, RAN2 (contact company: Samsung)

- R1-2306217, Reply LS on RACH-less Handover, RAN1 (contact company: Samsung)

- R2-2306541, Report from Break-Out Session on NR NTN and IoT NTN, Vice Chairman (ZTE)

- R1-2306152, Moderator summary on reply LS on RACH-less handover, Samsung