3GPP TSG-RAN WG2 Meeting #123      	                          R2-2308037

Toulouse, France, August 21-25, 2023

Agenda Item:	7.4.2.1

Source: 	OPPO

Title:  	Discussion on early sync and RACH-less for LTM

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 has triggered the discussion regarding to PDCCH ordered early RACH based on RAN1 LS in. And the following agreements have been reached:

- For early TA acquisition for candidate Cells

For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network).

RAN2 doesn’t see a need for a solution with RAR in for Rel-18.

Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18

The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed).

In this contribution, we would like to provide our views on the potential RAN2 impacts on TA sync and RACH-LESS for LTM.

Discussion

Early synchronization

To reduce the latency caused by synchronization procedure, RAN1 has agreed to support early synchronization for TA acquisition of a candidate cell before cell switch command is received. PDCCH ordered-CFRA is performed for early synchronization to candidate cell(s) and RAN2 further confirmed that without RAR is prioritized in Rel-18.

In legacy RACH procedure, UE performs carrier selection as well BWP operation during Random Access procedure initialization. If the carrier to use for the Random Access procedure is explicitly signalled, UE selects the signalled carrier for performing Random Access procedure. For PDCCH-ordered early sync procedure, if the RACH resource is configured on both NUL and SUL, NW can follow the same way as legacy PDCCH-ordered RACH procedure, i.e., NUL/SUL is explicitly indicated in PDCCH order.

For BWP operation, upon initiation of the Random Access procedure on a serving Cell, after the selection of carrier for performing Random Access procedure, UE shall perform the Random Access procedure on the active BWP for the selected carrier of this serving Cell. While there is no active BWP for non-serving cell. For PDCCH-ordered early sync, it is questionable on how to perform the BWP operation on a non-serving cell. As RAN1 has agreed to trigger the early sync procedure towards candidate cell based on PDCCH order. And the candidate cell index is also indicated by the PDDCH order to determine cell-specific early sync resources. For the BWP, we understand the PDDCH order can also be used for BWP indication. With the received PDCCH order, UE selects the RACH resources on the indicated BWP of the associated candidate cell, and performs early sync procedure.

For PDCCH ordered early sync, NUL/SUL is indicated in PDCCH order.

For PDCCH ordered early sync, BWP info is indicated in PDCCH order.

RACH collision

Another open issue for early sync is about RACH collision. In legacy, if a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure. For LTM, RACH procedures toward serving cell and candidate cells may cause RACH storm. Stop the ongoing RACH towards serving cell may cause service interruption, and candidate cell early TA acquisition failure will not impact the service continuity. Thus, RACH towards serving cell should be prioritized if there is concurrent RACH to candidate.

RACH towards serving cell is prioritized if there is concurrent RACH for early TA acquisition.

And RAN2 may need to specify the UE behavior clearly since the priority of RACH towards serving cell and candidate cell is different. For example:

- If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

- If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH procedure.

- If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH trigger.

If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new RACH procedure.

RACH-LESS LTM

One more issue is how to declare the LTM completion. In R14, RACH-Less HO is considered as successfully completed if UE receives UE contention resolution MAC CE during T304 running. For RACH-less LTM, we agreed that the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

- For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

Reception of new UL transmission scheduling for the same HARQ process as the one used for first UL transmission can be an option, i.e., UE determines the first UL has been successfully transmitted if the NDI is toggled. While the reception of new transmission scheduling can not be guaranteed, waiting for the schedulling of new transmission may lead to latency on declaring a LTM completion, thus an explicit DL message is required. In our understanding, cell switch can be considered as completed if a DL message for LTM success indication confirmation is received, e.g., contention resolution MAC CE as in R14 RACH-Less HO can be reused or RAN2 can further discuss which DL message is utilized for LTM success indication confirmation.

For RACH-Less LTM, handover is considered as completed if a UE contention resolution MAC CE is received.

In RAN2#123 meeting, RAN2 agreed to focus on “without RAR” early sync procedure as TA of candidate cell can be indicated in LTM cell switch command.

For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network).

RAN2 doesn’t see a need for a solution with RAR in for Rel-18.

With the PDCCH order triggered by early sync, UE performs preamble transmission to assist candidate cell to obtain the TA info. Then NW triggers LTM cell switch with the previously obtianed TA. As we know, the TA is determined by candidate/target cell, and the cell switch is triggered by souce serving cell. One potential issue is that the source serving cell may include the TA info that indicated by candidate/target cell in the LTM cell switch command and deliver it to UE to trigger cell switching. While the indicated TA may be outdated due to UE movement since it is not a real-time estimation.

To guarantee the validity of the indicated TA, we can follow similar way in R17 SDT to check the validity of the TA. In R17 SDT, UE will store the RSRP of the downlink pathloss reference for TA validation upon reception of RRC release message, and then compare the RSRP difference when UE is performing the TA validation. If the RSRP change does not exceed the configured threshold, UE assumes the TA is valid.

In RACH-less LTM, similar principle can be followed. UE stores the RSRP of the downlink pathloss reference upon performing the preamble transmission. And when LTM cell switch command is received, UE compares the RSRP change for TA validation. If TA is not valid, UE can fallback to RACH-based LTM.

RAN2 further study whether to evaluate the TA validity, i.e. upon reception of LTM cell switch command, UE check the TA validity of target cell by comparing the RSRP change.

Conclusion

Based on the discussion in section 2 we have following observations:

Proposal 1	For PDCCH ordered early sync, NUL/SUL is indicated in PDCCH order.

Proposal 2	For PDCCH ordered early sync, BWP info is indicated in PDCCH order.

Proposal 3	RACH towards serving cell is prioritized if there is concurrent RACH for early TA acquisition.

Proposal 4	If a new RACH towards serving cell is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, UE stops the ongoing RACH towards candidate and initiates the new RACH procedure towards serving cell.

Proposal 5	If a new RACH for early TA acquisition is triggered while another RACH procedure towards serving cell is already ongoing in the MAC entity, UE keeps the ongoing procedure and ignores the new RACH trigger.

Proposal 6	If a new RACH for early TA acquisition is triggered while another RACH for early TA acquisition is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new RACH procedure.

Proposal 7	For RACH-Less LTM, handover is considered as completed if a UE contention resolution MAC CE is received.

Proposal 8	RAN2 further study whether to evaluate the TA validity, i.e. upon reception of LTM cell switch command, UE check the TA validity of target cell by comparing the RSRP change.