3GPP TSG-RAN WG2 Meeting #123	R2-2308048

Toulouse, FR, Aug 21-25, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RAN impact for NPN enhancement in Rel-18

Agenda Item:	7.25.3

Document for:	Discussion and decision

- Introduction

In RAN#99 meeting, the WID of NR_ePRN_Ph2 [1] was approved, with the following objectives:

The objective of this work item is to perform normative work for RAN functionalities required to support 5GS functions of the SA2 WID “Enhanced support of Non-Public Networks Phase 2”, with the following features being of RAN relevance: - Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] - Support for non-3GPP access for SNPN [RAN3].

In RAN2#121bis meeting, RAN2 impacts of NPN enhancement were discussed, with the following agreements [2]:

- RAN2 assumes that the following need to be addressed For TS 38.304, the impact to RAN2 is on the following to extend equivalent PLMN to also SNPN: - Equivalent SNPN list definition - NAS interactions with AS for equivalent SNPN - Suitable cell definition while operating in SNPN access mode - Suitability check - Intra-frequency Reselection Indication For TS 38.331, to extend the cell barring for IAB to also if the selected SNPN is equivalent SNPN.

In RAN2#122 meeting, RAN2 impacts of NPN enhancement were further discussed and the following agreements were reached [3]:

R2 assumes that No new AS UE capability (signalled) is needed for supporting R18 eNPN features. FFS any 38.306 impact for conditional UE cap dependent on NAS cap.

This paper discusses the FFS issue on UE capabilities for NPN enhancement in Rel-18.

- Discussion

One remaining issue is whether to introduce conditionally mandatory UE AS capabilities for Rel-18 NPN features. Below analyses the necessity of introducing such UE AS capabilities.

UE AS capability to support equivalent SNPNs

The support of equivalent SNPNs is mainly a NAS feature and in TS 24.501, a new UE NAS capability for the support of equivalent SNPNs is defined for mobility management. Since such feature impacts the UE AS functions, e.g. cell selection and reselection, some companies think the corresponding conditional mandatory UE AS capability needs to be defined. However, we think this is not needed, since:

- The “conditional mandatory features” specified in TS 38.306 are related to the dependency between AS capabilities but not that between AS capability and NAS capacity. So it is not proper to introduce a conditional mandatory UE AS capability for the support of equivalent SNPNs dependent on the support of equivalent SNPNs in NAS. One example is that we agreed in R17 to reuse the following capability for IMS emergency call for SNPN access mode, which is a conditionally mandatory feature and the corresponding capability is also an AS capability:

| IMS emergency call | It is mandatory to support IMS emergency call over PLMN for UEs which are IMS voice capable in NR. It is mandatory to support IMS emergency call over SNPN for UEs that are SNPN capable and IMS voice capable over SNPNs. |
| --- | --- |

| voiceOverNR, voiceOverNR-r17 Indicates whether the UE supports IMS voice over NR. It is mandated to the UE if the UE is capable of IMS voice over NR (including SNPN if the UE is SNPN capable). Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. | UE | No | No | Yes (Incl FR2-2 DIFF) |
| --- | --- | --- | --- | --- |

- One alternative is to define this as an optional UE AS capability, e.g. it is optional for a SNPN-capable UE to support equivalent SNPNs. However, we do not see strong motivations to introduce this AS capability, as this completely depends on NAS capability (considering that if this feature is supported in NAS, it is natural that this feature should be supported in AS).

- Similar examples include the support of CAG (where a NAS capability is defined in TS 24.501 as shown below), credential holder and onboarding functionalities. No optional or conditional mandatory UE AS capabilities have been introduced for these features. Also, there are no UE AS capabilities for the support of equivalent PLMNs. Following the same logic of the above examples, there is no need to introduce new UE AS capability for the support of equivalent SNPNs.

- 9.11.3 5GS mobility management (5GMM) information elements - 9.11.3.1 5GMM capability The purpose of the 5GMM capability information element is to provide the network with information concerning aspects of the UE related to the 5GCN or interworking with the EPS. The contents might affect the manner in which the network handles the operation of the UE. The 5GMM capability information element is coded as shown in figure 9.11.3.1.1 and table 9.11.3.1.1. < Irrelevant text skipped> Table 9.11.3.1.1: 5GMM capability information element < Irrelevant text skipped>. Closed Access Group (CAG) capability (octet 5, bit 1) Bit. 1. 0; CAG not supported. 1; CAG supported. < Irrelevant text skipped>. Equivalent SNPNs indicator (ESI) (octet 7, bit 7). This bit indicates the capability to support equivalent SNPNs. Bit. 7. 0; Equivalent SNPNs not supported. 1; Equivalent SNPNs supported. < Irrelevant text skipped>. >

UE AS capability to support non-3GPP access for SNPN

Since the support of non-3GPP access is irelevant to radio access, no corresponding UE radio capabilities are needed.

Based on the above analysis, we propose the following:

Proposal 1: No need to introduce conditional mandatory or optional UE AS capabilities for Rel-18 NPN features.

Conclusion

In this paper, we discussed the UE capabilities for NPN enhancement in Rel-18 and made the following proposal:

Proposal 1: No need to introduce conditional mandatory or optional UE AS capabilities for Rel-18 NPN features.

References

- RP-230788, New WID on Further Enhancement of Private Network Support for NG-RAN, RAN #99, China Telecom

- R2-2306553, Report of 3GPP TSG RAN WG2 meeting #121bis-e

- Draft_R2_122_meeting_report_v1.zip