3GPP TSG-RAN WG2 Meeting #123	R2-2308051

Toulouse, France, August 21st – 25th, 2023

Source: 			OPPO

Title: 	Discussion on LPHAP enhancement

Agenda item:		7.2.4

Document for: 	Discussion and Decision

# 1. Introduction

In RAN2#122 meeting, agreements regarding LPHAP enhancement have been achieved as follows:

Agreements:

Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state.

- The UE starts/restarts the area-specific TA timer when it receives the TA command.

- The UE stops the SRS transmission when the area-specific TA timer expires.

- The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area.

- Other stop/restart conditions can be discussed.

Agreements:

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters.  To be included in the LS from [AT122][415].

Agreement:

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

This paper would like to further address our views on the LPHAP enhancement.

# 2. Discussion

## 2.1 Alignment between (e)DRX and PRS configurations

In the last RAN plenary meeting, the objective "Specify solutions for alignment between eDRX and PRS configurations [RAN2]" has been revised as follows [1]:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2].

Therefore, both DRX and (e)DRX in RRC_INACTIVE are within the scope of this objective.

- “(e)DRX” in the objective “Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2]” refers to both DRX and (e)DRX configurations for paging in RRC_INACTIVE.

In the R18 SI phase, RAN2 agrees to specify solutions for alignment between (e)DRX and PRS configurations since such alignment could save the UE power for performing the PRS measurement during the DRX ‘off’ period. However, the DRX configuration consisting of PF, PO, and paging search space comes from the serving gNB, while the DL-PRS configuration such as dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-ResourceSlotOffset, dl-PRS-ResourceTimeGap, etc, comes from the LMF not only collecting the configuration from the serving gNB but also from the anchor TRPs. As can be easily found, the alignment between (e)DRX and PRS configuration should be done between serving gNB, LMF, and anchor gNBs.

- Alignment between (e)DRX and PRS configurations should be done between serving gNB, LMF, and anchor gNBs, since the DRX configuration comes from the serving gNB and the DL-PRS configuration comes from the LMF not only collecting the configuration from the serving gNB but also from the anchor TRPs.

Obviously, there exist two approaches to the implementation of the alignment: 1) to align the (e)DRX configuration with the fixed DL-PRS timing allocation, and 2) to align the DL-PRS timing allocation with the fixed (e)DRX configuration.

To align the (e)DRX configuration with the fixed DL-PRS timing pattern, the serving gNB needs to obtain knowledge of other anchor gNBs’ DL-PRS timing patterns. Note that, conventionally, the LMF sends the anchor TRPs’ DL-PRS configuration towards the UE by taking advantage of the LPP ProvideAssistanceData msg, and hence such configuration is transparent to the serving gNB. So enhancement such as letting the LMF also forward the anchor TRPs’ DL-PRS configuration via the NRPPa protocol toward the serving gNB could be made. However, (e)DRX configuration may not be easily changed according to the TRPs DL-PRS configuration, since the (e)DRX configuration applies to all UEs under the coverage of one serving gNB. For instance, suppose two UEs are sharing the same paging monitoring timing occasion, such as the same PF and PO but with different ue-Identity as the paging ID; if the DL-PRS transmitted from one particular TRP is configured towards the UE1, and the network would like to align the (e)DRX configuration with the fixed DL-PRS window, the UE2 needs to change its (e)DRX configuration also, as illustrated in the following figure:

Figure 1: (e)DRX configuration of UE2 will be influenced by the DL-PRS configuration of UE1

- If aligning the (e)DRX configuration with the fixed DL-PRS timing pattern is applied, all UEs sharing the same (e)DRX setting with the UE with the DL-PRS configuration will be impacted in terms of the (e)DRX setting.

On the other hand, if aligning the DL-PRS timing allocation with the fixed (e)DRX configuration, the anchor TRPs need to be informed of the UE’s (e)DRX information. This could be done when the LMF sends an NRPPa msg such as the NRPPa TRP INFORMATION REQUEST msg or NRPPa PRS CONFIGURATION REQUEST msg towards anchor gNBs. From our point of view, this approach is preferred, since the TRP could flexibly add additional/ change DL-PRS on suitable timing occasions if necessary.

- RAN2 to agree to align the DL-PRS timing pattern with the fixed (e)DRX configuration.

In addition, we support to reuse the UE-initiated LPP on-demand PRS configuration Request procedure to let the LMF align the DL-PRS configuration with the (e)DRX implicitly. In other words, the explicit parameters included in the UE-initiated on-demand PRS configuration Request msg could reflect the need for the periodicity of the DL-PRS configuration, such as setting a proper value for the dl-prs-ResourceSetPeriodicityReq IE.

- No spec impact is foreseen by reusing the UE-initiated LPP on-demand PRS configuration Request msg to align the DL-PRS configuration with the fixed (e)DRX.

- Reuse UE-initiated LPP on-demand PRS procedure for PRS to align with fixed (e)DRX.

## 2.2 The behaviors of validity area-specific TA timer

In RAN2#122 meeting, as shown in the agreement above, an SRS for positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state is agreed to define. UE may transmit SRS for positioning within the validity area during the validity area-specific TA timer. Some timer-related behaviors are specified, but it is still FFS on whether there is any other stop/restart condition for validity area-specific TA timer. In our understanding, the principle is that we re-use the behavior of legacy Rel-17 inactivePosSRS-TimeAlignmentTimer as the baseline.

- For the behaviors of SRS validity area-specific TA timer, the principle is to re-use the design for legacy Rel-17 inactivePosSRS-TimeAlignmentTimer as the baseline.

The (re)start/stop conditions of Rel-17 inactivePosSRS-TimeAlignmentTimer specified in TS 38.321 and TS 38.331, as well as those of validity area-specific TA timer agreed in the last meeting, could be summarized in the following table.

|  | inactivePosSRS-TimeAlignmentTimer | validity area-specific TA timer |
| --- | --- | --- |
| Start/restart conditions | - When a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG - When a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell, and when the Contention Resolution is considered successful for Random Access procedure while SRS transmission in RRC_INACTIVE is ongoing - When an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE as specified in clause 5.1.4a | The UE starts/restarts the area-specific TA timer when it receives the TA command. |
|  | When the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer: - Upon reception of the RRCRelease by the UE: if the RRCRelease includes suspendConfig and if srs-PosRRC-Inactive is configured | ? |
| Stop conditions | When the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer: |  |
|  | - Upon reception of the RRCSetup by the UE: if the RRCSetup is received in response to an RRCReestablishmentRequest, RRCResumeRequest, or RRCResumeRequest1, and if srs-PosRRC-InactiveConfig is configured | ? |
|  | - Upon reception of the RRCResume by the UE: if srs-PosRRC-InactiveConfig is configured |  |
|  | - If cell reselection occurs when srs-PosRRC-Inactive is configured | The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area. |

As highlighted in the table, there are still leftover cases that need to be considered. In our understanding, it is straightforward to reuse the behaviors of Rel-17 inactivePosSRS-TimeAlignmentTimer upon reception of RRCRelease, RRCSetup, or RRCResume. For the start/restart conditions, when UE receivers SRS for positioning configuration with validity area in RRCRelease with suspendConfig, UE should indicate MAC layer to start or restart the validity area-specific TA timer. For the stop condition, similar to Rel-17 inactivePosSRS-TimeAlignmentTimer, if RRCSetup or RRC Resume is received, UE should indicate MAC layer to stop the validity area-specific TA timer.

- UE starts/restarts the validity area-specific TA timer when it receives the configuration of SRS for positioning with validity area in RRCRelease with suspendConfig.

- UE stops the validity area-specific TA timer if RRCSetup or RRCResume is received.

In Rel-17, when inactivePosSRS-TimeAlignmentTimer is not running, i.e., the timer is expired or stopped, UE should not perform the transmission of SRS for positioning in RRC_INACTIVE, and the configuration of Positioning SRS for RRC_INACTIVE is also released. For the validity area-specific TA timer, we agreed in the last meeting that UE should stop the SRS transmission when the area-specific TA timer expires. It should also be extended to the case that the validity area-specific TA timer is stopped.

- UE stops the SRS transmission when the area-specific TA timer is not running, i.e., the timer is expired or stopped.

Besides, when the validity area-specific TA timer is not running, whether to release the SRS for positioning configuration or not is still FFS, which could be looked into case by case:

- For the cases that the timer is stopped since UE reselects to a cell out of the SRS validity area and the timer is expired, UE cannot keep transmitting SRS for positioning, therefore it is not necessary for UE to keep the invalid SRS configuration.

- For the case that the timer is stopped since RRCSetup is received by UE and if the RRCSetup is in response to RRCReestablishmentRequest, RRCResumeRequest, or RRCResumeRequest1, it means that UE falls back to RRC connection establishment from RRC connection re-establishment or resume, so any stored UE Inactive AS context and suspendConfig including the SRS for positioning configuration would be discarded for this exception situation.

- For the case that the timer is stopped since RRCResume is received by UE, UE returns to RRC_CONNECTED from RRC_INACTIVE, therefore UE should release the suspendConfig including the SRS for positioning configuration.

To sum up, as long as the validity area-specific TA timer is not running, i.e., the timer is expired or stopped, UE should perform the same actions as for Rel-17 inactivePosSRS-TimeAlignmentTimer, i.e., stop the SRS transmission and release the SRS configuration.

- UE releases the SRS configuration when the validity area-specific TA timer is expired.

- UE releases the SRS configuration when the validity area-specific TA timer is stopped due to the reception of RRCSetup/RRCResume or the reselection to a cell out of the SRS validity area.

## 2.3 Request/activation for UE-specific SRS configuration

In RAN2#121bis-e meeting, it has been agreed that SRS configuration requests can be indicated via Msg3/MsgA transmission. From our point of view, anyway, the UE needs to perform the transmission of RRCResumeRequest towards the network when sending the SRS configuration request, so the most efficient approach is to include the request in the Msg3. Considering there are still a lot of spare bits in ResumeCause, it is enough to define a new resume cause value for SRS configuration request.

- Define a new resume cause for SRS configuration request in RRCResumeRequest.

Regarding SRS activation/deactivation for UE-specific SRS, the following agreement is made in RAN2#122 meeting, however, the use case for SRS activation/deactivation is still unclear.

RAN2#122 Agreement:

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

In our understanding, the need of activation/deactivation for UE-specific SRS depends on the supported SRS types including periodic/semi-persistent/aperiodic SRS. For aperiodic SRS as well as periodic SRS, a certain time-frequency resource is assigned for the UE-specific SRS. Therefore, once UE receives the UE-specific SRS configuration, the SRS configuration is considered as activated for the UE and the UE does not have to get permission for UE-specific SRS transmission, which is similar to the current implementation. Only for semi-persistent SRS, the activation/deactivation procedure is needed.

- The need of activation/deactivation for UE-specific SRS depends on the supported SRS types including periodic/semi-persistent/aperiodic SRS.

- The activation/deactivation procedure is needed only for semi-persistent SRS.

Note that according to the Rel-18 Pos WID [1], the objective for LPHAP enhancement aims to enable the use case #6 as defined TS 22.104, which corresponds to tracking of workpiece (in- and outdoor) in assembly area and warehouse with a target accuracy of <1m, a positioning interval of 15-30 seconds, and a battery life of 6-12 months. Aperiodic SRS is not suitable for this kind of periodic positioning service. Besides, we also do not see the additional benefit of semi-persistent SRS; on the contrary, its activation/deactivation procedure would definitely lead to additional unnecessary UE power consumption, which is against the LPHAP enhancement purposes. Therefore, in our understanding, periodic SRS is the better SRS type for LPHAP use case #6, and we can live with the support of periodic SRS in Rel-18.

- For LPHAP use case #6, aperiodic and semi-persistent SRS is not suitable, and only supporting periodic SRS is sufficient in Rel-18.

- Only periodic SRS is supported for SRS configuration with validity area.

Besides, companies propose another motivation for SRS activation/deactivation for UE-specific SRS, i.e., if the activation/deactivation is not needed, network may have to continuously monitor SRS in all cells within one validity area and have additional power consumption in network side, since network does not know which cell the RRC_INACTIVE UE is camping on. As is well-known to all, Rel-18 LPHAP enhancement focuses on the reduction of power consumption in the UE side. Therefore, it seems that this motivation is not in the scope of Rel-18 LPHAP enhancement. On the other hand, we think that instead of the UE-initiated activation/deactivation method, the network-based solution could be pursued and is more power-efficient from the perspective of UE.

- Introducing SRS activation/deactivation for UE-specific SRS for the purpose of power saving in network side is out of the scope of Rel-18 LPHAP enhancement.

- RAN2 not to pursue any solution on SRS activation/deactivation for UE-specific SRS.

## 2.4 Pre-configuration of common SRSs via system information

In the post-meeting email discussion [2], pre-configuration of common SRSs via system information is discussed. In our understanding, it is not feasible and should not be pursued due to lack of motivation from UE power saving perspective. For the common SRSs broadcasted via system information, network needs to keep updating the available common SRSs to avoid SRS collision among UEs. If one of the common SRSs broadcasted via system information is requested/activated for a target UE, network has to remove it from system information. In this case, it leads to more power consumption for UE due to the frequent system information modification, which is against the purpose of LPHAP enhancement. Instead, network can pre-configure the UE-specific SRS relying on the dedicated signalling, such as RRCRelease with suspendConfig, before UE goes to RRC_INACTIVE.

- For the common SRSs broadcasted via system information, network needs to keep updating the available common SRSs to avoid SRS collision among UEs, which leads to more power consumption for UE due to the frequent system information modification and is against the purpose of LPHAP enhancement.

- RAN2 not to pursue any solution on pre-configuration of common SRSs via system information.

# 3. Conclusion

Based on the discussion above, we made the following observations:

- “(e)DRX” in the objective “Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2]” refers to both DRX and (e)DRX configurations for paging in RRC_INACTIVE.

- Alignment between (e)DRX and PRS configurations should be done between serving gNB, LMF, and anchor gNBs, since the DRX configuration comes from the serving gNB and the DL-PRS configuration comes from the LMF not only collecting the configuration from the serving gNB but also from the anchor TRPs.

- If aligning the (e)DRX configuration with the fixed DL-PRS timing pattern is applied, all UEs sharing the same (e)DRX setting with the UE with the DL-PRS configuration will be impacted in terms of the (e)DRX setting.

- No spec impact is foreseen by reusing the UE-initiated LPP on-demand PRS configuration Request msg to align the DL-PRS configuration with the fixed (e)DRX.

- For the behaviors of SRS validity area-specific TA timer, the principle is to re-use the design for legacy Rel-17 inactivePosSRS-TimeAlignmentTimer as the baseline.

- The need of activation/deactivation for UE-specific SRS depends on the supported SRS types including periodic/semi-persistent/aperiodic SRS.

- The activation/deactivation procedure is needed only for semi-persistent SRS.

- For LPHAP use case #6, aperiodic and semi-persistent SRS is not suitable, and only supporting periodic SRS is sufficient in Rel-18.

- Introducing SRS activation/deactivation for UE-specific SRS for the purpose of power saving in network side is out of the scope of Rel-18 LPHAP enhancement.

- For the common SRSs broadcasted via system information, network needs to keep updating the available common SRSs to avoid SRS collision among UEs, which leads to more power consumption for UE due to the frequent system information modification and is against the purpose of LPHAP enhancement.

Based on the discussion above, we give the following proposals:

- RAN2 to agree to align the DL-PRS timing pattern with the fixed (e)DRX configuration.

- Reuse UE-initiated LPP on-demand PRS procedure for PRS to align with fixed (e)DRX.

- UE starts/restarts the validity area-specific TA timer when it receives the configuration of SRS for positioning with validity area in RRCRelease with suspendConfig.

- UE stops the validity area-specific TA timer if RRCSetup or RRCResume is received.

- UE stops the SRS transmission when the area-specific TA timer is not running, i.e., the timer is expired or stopped.

- UE releases the SRS configuration when the validity area-specific TA timer is expired.

- UE releases the SRS configuration when the validity area-specific TA timer is stopped due to the reception of RRCSetup/RRCResume or the reselection to a cell out of the SRS validity area.

- Define a new resume cause for SRS configuration request in RRCResumeRequest.

- Only periodic SRS is supported for SRS configuration with validity area.

- RAN2 not to pursue any solution on SRS activation/deactivation for UE-specific SRS.

- RAN2 not to pursue any solution on pre-configuration of common SRSs via system information.

# 4. Reference

[1] RP-231460, Revised WID on Expanded and Improved NR Positioning, Intel

[2] [Post122][401][POS] SRS configuration and activation in LPHAP_v19_summary, CATT