3GPP TSG-RAN WG2 Meeting #123                                   R2-2308059

Toulouse, France, Aug 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on pre-configured gap for RedCap UE

Agenda item:		6.1.3.1

Document for: 	Discussion and decision

- Introduction

In RAN2#123, RAN2 receives an LS[1] from RAN3 asking about the applicability of pre-configured gaps for RedCap UEs, the content of LS is shown below:

1. Overall Description: During Release 17 RedCap maintenance discussion, RAN3 has discussed the topic of RedCap UE configured with BWP specific Serving Cell Measurement Object (ServingCellMO) support over F1. To support the scenario of DCI based BWP switching scenario for a RedCap UE, based on the introduction of preConfGapStatus-r17 IE in BWP-DownlinkDedicated IE in TS 38.331, RAN3 is discussing how the gNB-CU can request the gNB-DU to provide configuration of all Pre-Configured Measurement Gaps (Pre-MG) that can be used after the active BWP is switched via DCI for the RedCap UE that supports this capability. However, based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP. 2. Actions: To RAN2: ACTION: RAN3 kindly asks RAN2 to feedback on the above questions.

In this contribution, we discuss the issue and provide our views.

- Background

The issue mentioned in RAN3 LS relates to discussion on CU-DU operation for providing pre-configured gap configuration for RedCap UE. To facilitate the discussion in RAN2, In this section, we provide the background of the discussion in RAN3.

Under Rel-17 MGE WI, besides legacy measurement gap, we have introduced new gap types: pre-configured gap, concurrent gaps and ncsg. Based on UE capability and gap requirements, the network can configure different gap configurations to the UE.

- Gap configuration for non-RedCap UEs

From BS point of view, in case of gNB CU/DU split, for non-RedCap UEs, the general gap configuration procedure is given below according to TS 38.473 (F1 interface):

- Step 1: During UE Context Setup or UE Context Modification procedure, gNB-CU can provide a list of measured frequencies to gNB-DU, ask gNB-DU to provide gap configuration;

- Step 2: Based on the frequency location of CD-SSB and the received measured frequencies from gNB-CU, gNB-DU generates the gap configuration and delivers the gap configuration to gNB-CU;

- Step 3: gNB-CU includes the gap configuration in RRC message and sends to UE.

Note: Considering the gNB-DU can obtain UE’s capability and it knows UE’s physical resources configuration (e.g. SR, SRS), the gNB-DU is able to determine a suitable gap configuration for the UE. More specifically, the gNB-DU is also responsible to determine the gap types (per-UE, per-FR).

Observation 1	Based on current RAN3 specification, in CU/DU split, gNB-CU will transmit the measured frequencies to gNB-DU, gNB-DU is responsible to determine and generate the gap configuration and gap type (per-UE，per-FR).

- Gap configuration for RedCap UEs—under RAN3 discussion

For RedCap UE, by introducing NCD-SSB in dedicated BWP, the UE may perform serving cell measurements on different SSBs when active BWP is changed, according to RAN2 spec, the network can configure separate servingCellMO in dedicated BWP, used to indicate the SSB for serving cell measurement when this BWP is activated. To support this per-BWP servingCellMO configuration, in previous RAN3 meetings, RAN3 approved a CR[2] to facilitate the operation between gNB-CU and gNB-DU. The signalling flow is shown in below figure.

Figure x ServingCellMO configuration coordination over F1 agreed in RAN1

In short, because measurement objects are generated by gNB-CU, but per-BWP servingCellMO and BWP configuration are generated by gNB-DU, so gNB-CU can firstly deliver a set of ServingCellMOs to gNB-DU, it is up to gNB-DU to select the ServingCellMOs based on BWP configurations and provides feedback to gNB-CU.

Observation 2	For RedCap UEs, considering NCD-SSB can be configured for serving cell measurements, RAN3 introduces new parameters in F1 interface to support ServingCellMO coordination between gNB-CU and gNB-DU.

After such coordination between gNB-CU and gNB-DU, gNB-CU is aware of the SSBs that can be used for serving cell measurements, however, gNB-CU does not know which SSB is used at the moment because UE’s active BWP can be changed via DCI and it is transparent to gNB-CU. Therefore, companies in RAN3 raised issue [3] that the measured frequencies may vary upon BWP switching, correspondingly, the gap requirement may also change upon BWP switching. The latest RAN3 CR is provided in [4].

Below is the analysis cited from RAN3 paper[3]:

R3-2 - Based on RAN2 and RAN4 definition of preConfGapStatus-r17 IE, for network-controlled mode, the UE expects the network to configure all possible preconfigured measurement gaps that will be activated by the UE in case of active BWP switching, It is not clear however how both gNB-CU and gNB-DU can know the different pre-configured MGs for all the BWPs which will be used by UE, so that gNB-CU can prepare the RRC message to UE. In fact, gNB-CU needs to configure multiple measurement gaps in advance, covering all possible gaps considering the list of BWPs that can be configured active via DCI at some point in time, so that the gNB-CU can provide a presetting via RRC signaling to the UE. The UE will use this RRC pre-setting according to TS 38.331 to know which gaps it should apply when active BWP has been switched via DCI without any further RRC reconfiguration from the gNB-CU. - To support the case of active BWP switching, the gNB-CU needs to provide in one RRC message all MGs required for multiple BWPs to UE, which UE will use to know which gap it should apply when active BWP is switched via DCI.

As we can see, company thinks network can configure multiple pre-configured gaps to the UE, when active BWP is switched via DCI, the UE can apply one of the gaps according to the gap configurations. To support such configuration, the proposal in [4] is summarized as below:

- Step1: gNB-DU informs gNB-CU the bandwidth and location of all configured BWPs, in addition, gNB-DU also informs gNB-CU which BWP is currently activated.

- Step2: Based on the BWP information received from gNB-DU, for each configured BWP, gNB-CU provides list of measured frequencies that requires pre-configured gap, used to request gNB-DU to provide pre-configured gaps.

- Step3: Based on the request from gNB-CU, gNB-DU generates a list of pre-configured gaps and send to gNB-CU. After the gap configuration is sent to UE, gNB-DU will schedule the UE according to the pre-configured gap activated in each BWP.

Observation 3	In RAN3 discussion, some companies think that network can configure multiple pre-configured gaps to the UE, and UE will use one of them when active BWP is switched via DCI.

However, in RAN3, some companies commented that multiple pre-configured gaps are not supported in Rel-17 according to RAN2 specification, therefore LS is sent to RAN2 for clarification.

- Discussion

As explained in previous section, RAN3 asks RAN2 whether network can different pre-configured MGs for different BWPs?

- However, based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP.

According to Rel-17 TS38.331, the field description of gapToAddModList is shown below:

gapToAddModList A list of of measurement gap configuration to be added or modified. If more than one measurement gap is configured (i.e. concurrent measurement gap as specified in TS 38.133[14], clause 9.1.8), the maximum number of configured measurement gap is limited by the gap combinations defined in Table 9.1.8-1 in TS 38.133 [14]. The network configures at most one NCSG or pre-configured measurement gap for a given gap type. In this version of the specification, the network configures this field only in NR standalone. This field is used only for a UE that supports pre-configured measurement gap, concurrent measurement gap, or NCSG. In this version of the specification, the network does not configure concurrent measurement gap together with MUSIM gap or preconfigured measurement gap for positioning.

Based on the yellow sentence, it is clear that network can only configure one pre-configured gap for a given gap type, here, “gap type” means pre-UE gap or FR1 gap or FR2 gap. Because RedCap UE does not support CA or DC, it only has one PCell, therefore, no matter of gap type, the network can only configure one gap (FR1 gap and FR2 gap will not be configured simultaneously). Although pre-configured gap can be activated or deactivated in different active BWPs, gNB-DU is able to determine the activation/deactivation status based on the UE capability and BWP configurations.

Someone may argue about the combination of concurrent gap and pre-configured gap, however, according to Rel-18 MGE WID[5], such combination is within the scope of Rel-18 WI.

Observation 4	In Rel-17, the network can only configure at most one pre-configured gap to RedCap UE via gapToAddModList-r17 IE.

So we suggest:

Proposal 1	Reply to RAN3 that in Rel-17, network can only configure at most one pre-configured gap to RedCap via gapToAddModList-r17 IE. Different pre-configured gaps for different BWPs is within Rel-18 scope.

In addition, regarding the discussion in RAN3, we think it is not suitable to let gNB-CU to determine whether to configure pre-configured gap, and it is not necessary to expose the BWP configuration and active BWP to gNB-CU. Similar to normal gap configuration, even for RedCap UEs, the gNB-DU knows which SSB is used for serving cell measurements when a BWP is activated, and gNB-DU knows UE’s capability and all BWP configuration, thus gNB-DU is more suitable to determine the gap requirements, e.g. whether to configure concurrent gap or NCSG or pre-configured gap? If pre-configured gap is needed, whether it is activated/deactivated in one BWP?

Please note that even for non-RedCap UEs, gap may be needed if CD-SSB(for intra-f measurement) is outside UE’s active BWP, if pre-configured gap is configured, the preConfGapStatus-r17 IE can be set to “activated” for the BWP that does not contain CD-SSB, and it can be set to “deactivated” for the BWP that contains CD-SSB.  but in RAN3, there is no enhancements specified for pre-configured gap configuration coordination between CU-DU. Similarly, in Rel-16, if the measured inter-freq SSB is contained in UE’s active BWP, it is possible that measurement gap can be avoided. However, there is no enhancements defined in F1 interface to inform gNB-CU about the location&bandwidth of configured BWPs, so in our view, the assumption in RAN3 is to let gNB-DU to determine whether gap is needed based on measured frequencies, UE capability and BWP configurations.

However, we understand the design of F1 interface should be further discussed in RAN3. From RAN2 perspective, it is enough to just answer their question. We have provided a draft reply LS in Annex.

Proposal 2	Approve the reply LS in Annex.

- Conclusion

In this contribution, proposals and observations are:

Observation 1	Based on current RAN3 specification, in CU/DU split, gNB-CU will transmit the measured frequencies to gNB-DU, gNB-DU is responsible to determine and generate the gap configuration and gap type (per-UE，per-FR).

Observation 2	For RedCap UEs, considering NCD-SSB can be configured for serving cell measurements, RAN3 introduces new parameters in F1 interface to support ServingCellMO coordination between gNB-CU and gNB-DU.

Observation 3	In RAN3 discussion, some companies think that network can configure multiple pre-configured gaps to the UE, and UE will use one of them when active BWP is switched via DCI.

Observation 4	In Rel-17, the network can only configure at most one pre-configured gap to RedCap UE via gapToAddModList-r17 IE.

So, we suggest:

Proposal 1	Reply to RAN3 that in Rel-17, network can only configure at most one pre-configured gap to RedCap via gapToAddModList-r17 IE.

Proposal 2	Approve the reply LS in Annex.

- Reference

[1] R2-2307033(R3-233478) 	LS on applicability of pre-configured measurement gaps for RedCap UE

[2] R3-231063	Correction to support NCD-SSB RedCap requirements in F1AP	CR  37.340  1093  17.3.0

[3] R3-231637 	Discussion on support of preconfigured GAPs for different configured BWP for RedCap UE 	Ericsson, Qualcomm Incorporated, CATT

[4] R3-233058	Support of preconfigured Measurement GAPs for RedCap UE	Ericsson, Qualcomm Incorporated, CATT

- Annex---draft reply LS

3GPP TSG RAN WG2 #123                               R2-230xxxx

Toulouse, FR,  21st – 25th Aug, 2023

Title:	(draft) Reply LS on applicability of pre-configured measurement gaps for RedCap UE

Response to:	R2-2307033 (R3-233478)

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	ZTE Corporation [to be RAN2]

To:	RAN3

Cc:	RAN4

Contact Person:

Name:	Jing Liu

E-mail Address:	liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 thanks RAN3 for the LS on applicability of pre-configured measurement gaps for RedCap UE.

Regarding the question asked, RAN2 would like to confirm that in Rel-17, for RedCap UE, gapToAddModList IE can only be used to configure at most one pre-configured gap, the pre-configured gap can be activated or deactivated in different active BWPs.

2. Actions:

To RAN5

ACTION: 	RAN2 respectfully asks RAN3 to take above into consideration in their future work.

3. Date of Next TSG-RAN2 Meetings:

3GPP  RAN2#123bis 	                 09th – 13th Oct 2023			Xiamen, CN

3GPP  RAN2#124 	                 13th – 17th Nov 2023		Chicago, US