3GPP TSG-RAN WG2 Meeting #123                                R2-2308060

Toulouse, France, Aug 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Clarification on multipleCORESET for RedCap UE

Agenda item:		6.1.3.2

Document for: 	Discussion and decision

- Introduction

In this contribution, we clarify the usage of UE capability “multipleCORESET” for RedCap UEs when RedCap-specific initial BWP is configured.

- Discussion

In Rel-15, UE capability “multipleCORESET” is defined to indicate the support of more than one CORESETs.

| multipleCORESET Indicates whether the UE supports configuration of up to two PDCCH CORESETs per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. If this is not supported, the UE supports one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signalling for FR2 and optional for FR1. | UE | CY | No | Yes |
| --- | --- | --- | --- | --- |

Based on the definition of the UE capability, if UE signals the filed, it means the UE can be configured with up to 2 CORESETs in addition to CORESET#0, otherwise, the UE can only be configured with one CORESET per BWP in addition to CORESET#0 (i.e. 2 CORESETs in total).

For RedCap UEs, a RedCap-specific initial BWP can be configured in SIB1 in addition to legacy initial BWP. As agreed in RAN1/2, the RedCap-specific initial BWP may or may not contain CD-SSB and CORESET#0. When RedCap-specific initial BWP is configured, the UE will use this BWP instead of legacy initial BWP, e.g. to perform RACH.

If the RedCap-specific initial BWP is configured to contain CD-SSB and CORESET#0, for all RedCap UEs that do not support multipleCORESET#0, after RRC establishment, the network is able to configure another dedicated CORESET to the UE. However, if the RedCap-specific initial BWP does not contain CORESET#0, then another common CORESET (with non-zero CORESET ID) must be provided within RedCap-specific initial DL BWP, in order to indicate the PDCCH for at least RAR reception. Literally, considering the newly configured common CORESET is treated as other CORESET in addition to CORESET#0, then for RedCap UEs that do not support multipleCORESET#0, it implies the UE can not be configured with dedicated CORSET after entering RRC_CONNECTED mode. In real deployment, this is an undesirable configuration restriction.

An example is shown in below figure.

Figure 1 Example of CORESET configuration

Based on current definition of multipleCORESET, the comparison between non-RedCap UE and RedCap UE is provided in below table:

Table 1 Comparison between non-RedCap UE and RedCap UE in CORESET configuration

| UE type | support of multipleCORESET | CORESET configuration in initial BWP (RedCap-specific initial BWP is configured and does not contain CORESET#0) | Maximum number of CORESTs can be configured |
| --- | --- | --- | --- |
| non-RedCap UE | not support | CORESET#0 + 1 dedicated CORESET | 2 |
|  | support | CORESET#0 + 2 dedicated CORESETs | 3 |
| RedCap UE | not support | 1 common CORESET | 1 |
|  | support | 1 common CORESET +1 dedicated CORESET | 2 |

Observation 1	Based on current specification, if RedCap-specific initial DL BWP is configured and does not contain CORESET#0, for RedCap UEs, the maximum number of CORESETs that can be configured in initial BWP is less than non-RedCap UEs.

For RedCap UEs, when RedCap-specific initial BWP is configured, it will be used instead of legacy initial BWP, the common CORESET configured in this RedCap-specific initial DL BWP is equivalent to the CORESET#0 configured in legacy initial DL BWP. From network perspective, in such configuration, it is possible, the UE will never be configured with a BWP that contains CD-SSB and CORESET#0.

If we keep the current definition of multipleCORESET capability, then for RedCap UEs who do not support multipleCORESET, they are forced to use the same common CORESET for both CSS and USS. Someone may argue that network can configure one dedicated BWP with one dedicated CORESET to the UE, but this implies BWP switching after RRC establishment which causes additional delay.

On the other hand, for UEs supporting multipleCORESET, the network can configure up to 3 CORESETs in one BWP, but this cannot be supported by RedCap UEs.

To address this problem, we think there are three solutions:

Solution 1: To clarify for RedCap UEs, the multipleCORESET capability can also be used to indicate whether the UE supports up to two PDCCH CORESETs in addition to the common CORESET in the RedCap-specific initial DL BWP when the BWP does not contain CORESET#0.

Solution 2: To allow the network to set the common CORESET of RedCap-specific initial BWP with CORESET ID 0, so the restriction is not applicable any more.

Solution 3: Ask RedCap UEs to mandatorily support multipleCORESET capability.

Among the three solutions, Solution 2 breaks the rule of unified CORESET ID across all configured BWPs, but if the common CORESET can be configured with CORESET ID=0, that common CORESET can be associated with the search space of other BWPs based on current specification. For Solution 3, although it can solve the problem of one common CORESET, the RedCap UE cannot be configured with 3 CORESETs in RedCap-specific initial DL BWP. Therefore, we think Solution 1 is more suitable.

Proposal 1	To clarify for RedCap UEs, the multipleCORESET capability can also be used to indicate whether the UE supports up to two PDCCH CORESETs in addition to the common CORESET in the RedCap-specific initial DL BWP when the BWP does not contain CORESET#0.

We understand there may be NBC concern on Solution 1, but considering we are at the early stage of RedCap deployment, we think the NBC problem is not that serious and the change is acceptable.

A draft TS 38.306 CR of Solution 1 is provided in [1].

Proposal 2	Agree the TS 38.306 CR in [1].

- Conclusion

In this contribution, proposals and observations are:

Observation 1	Based on current specification, if RedCap-specific initial DL BWP is configured and does not contain CORESET#0, for RedCap UEs, the maximum number of CORESETs that can be configured in initial BWP is less than non-RedCap UEs.

Proposal 1	To clarify for RedCap UEs, the multipleCORESET capability can also be used to indicate whether the UE supports up to two PDCCH CORESETs in addition to the common CORESET in the RedCap-specific initial DL BWP when the BWP does not contain CORESET#0.

Proposal 2	Agree the TS 38.306 CR in [1].

- Reference

[1] R2-2308061  Correction on multipleCORESET for RedCap UE  ZTE Corporation, Sanechips  CR  38.306  v17.5.0   0943  F.