3GPP TSG-RAN WG2 Meeting #123                                  R2-2308067

Toulouse, France, Aug 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Remaining UP issues for Rel-18 CE

Agenda item:		7.21.3

Document for: 	Discussion and decision

- Introduction

For PRACH enhancement in Rel-18 further NR coverage enhancements, companies discussed the RACH fallback cases in previous RAN2 meetings and made the following agreements:

RAN2#121bis agreement: - RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS RAN2#122 agreement: - RAN2 to further discusss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure.

In the post email discussion [1], whether and how to support fallback cases are further discussed. In this contribution, we continue the discussion and share our views.

- Discussion

  - Fallback from lower number to higher number

According to the RAN2 post email discussion, companies discussed whether to support the fallback from Msg1 repetition with lower number to higher number and several models are discussed. Based on company inputs, the below two models are prioritized:

- Option 1: No fallback. Each Msg1 repetition number is treated as a separate feature.

- Option 2.2: Fallback is supported. All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type.

| - Option 1 | - Option 2.2 |
| --- | --- |

We prefer not to support fallback from lower number to higher number because RAN2 already concluded the following fallback cases are not supported:

- Rel-17 fallback from legacy RACH to 4-step RA with Msg3 repetition;

- Rel-18 fallback from legacy RACH to 4-step RA with Msg1 repetition;

In our understanding, the motivation of supporting these fallback cases are the same, i.e. to improve the RACH success rate when the UE is in bad coverage. However, we think in most cases, the DL RSRP won’t change so much during the short period and it is acceptable to not support fallback.

If we intend to support fallback from lower number to higher number, in terms of functional rationality, it is questionable whether fallback from CBRA without Msg1 repetition to CBRA with Msg1 repetition is not considered. Please note that fallback from CBRA without repetition to CBRA with repetition cannot be supported by Option 2.2. The other drawback of Option 2.2 is that, for a given feature combination, different Msg1 repetition numbers must be configured with separate preambles on the shared RO, different repetition numbers with separate ROs is not supported.

In our view, more discussions are needed to support the fallback scenario, for example, we need to discuss in which condition the UE can trigger fallback, we also need to go through the RACH related parameters in order to see which parameters need to be re-initialized upon fallback, most likely, we have to consult RAN1 on the details. Considering there are two meetings left, and we haven’t seen significant benefit for supporting such fallback, we suggest to not consider it in Rel-18.

Proposal 1	Fallback from Msg1 repetition with lower number to higher number is not supported.

  - Fallback from CFRA with Msg1 repetition to CBRA with Msg1 repetition

Based on company feedbacks in post email discussion[1], most companies prefer to support fallback from CFRA with Msg1 repetition to CBRA with Msg1 repetition. In this section, we discuss the potential solution for support such fallback.

Based on MAC spec, RACH partition and RACH resource selection includes three steps:

Step 1: Determine the applicable feature(s) for the current RACH procedure;

Step 2: Select a RACH partition based on the identified applicable features. (described in clause 5.1.1b)

Step 3: Select RACH resource. (described in clause 5.1.2)

Figure 1 RACH partition and resource selection procedure

The UE performs RACH partition selection during RACH initialization. So, if we want to support fallback from CFRA with Msg1 repetition to CBRA with Msg1 repetition, then the procedures included in red block should be revised.

Similar to the handling of RedCap UE, and to accommodate both RedCap and non-RedCap UEs, we think the procedure should be:

When CFRA with Msg1 repetition is configured:

  - If the UE is RedCap UE:

    - If there is one RACH partition available and associated with only RedCap feature and Msg1 repetition feature;

      - Select this RACH partition;

    - elseif there is one RACH partition available and associated with only RedCap feature:

      - Select this RACH partition;

    - else:

      - Select the legacy set of RACH resources that not associated with any feature;

  - else:

    - If there is one RACH partition available and associated with only Msg1 repetition feature:

      - Select this RACH partition;

    - else:

      - Select the legacy set of RACH resources that not associated with any feature;

If RACH partition framework Option 1 is adopted, different repetition numbers are treated as separate features, then in above logic, the UE should select the RACH partition with the same repetition number as indicated by network for CFRA. If RACH partition framework Option 2.2 is adopted, different repetition numbers are treated as a single feature, then in our view, it doesn’t matter whether the selected RACH partition must associate with the same repetition number or more than one repetition numbers, if multiple numbers are configured, the UE can select the RACH resources based on the evaluation of DL RSRP.

Proposal 2	If fallback from CFRA with Msg1 repetition to CBRA with Msg1 repetition is supported, RAN2 agree the below logic for RACH partition selection.

When CFRA with Msg1 repetition is configured:

  - If the UE is RedCap UE:

    - If there is one RACH partition available and associated with only RedCap feature and Msg1 repetition feature;

      - Select this RACH partition;

    - else if there is one RACH partition available and associated with only RedCap feature:

      - Select this RACH partition;

    - else:

      - Select the set of RACH resources that not associated with any feature;

  - else:

    - If there is one RACH partition available and associated with only Msg1 repetition feature:

      - Select this RACH partition;

    - else:

      - Select the set of RACH resources that not associated with any feature;

Proposal 3	If different repetition numbers are treated as separate features, for CFRA fallback, the select the RACH partition associate with the same repetition number as indicated for CFRA.

Proposal 4	If different repetition numbers are treated as a single feature, for CFRA fallback, the select the RACH partition may associate with the multiple repetition numbers, the UE determines the applicable repetition number based on the evaluation of DL RSRP.

- Conclusion

In this contribution, proposals and observations are:

Proposal 1	Fallback from Msg1 repetition with lower number to higher number is not supported.

Proposal 2	If fallback from CFRA with Msg1 repetition to CBRA with Msg1 repetition is supported, RAN2 agree the below logic for RACH partition selection.

When CFRA with Msg1 repetition is configured:

  - If the UE is RedCap UE:

    - If there is one RACH partition available and associated with only RedCap feature and Msg1 repetition feature;

      - Select this RACH partition;

    - else if there is one RACH partition available and associated with only RedCap feature:

      - Select this RACH partition;

    - else:

      - Select the set of RACH resources that not associated with any feature;

  - else:

    - If there is one RACH partition available and associated with only Msg1 repetition feature:

      - Select this RACH partition;

    - else:

      - Select the set of RACH resources that not associated with any feature;

Proposal 3	If different repetition numbers are treated as separate features, for CFRA fallback, the select the RACH partition associate with the same repetition number as indicated for CFRA.

Proposal 4	If different repetition numbers are treated as a single feature, for CFRA fallback, the select the RACH partition may associate with the multiple repetition numbers, the UE determines the applicable repetition number based on the evaluation of DL RSRP.

- Reference

[1] R2-2308065	Report of [Post122][802][R18CEenh-UP] UP open issues	ZTE Corporation