3GPP TSG RAN WG2 Meeting #123                                                         R2-2308076

# Toulouse, France, August 21-25, 2023

Agenda item:	7.5.4.1

Source:	Intel Corporation

Title:	BSR Enhancements for XR

Document for:	Discussion

# Introduction

So far, RAN2 agreements on BSR related enhancements for XR traffic are the following:

RAN2 #122

- UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

- We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

RAN2 #121bis-e

- Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

  - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs.

  - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- At least linear distribution is used for generating code points in new BSR table(s).  FFS whether exponential distribution can be considered too.  FFS if piecewise linear distribution is supported.

  - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size.

RAN2 #121

- New BSR tables are fixed (=specified) or semi-static (RRC-based).

- FFS how many BSR tables are defined.

RAN2 #120

- RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase.

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

- RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

This document discusses how BSR could be enhanced for XR traffic addressing: delay reporting, BSR in the event of PDU discard and additional BSR table(s).

# Discussion

## Delay Reporting

### Aggregation level and reported value

RAN2 has agreed to introduce data volume information associated with delay information (e.g., remaining time) in a MAC CE which is calculated based on the remaining timer of the PDCP discard timer. However, details on the format, value reported etc has not been decided in RAN2.

At any given time, there may be multiple PDU sets stored in the UL buffer, and each PDU set can have its own PSDB value. The definition of remaining time for PDUs or data stored in the UL buffer could vary according to the delay budget value of their associated PDU sets, however reporting delay information of remaining time per PDU set could potentially result in significant overhead, especially for smaller PDU sets. RAN2 therefore needs to decide on an aggregation level over which delay values (or remaining time) can be reported e.g., over all PDUs/PDU-sets stored in the UL buffer, or per sub-group of PDU-sets. If the BSR is used to convey this aggregated delay information to the gNB, an appropriate aggregation level could be per logical channel group (LCG). This delay value could be reported for all, of only few LCGs, and the reporting can be configurable per LCG.

- Delay information is determined/reported by UE in the enhanced BSR at an aggregation level per logical channel group (LCG).

- Network can configure the UE when reporting of delay information is enabled and optionally for which (or all) LCGs this delay information can be provided.

In the case of aggregation, it needs to be decided how the delay value is calculated e.g., it could be the mean or worst-case value across multiple PDU sets of a given LCG, that is, the smallest or mean value of remaining delay budget among PDU-sets mapped to that LCG. Since the aim is to aid faster scheduling especially for delay stringent XR traffic, it seems prudent to report the worst-case value, since this would dictate whether the PDU set is about to exceed its delay budget. Reporting the average value could be inefficient especially if there is large difference in remaining time of different PDUs or PDU sets.

- If multiple remaining times are available for one aggregation level, UE reports the worst-case (i.e. the smallest value) of remaining delay budget at the reporting aggregation level (i.e., per LCG).

### Remaining Time Based Configuration and Trigger

For data in the UL buffer, the UE can determine the remaining time before expiration of preconfigured timer based on the maximum delay constraint (e.g., based on PDB/PSDB value) and the amount of time that the data has been queued/buffered. A new trigger condition can be introduced for when to provide the information associated with the remaining delay, based on a preconfigured delayThreshold value.

- A new threshold (e.g., delayThreshold) is configured to the UE in order to determine when to include delay information for the specific logical group.

### Reporting Format

To reflect delay in BSR, a new BSR mapping table for delay values similar to Table 6.1.3.1-1 and Table 6.1.3.1-2 in TS 38.321 could be created. A new MAC CE for enhanced BSR can be defined to carry delay information for XR traffic, where this delay information is provided per logical channel group as the smallest or mean value of remaining delay budget of data carried by any given logical channel in a particular logical channel group. An example of such MAC CE is shown in Figures 1a and 1b below. In the example MAC CE in Figure 1a, it is assumed that for any LCG both the buffer size and the delay value is reported. In the example MAC CE in Figure 1b, it is assumed that LCG-ext is set to 1 for only those LCGs for which a delay value is reported.

Figure 1a: BSR MAC CE with Remaining Time Information reported for all LCGs

Figure 1b: BSR MAC CE with Remaining Time Information reported for some LCGs

- A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

## BSR Trigger for Discard Operation in XR

To avoid/minimize resource wastage following PDU discard, RAN2 can discuss methods of efficient buffer status reporting of XR traffic in uplink. One scenario where this is needed is in the event of XR traffic discard, i.e., when transmitting entity (i.e. the UE) decides to discard a PDU set, if some PDUs within that set are lost/corrupted/delayed during UL transmission. In this case, a problem in relation to feedback and BSR from the UE side is that network might have outdated BSR information when UE discards packets without knowledge of the network. In some cases, the network may have allocated UL grants for packets that are discarded by UE before gNB has received an updated BSR. In other cases, the network might not have allocated a grant yet, but it could still benefit from knowledge of reduction in UE’s buffered data volume e.g., for future scheduling, resource usage/planning. An example of such scenario is shown in Figure 2 below.

Figure 2. Updated BSR after PDU discard

For this issue, a new BSR triggering condition can be introduced such that when UE performs the discard of packets, it sends an updated BSR to the gNB with the updated data volume in the buffer (after subtracting the discarded packets’ volume), i.e., the UE reports a reduction in UL data volume via a BSR. It is also possible that following a discard operation, there may not be any data left in the UL buffer for that LCG, which could also be reported with the introduction of discard-based BSR trigger.

- PDU discard at UE transmitter triggers a BSR in order to report a reduction of the data volume in UE’s UL buffer.

The BSR transmitted in the event of PDU discard can potentially use the existing BSR MAC CE to report the data in the UL buffer, however some prohibiting conditions may be required considering also how often such PDU discard triggered BSRs can be generated. To this end, some additional condition(s) might need to be met in connection with the discard of the data for the UE.

- In the event of PDU-set discard, the discard update BSR is only triggered if the volume of discarded data is more than configured threshold.

- A prohibit timer can be introduced such that a BSR once triggered in the event of PDU set discard, can only be re-triggered after a preconfigured time duration.

- If Proposal 6 is agreed, to define a mechanism to restrict how frequent UE reports the reduction on UL data bugger (e.g., having network to set up a threshold on the amount of discarded data, or prohibit timer that controls the time elapsed between discard triggered BSRs due to this new condition).

## BS table

The need for additional BS table for XR is agreed in RAN2 with the purpose of reducing quantization errors for typical high volume of data in UL buffer which can be expected for XR applications with high bit rates, i.e., finer granularity than legacy one. During RAN2#122, one static new BS table with 8 bits was agreed. In our understanding, this approach is not ideal to accommodate all XR kind of traffic, but we also understood that it is the only agreeable compromise for Rel-18. Therefore, prefer leaving proponent companies to propose their preferred min/max/step and focus the discussion on how UE behaves with this new and legacy BS table.

RAN2 agreed that network configures which the new table to the UE per LCG. However, it was left FFS whether more than one table can be used by a UE for a given LCG (i.e., legacy vs the new table) and if so, how UE determines which one to use (e.g., based on buffer size) and how network knows which BSR table each LCG uses. The following options are possible:

- Single BS table is configured per LCG. This seems the simplest approach, however it has the limitation that the table may not be the optimum one for the current buffer volume and also the network would have to reconfigure the UE with the new BSR table to be used when traffic characteristic changes in that specific LCG.

- More than one BS table can be used per LCG (legacy and the new one) and an trigger event determines which one to be used. A straightforward approach could be to define a pre-configured threshold on the buffer volume. In this case, if BS value is greater than threshold, the UE can use the new BSR table, otherwise it uses the legacy BS tables. If the UE does use the new BSR table, it can indicate a different LCID to differentiate which BS table (legacy or extended/new table) is being used for the BSR. If RAN2 decides that the BS values in the additional BSR tables may have some (or complete) overlap with the range of values supported in previous BS tables, UE behavior needs to be defined, i.e., whether it will always use the new table, or whether it only uses the new table in scenarios when quantization error is above a certain threshold.

- A new trigger event determines which one of BS tables can be used per LCG to be used at any given time (i.e., legacy one or the new one, when configured). This trigger event used by UE is a threshold condition based on the buffer size aiming to reduce quantization error.

# Conclusion

The proposals are captured as follows:

Proposal 1.	Delay information is determined/reported by UE in the enhanced BSR at an aggregation level per logical channel group (LCG).

Proposal 2.	Network can configure the UE when reporting of delay information is enabled and optionally for which (or all) LCGs this delay information can be provided.

Proposal 3.	If multiple remaining times are available for one aggregation level, UE reports the worst-case (i.e. the smallest value) of remaining delay budget at the reporting aggregation level (i.e., per LCG).

Proposal 4.	A new threshold (e.g., delayThreshold) is configured to the UE in order to determine when to include delay information for the specific logical group.

Proposal 5.	A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

Proposal 6.	PDU discard at UE transmitter triggers a BSR in order to report a reduction of the data volume in UE’s UL buffer.

Proposal 7.	If Proposal 6 is agreed, to define a mechanism to restrict how frequent UE reports the reduction on UL data bugger (e.g., having network to set up a threshold on the amount of discarded data, or prohibit timer that controls the time elapsed between discard triggered BSRs due to this new condition).

Proposal 8.	A new trigger event determines which one of BS tables can be used per LCG to be used at any given time (i.e., legacy one or the new one, when configured). This trigger event used by UE is a threshold condition based on the buffer size aiming to reduce quantization error.