3GPP TSG-RAN WG2 Meeting #123	R2-2308096

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.4.2.1

Source:	Panasonic

Title: 	Fast cell recovery operations for LTM failures

Document for:	Discussion and Decision

# Introduction

As per the discussion until RAN2#122 meeting, the following agreements for cell recovery are made [1].

Timer based LTM cell switch failure procedure is supported. The UE starts the LTM supervisor timer upon reception of the cell switch command and stops the timer upon successful completion of LTM cell switch. The UE considers LTM cell switch failure and initiates RRC connection re-establishment procedure if the timer expires. At RLF or LTM cell switch failure, fast recovery to a candidate cell by LTM execution is supported. Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

This document discusses fast recovery operations during LTM execution.

# Discussion

From RAN2 agreements, the support for fast recovery to a candidate cell by LTM execution is intended, but it is not clear how this process is executed by the UE or indicated by the network.

During the process of LTM execution, the gNB configures the initial list of LTM candidate cells to UE, using RRCReconfiguration message with LTM config IE. Though all the LTM candidate cell information is available at UE, it doesn’t mean that each candidate cell is prepared during this process for the UE arrival during mobility, so not all candidate cells are assumed to be available for recovery. Further, network may prepare the LTM candidate cells for cell switch or recovery based on, either the position/ trajectory estimation of the UE moving away towards the cell edge of the source cell or using the L1 measurement result reported to evaluate the radio link status of the candidate cells, or due to other handover criteria which may not involve radio link conditions.

In this document a generic recovery mechanism is discussed, where the network controls the configuration towards potential candidate cells for recovery, which allows UE to benefit from LTM recovery feature, even if the UE does not support multi-TRP capability.

## LTM cell switch failure scenarios

During LTM execution procedure, the network can decide to send cell switch command to UE based on the possible cases observed below,

Case 1:  When source cell SINR or signal quality is better than the target cell SINR or signal quality, usually with some hysteresis.

Case 2: When source cell SINR or signal quality is similar as the target cell SINR or signal quality.

Case 3: When target cell SINR or signal quality is better than the source cell SINR or signal quality, usually with some hysteresis.

For Case 1 and Case 2 which are based on early handover scenario, there are more chances where the cell switch execution to the target cell fails. But this early handover criteria could necessarily be initiated by the network because, having better channel condition is not the only metric to decide the best services provided to the UE. Instead, even when the target cell having lower radio condition, it might be in a better state to provide good services to UE than the source cell. For example, when there is a congestion on the cell, or specific service is provided only by a specific cell. Such "light" operation of cell switch is one of the merits of LTM.

Observation 1: LTM cell switch can be operated even when the target cell quality is not better than the serving cell. This increases the failure rate of cell switch.

## Fast recovery to the source cell

When UE detects cell switch failure with the target cell, the best possible recovery cell could be the source cell, since the DL and UL synchronization characteristics as well as the radio bearer configurations prior to the cell switch command could be utilized again once the UE switches back to the source cell. Here the UE switching back to the source cell is considered for the case with a single active TRP capability of the UE, while for the multi-TRP case the UE is capable to retain the connectivity with the source cell until the cell switch to the target cell is successful. In both the cases the network should indicate to UE whether the source cell is available for fast recovery purpose.

Proposal 1: When source cell is available/ valid for communication after cell switch failure, then the cell switch command should also indicate to UE about cell switch with possible recovery to the source cell.

When UE can recover with the source cell, and when the UE switches back to the source cell upon detection of cell switch failure to the target cell, then the network needs to be aware the cell switch failure and recovery with the source cell situation. For this purpose, the source cell must provide UL channel resources for the UL transmission when the network issued cell switch command with possible recovery to the source cell.

For the UL transmission to the source cell as the recovery, it could be sufficient to use a lower layer data/signal such as CSI report of the source cell, SRS transmission signals, instead of any specific RRC message. Because, after cell switch command has been issued to the UE, if the network is still receiving UL transmission through the source cell instead of the target cell, then the network could explicitly detect the cell switch failure situation at the UE.

Observation 2: UL resources for the UL transmission to the source cell, after recovery from cell switch failure, should be provided for the UE when the network has indicated cell switch command with possible recovery to the source cell.

Observation 3: Transmitting a lower layer channel/signal to the source cell is sufficient for the network to detect cell switch failure situation at the UE side. E.g. channel/signal

· PUCCH carrying CSI report

· PUCCH carrying SR

· SRS

The UL resources for the source cell to transmit either the CSI report on PUCCH, or the SRS transmission signal, could be reserved as a periodic configuration, which could possibly be configured prior to the cell switch command, is continued to be available after the UE has switched back to the source cell. Or alternatively allocate the UL resources for this purpose using RACH procedure, by providing UL resources for first UL transmission. For example, when UE transmits PRACH and C-RNTI over msg3 similar to the contention resolution in RRC_CONNECTED mode, the UL resources for lower layer channel/signal is provided.

Proposal 2: When source cell is indicated for fast recovery, and periodic CSI or SR or SRS resources for the UL transmission to the source cell is intended to be reserved after cell switch failure, then the RACH-less recovery to source cell indication should be sent to UE prior to the cell switch command.

Proposal 3: When source cell is indicated for fast recovery, and when UE is not indicated of the reserved UL resources after cell switch failure, then RACH-based recovery to source cell is performed, where UL resources are obtained after RACH procedure.

When UE attempts to recover to the source cell, a new timer is necessary to monitor the success or failure status of the recovery with the source cell. When this timer expires, then RRC connection re-establishment could be triggered.

Proposal 4: A new timer should be defined, for monitoring the status of the recovery process with the source cell, likewise, to conclude the success or failure of the recovery process. If the recovery process to the source cell fails, then RRC connection re-establishment could be triggered.

During LTM cell switch execution when network determines that the source cell is not available for recovery, because of the possible load balancing situation over the source cell. Then network should be able to disable recovery to the source cell after cell switch failure. This flexibility option to disable or not allow the recovery process to the source cell could be indicated to UE along with the cell switch command.

Proposal 5: When network decides that the fast recovery to the source cell is not possible/ not allowed, then cell switch MAC CE should also indicate to the UE about cell switch without possible recovery to the source cell.

## Fast recovery to candidate cells

In this section, the fast recovery to source cell in the previous section is extended to other candidate cells.

When the target cell SINR/ signal quality is better than the source cell SINR/ signal quality, here the cell switch execution is most likely to succeed but not always guaranteed. For example, when UE moves to the boundary of two candidate cells, then the cell switch decision to the target cell may not always succeed (or not always guaranteed), but instead the cell switch to the other candidate cell towards this boundary, may succeed.

Further to the above scenario, when multiple candidate cells are identified to be used for recovery, then the recovery process could be extended to more than one candidate cells. But among those candidate cells which are configured to the UE in RRC LTM config list.

When one or multiple candidate cells are intended for recovery process, then it is expected that the network prepares all these indicated candidate cells with the UE context shared among its DUs, for the case of inter-DU candidate cells.

Proposal 6: A list of, with at least one, candidate cells is configured to UE for the fast recovery process, when recovery to the candidate cells are intended by the network.

Proposal 7: When all the candidate cells in LTM config list are intended for fast recovery process, it is still required to configure all the candidate cells to UE as an indication by the network.

Further, when the list of multiple candidate cells is configured to UE for recovery, then UE could perform recovery process with any of the candidate cells, based on the latest L1 measurements available with the UE after cell switch failure detection. But, since the channel status using L1 measurement result is not the only metric to determine the best candidate cell for cell switch or recovery, a priority order of the candidate cells is useful for UE to choose the preferred first cell for recovery, based on the network decided criteria.

Proposal 8: When multiple LTM candidate cells are intended for recovery process, then along with the list of candidate cells a priority order is configured to UE. Using which UE knows about the first preferred candidate cells for the recovery process.

The list of, at least one, candidate cells along with the priority ordering if intended for recovery process, could be configured to UE in two flexible options,

- Semi-statically, during the LTM configuration of candidate cells or before the cell switch command, using RRC message.

- Dynamically along with the cell switch command using MAC CE.

Proposal 9: If candidate cell recovery is intended, then RRC message or MAC CE is used to configure semi-statically or dynamically the list of, at least one, recovery candidate cells along with the priority order. For example,

- RRCReconfiguration message with the list of, at least one, candidate cells along with the priority order, is configured semi-statically before the cell switch command is executed.

- MAC CE with a list of, at least one, candidate cell ID for recovery, is indicated dynamically during the cell switch command sent to UE.

When UE attempts to recover with the configured list of candidate cells, a new timer is needed to monitor the success or failure status of recovery with the candidate cells. When this timer expires, RRC connection re-establishment could be triggered.

Proposal 10: A new timer should be defined, for monitoring the status of the recovery process with the candidate cells, likewise, to conclude the success or failure of the recovery process. If the recovery process fails, then RRC connection re-establishment could be triggered.

# Conclusions

Observation 1: LTM cell switch can be operated even when the target cell quality is not better than the serving cell. This increases the failure rate of cell switch.

Proposal 1: When source cell is available/ valid for communication after cell switch failure, then the cell switch command should also indicate to UE about cell switch with possible recovery to the source cell.

Observation 2: UL resources for the first UL transmission to the source cell, after recovery from cell switch failure, should be provided for the UE when the network has indicated cell switch command with possible recovery to the source cell.

Observation 3: Transmitting a lower layer channel/signal to the source cell is sufficient for the network to detect cell switch failure situation at the UE side. E.g. channel/signal

· PUCCH carrying CSI report

· PUCCH carrying SR

· SRS

Proposal 2: When source cell is indicated for fast recovery, and periodic CSI or SR or SRS resources for the first UL transmission to the source cell is intended to be reserved after cell switch failure, then the RACH-less recovery to source cell indication should be sent to UE prior to the cell switch command.

Proposal 3: When source cell is indicated for fast recovery, and when UE is not indicated of the reserved UL resources, then RACH-based recovery to source cell is performed, where UL resources are obtained after RACH procedure.

Proposal 4: A new timer should be defined, for monitoring the status of the recovery process with the source cell, likewise, to conclude the success or failure of the recovery process. If the recovery process to the source cell fails, then RRC connection re-establishment could be triggered.

Proposal 5: When network decides that the fast recovery to the source cell is not possible/ not allowed, then cell switch MAC CE should also indicate to the UE about cell switch without possible recovery to the source cell.

Proposal 6: A list of, with at least one, candidate cells is configured to UE for the fast recovery process, when recovery to the candidate cells are intended by the network.

Proposal 7: When all the candidate cells in LTM config list are intended for fast recovery process, it is still required to configure all the candidate cells to UE as an indication by the network.

Proposal 8: When multiple LTM candidate cells are intended for recovery process, then along with the list of candidate cells a priority order is configured to UE. Using which UE knows about the first preferred candidate cells for the recovery process.

Proposal 9: If candidate cell recovery is intended, then RRC message or MAC CE is used to configure semi-statically or dynamically the list of, at least one, recovery candidate cells along with the priority order. For example,

- RRCReconfiguration message with the list of, at least one, candidate cells along with the priority order, is configured semi-statically before the cell switch command is executed.

- MAC CE with a list of, at least one, candidate cell ID for recovery, is indicated dynamically during the cell switch command sent to UE.

Proposal 10: A new timer should be defined, for monitoring the status of the recovery process with the candidate cells, likewise, to conclude the success or failure of the recovery process. If the recovery process fails, then RRC connection re-establishment could be triggered.

# References

[1]		R2-2305303, 38.300 running CR for introduction of NR further mobility enhancements.

[2]		R2-2304784, 37.340 running CR for introduction of NR further mobility enhancements.