3GPP TSG-RAN WG2 Meeting #123	R2-2308099

Toulouse, France, August 21-25, 2023

Title: 	Discussion on remaining issues for mobile IAB

Source: 	ZTE, Sanechips

Agenda item:	7.12.3

Document for:	Discussion and Approval

Introduction

In this paper, we will analyze the remaining issues for the support of mobile IAB, such as BAP, PCI collision and ANR.

Discussion

BAP impact

During the last RAN2 meeting, the BAP impact was discussed and the following agreement was reached:

- RAN2 agreed in last meeting: RAN2 assumes that there is no need to introduce logical-DU-specific default BAP configuration in mobile IAB from RAN2 perspective, unless requested by RAN3 otherwise (no LS for now). RAN2 understands that the F1AP (re)configured BAP configuration to one DU will not impact/override the usage of default BAP configuration by another DU. P2: RAN2 assumes there may be redundant BAP configuration entries for non-F1-U traffic and it is up to IAB node's implementation to decide which entry is selected. FFS if there is any specification impact.

As we can see, RAN2 assumes that there may be redundant BAP configuration entries for non-F1-U traffic and it is up to IAB node's implementation to decide which entry is selected. It is FFS if there is any specification impact.

We have agreed that F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU. So it may be implemented with two separate tables for the Uplink Traffic to Routing ID Mapping configuration associated with two donor-CUs respectively. And logical DU may determine whether the non-F1-U traffic is from the source logical DU or from target logical DU, and then look up the corresponding table for non-F1-U traffic delivering. In this sense, the potential redundant BAP configuration entries can be avoided by the implementation of DU and it is not necessary to consider the specification impact.

Proposal 1: The potential redundant BAP configuration entries can be avoided by the implementation of DU and it is not necessary to consider the specification impact.

PCI collision

The PCI of mobile IAB-DU cell may collide with other cells due to the movement of mobile IAB-MT. During RAN3#117 bis meeting, it has been agreed that PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node. For example, the donor CU can know the PCI used by the mobile IAB-DU via the F1 Setup procedure. During the full migration, the target donor CU could detect PCI collision upon receiving the F1 setup request (which include the PCI used by the mobile IAB-DU) from mobile IAB-DU. Suppose the PCI collision is detected, the target donor CU may request a new PCI from OAM or select a new PCI value from the preconfigured PCI list, and then send it to the mobile IAB-DU.

When it comes to the partial migration, the mobile IAB-DU has only F1 connection with F1-terminating donor CU and the F1-terminating donor CU may be not aware of the PCIs used in the neighbouring cells of the mobile IAB cell. It is doubtful how the F1-terminating donor CU detects the PCI collision of mobile IAB-DU. In our opinion, if there is Xn connection between the F1-terminating donor CU and target donor CU, the F1-terminating donor CU can be aware of PCIs of serving cells and neighbour cells of the target donor CU via existing XnAP signaling, e.g., Xn SETUP/NG-RAN NODE CONFIGURATION UPDATE. Thus F1-terminating donor CU can detect potential PCI collision and then reconfigure the PCI for mobile IAB-DU’s cell when necessary. However, if the Xn connection is not available, it is hard for the F1-terminating donor CU to be aware of the PCI collision. On the other hand, the PCI reconfiguration after the F1 connection SETUP or NG-RAN node CONFIGURATION UPDATE may lead to longer latency and cause interference to neighbor cells.

Based on the above analysis, it is suggested that the new PCI be reconfigured as soon as possible before the potential collision happens. To be specific, the PCI used by mobile IAB DU’s cells may be sent by source donor CU to the target donor CU during HO preparation phase of co-located mobile IAB-MT. If the target donor CU receives the PCI of mobile IAB DU’s cells and predicts PCI collision, the target donor CU may re-assign the PCI for the mobile IAB-DU’s cells and send it to source donor CU. Upon receiving the HO command from source donor CU, the mobile IAB-MT delivers the new PCI configuration of mobile IAB cells to the co-located mobile IAB-DU, which will then activate the mobile IAB cells with the new PCIs.

Proposal 2: RAN2 to consider the PCI re-assignment for mobile IAB-DU’s cell during the HO preparation phase of co-located mobile IAB-MT.

ANR

As we know, the ANR function resides in the gNB, which manages the Neighbour Cell Relation Table (NCRT). Generally speaking, the NCR may contain a list of entries for each source cell which include the NR CGI, NR PCI of target cell and attributes such as Xn availability, HO, etc. Located within ANR, the Neighbour Detection Function finds new neighbors and adds them to the NCRT. The gNB may obtain the NCR information via OAM configuration, neighbor information exchange via Xn setup or gNB Configuration Update procedure. In addition, gNB may request UE to use the newly discovered PCI as parameter, to read all the broadcast NCGI(s) /ECGI(s), TAC(s), RANAC(s), PLMN ID(s) and, for neighbour NR cells, NR frequency band(s). Then UE report these information of neighboring NR cells to gNB. gNB may decide to add this neighbour relation to NCRT. In addition, gNB may use PCI and NCGI(s) to lookup a transport layer address to the new NG-RAN node and setup a new Xn interface towards this NG-RAN node.

When it comes to the mobile IAB scenario, the mobile IAB node may notify the cells information to donor CU via F1 interface once the mobile IAB node MT has connected or handover to donor CU. This donor CU may then notify the neighbor gNB with the updated served cell info via Xn interface. For the other neighbor gNB which has no Xn interface with donor CU, the UE may detect the PCI of mobile IAB cells and report to the neighbor gNB.

As a matter of fact, it is not recommended for the surrounding UE served by stationary neighboring cells to HO to the mobile IAB cells. Therefore it is also not necessary to add the mobile IAB cells to the neighboring gNB’s NCRT. To support this, when UE report the mobile IAB indication together with other NCGI(s) /ECGI(s), TAC(s), RANAC(s), PLMN ID(s) and, NR frequency band(s) information. Based on this information, the neighboring gNB decides not to add the entry for mobile IAB cells into the NCRT.

Proposal 3: To avoid stationary neighboring gNB to add the mobile IAB cells into NCRT, it is suggested UE indicates to gNB that the cell is for mobile IAB when UE is requested to report more information for a PCI associated with mobile IAB cell.

Conclusion

In this contribution, we focus on the remaining issues for mobile IAB and present our point of view. The following proposals are given:

Proposal 1: The potential redundant BAP configuration entries can be avoided by the implementation of DU and it is not necessary to consider the specification impact.

Proposal 2: RAN2 to consider the PCI re-assignment for mobile IAB-DU’s cell during the HO preparation phase of co-located mobile IAB-MT.

Proposal 3: To avoid neighbor gNB to add the mobile IAB cells into NCRT, it is suggested UE indicates to gNB that the cell is for mobile IAB when UE is requested to report more information for a PCI associated with mobile IAB cell.

Reference

- RAN2#122 Chairman notes.