3GPP TSG-RAN WG2 Meeting #123	R2-2308151

Toulouse, France, 21- 25 August, 2023

Agenda Item:	7.16.2.2

Source: 	MediaTek Inc.

Title:  	Data Collection for Model Training at UE Side

Document for:	Discussion and decision

Introduction

In Rel-18 AI/ML over air interface, we focus on three scenarios for AI/ML model operation: the UE-sided model, the NW-sided model, and the two-sided model.

- UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

- Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

- Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

In this contribution, we explain the necessity and requirements for UE-side data collection and discuss the potential issues that could arise. UE-sided data collection pertains to the collection, delivery or sharing of data for the purpose of making a large dataset available or accessible at the OTT server on the UE side.

Discussion

Necessity of UE-side Data Collection

The need for UE-sided data collection stems from two major considerations:

- Consideration 1) Rather than network entities, the UE-sided model is trained by the UE-side.

- Consideration 2) Due to limitations such as computation, power, memory, and available data for training, the UE-sided model is trained by the UE-side Over the Top (OTT) server.

Given Consideration 1) for the UE-sided model and considering factors like device capability, hardware efficiency, proprietary information disclosure, model performance, and specification efforts, the most practical way to implement AI/ML technology in a UE device is for the AI/ML model to be trained directly on the UE-side itself (via an OTT server) rather than depending on the network.

Striking an optimal balance between the complexity of AI/ML model development and deployment, and the performance gain AI/ML model can offer, it's desired for the AI/ML model on a UE device to be trained within its own development environment, customized to its unique hardware, firmware, and software characteristics. This specificity can make it challenging for a UE device to compile and run an AI/ML model developed by a different vendor, such as a network vendor.

Moreover, if the network trains an AI/ML model on the UE side, this necessitates transferring the model from the network to the UE via the air interface — a process which demands substantial specification effort. One practical concern to highlight here when implementing AI/ML algorithm on UE device is the difference between generating an AI/ML model and performing a RRC reconfiguration. Unlike executing an RRC reconfiguration, which does not require recompiling or generating new executable codes, realizing an AI/ML model on a UE device either involves using an executable code generated on an offline server or recompiling an executable image on the UE device. Given that the AI/ML model for the air interface interacts with the modem module, and that it doesn't run on a common operating system, even a standard AI/ML model would result in different executables. It is important to note that only UE or chipset vendors have the ability to generate suitable executable images for their respective devices.

RAN1 has agreed upon the following detailed cases for model delivery/transfer to the UE for UE-side models and the UE part of two-sided models. The feasibility, benefits, and challenges of model transfer are currently being evaluated by RAN1, with final conclusions still pending.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Given Consideration 2) for the UE-sided model and considering the restrictions of UE devices, it is not practical to perform UE-sided model training on the UE device due to the lack of a suitable training environment. The restrictions include but not limited to e.g., data availability, storage capacity, computational capacity, and compilation capabilities. First, the computational power and memory required for training an AI/ML model can be quite large, potentially exceeding the capabilities of a UE device. Second, training the model on a server allows for easier updates and modifications to the model without necessitating changes to the UE device. Lastly, it can offer a more secure and controlled environment for model training.

Observation 1: UE-sided data collection is required due to the following two facts:

- Rather than network entities, the UE-sided model is trained by the UE-side.

- Due to limitations such as computation, power, memory, and available data for training, the UE-sided model is trained by the UE-side Over the Top (OTT) server.

Proposal 1: UE-sided data collection should be supported for conducting model training on the UE side.

Requirements of UE-sided Data Collection

In RAN2#121 meeting, following data collection methods were identified.

| Methods | Logged MDT | Immediate MDT | L3 Meas. | L1 Meas. | UAI | Early Meas. | LPP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Terminated entity | Between UE and TCE/OAM | Between UE and TCE/OAM | Between UE and gNB | Between UE and gNB | Between UE and gNB | Between UE and gNB | Between UE and LMF |

Regarding all the data collection methods we've identified, data is sourced from individual UE and subsequently stored or terminated at the network entity (gNB, OAM, or LMF). Inherently, these methods are well-suited to facilitate model training on the network side. This naturally enables network-sided model operation, eliminating the need for model transfer, as both model training and inference occur within the network domain. However, the challenges of facilitating UE-sided model operation and making a large dataset accessible for model training to UE side (e.g., OTT server) persist. For effective offline model training at UE-side, the data collection solution should be designed to make the dataset "reach" the UE-side OTT server.

Observation 2: The current data collection methods identified allow data sourcing from UEs and storage or termination at network entities. While these methods are well-adapted for model training on the network side, they do not sufficiently support model training on the UE side.

It's generally acknowledged that model training requires a comprehensive, high-quality dataset to ensure the resultant AI/ML model is both accurate and efficient. A substantial dataset offers a wider scope of examples, enhancing the model's ability to generalize to novel, unseen data. The data collection function is usually tasked with acquiring data from a multitude of UEs over a significant duration. The data may be logged over a particular period and subsequently collated all at once, resulting in a larger dataset. Therefore, data collection for offline training tends to be characterized by less stringent latency requirements, larger data sizes, and extended validity periods.

Observation 3: Model training demands a dataset that is rich in both quantity and quality.

It's understood that AI/ML algorithms and models are tailored to each specific implementation and do not require standardization. Accordingly, the data types used for offline training might not need to be standardized. Consider, for instance, a UE-sided model: It can be trained on the UE side using a dataset collected by the UE itself, thereby obviating the need to standardize the data's format and content. Moreover, proprietary data produced by the UE can be collected and used for model training, which could potentially enhance the performance of AI/ML algorithms. On the other hand, the situation is different for network-sided models that rely on UE measurement reports for dataset collection and training. Despite the fact that some proprietary data generated by the network could be leveraged for model training, the data's format and content, when gathered from the UE to the network side, might still necessitate standardization.

Observation 4: The data content and format, used for model training on the UE side, can be vendor-dependant and aren't necessarily required to specified.

According to RAN1 discussion, a model may be associated with specific configurations/conditions and additional conditions (e.g., scenarios, sites, and datasets) as determined between UE-side and NW-side. This suggests that different datasets used to train different AI/ML models are linked with certain combinations of configurations, conditions, and scenarios and have distinct characteristics. Therefore, different datasets meant for different configurations, conditions, and scenarios should be differentiated. This is why RAN1 agreed to use assistance information for UE data collection to categorize data in forms of IDs to differentiate data characteristics due to specific configurations, scenarios, sites, etc. For UE side data collection, categorizing or assistance information related to RAN configurations, conditions, scenarios should be linked with datasets with different characteristics and known to the UE side.

Observation 5: Different datasets used to train different AI/ML models are linked with certain combinations of configurations, conditions, and scenarios and have distinct characteristics.

Proposal 2: For UE-side data collection, following requirements should be met:

- The collected dataset should be accessible on the UE-side OTT server.

- The collected dataset should be both abundant and of high quality.

- The collected dataset may include vendor-specific and proprietary data.

- Assistance information, pertinent to RAN configuration, conditions, and scenarios, is attached to the respective dataset and is recognizable to the UE side.

Potential Issues for UE-sided Data Collection

Considering the requirements of UE-side data collection, we can endeavor to identify potential issues in facilitating such data collection.

In light of the requirements for UE-side data collection, two potential options arise regarding the attachment and provision of assistance information relevant to RAN configuration, conditions, and scenarios. Under the first option, the network sends the assistance information to the UE over the air interface, and the UE subsequently delivers this information to the OTT server. However, there may be privacy or security concerns associated with giving the UE-side OTT server access to RAN-related assistance information. An alternative approach involves the network being aware of and intervening in the data collection operations from the UE to the UE-side OTT server. In this option, the network would provide the assistance information, bind it to the dataset collected from the UE, and then forward the dataset along with the assistance information to the UE-side OTT server.

The subsequent query pertains to the entity and mechanism triggering the UE-side data collection. One straightforward option involves the OTT server initiating the UE-sided data collection, remaining invisible to the network. However, it's uncertain how the OTT server would obtain the RAN-related assistance information to categorize the collected dataset, or on what basis it would trigger the data collection operation. Another option involves the network triggering the UE-sided data collection and managing when to commence or conclude the operation. This approach allows the network to intervene in data collection and aids in the integration of assistance information with the collected data.

Another crucial consideration pertains to privacy and security issues. However, RAN2 isn't the suitable WG for identifying and evaluating these issues. Despite companies having privacy and security concerns regarding UE data collection on an OTT server, SA3 should be responsible for identifying the precise privacy and security issues and should convey its reasoning to RAN2. For instance, it should determine which information can’t be delivered between which entities and why.

Proposal 3: For UE-side data collection, RAN2 discuss the issues of RAN-related assistance information attachment to the dataset collected and data collection triggering. Other aspects are not excluded. Privacy and security issues for UE-side data collection should be studied and evaluated by SA3.

UE-side Data Collection for Offline Training

Considering the requirements and potential issues previously discussed for UE-side data collection, we classify the UE-side data collection methods into two types.

- Option 1: Data transfer from UEs to OTT server with RAN awareness

- Option 2: Data transfer from UEs to OTT server without RAN awareness

Figure 1 Data transfer from UEs to an OTT server

In option 1, RAN node is aware of UE data transfer. When UE data arrives at RAN node, RAN node processes those data, add RAN related information and transfer the UE data to the OTT server through 5GS. Based on current system architecture, UE data can be transferred through CP/UP. The UE data can be included in a container if it is transferred through CP. In this case, RAN node doesn’t need to know exactly what information is being transferred from the UE to the OTT server. But UE can be configured to provide the information related to what is being transferred from the UE to the OTT server. This option allows the network to intervene and control the data collection from the UE to the OTT server.

Furthermore, RAN node can attach the assistance information pertinent to RAN configuration, conditions, and scenarios to the respective dataset. Current data collection methods including immediate/log MDT, L1 measurement, L3 measurement, early measurement and UAI can also serve the same purpose. The additional operation at the network is that the assistance information can be attached, and the network can forward the collected data together with the assistance information to the OTT server. If those methods are used to collect UE data, RAN node knows what information is delivered as UE data.

In option 2, RAN node is not aware of UE data transfer, which is delivered through UP. Even if the data transfer through UP channel is transparent to air interface, it may not be out of 3GPP scope and requires SA involvement, especially SA2. In previous RAN2 meeting, the EVEX framework was proposed to be studied as one of the data collection methods. We assume it is one variation of option 2.

From RAN2 aspect, both of the options can be considered but option 1is more of RAN2 domain.

Proposal 4: RAN2 consider UE data transfer from UEs to the OTT server with the following ways. FFS on others.

  - Option 1: Data transfer from UEs to OTT server with RAN awareness

  - Option 2: Data transfer from UEs to OTT server without RAN awareness

Conclusion

Observations:

Observation 1: UE-sided data collection is required due to the following two facts:

- Rather than network entities, the UE-sided model is trained by the UE-side.

- Due to limitations such as computation, power, memory, and available data for training, the UE-sided model is trained by the UE-side Over the Top (OTT) server.

Observation 2: The current data collection methods identified allow data sourcing from UEs and storage or termination at network entities. While these methods are well-adapted for model training on the network side, they do not sufficiently support model training on the UE side.

Observation 3: Model training demands a dataset that is rich in both quantity and quality.

Observation 4: The data content and format, used for model training on the UE side, can be vendor-dependant and aren't necessarily required to specified.

Observation 5: Different datasets used to train different AI/ML models are linked with certain combinations of configurations, conditions, and scenarios and have distinct characteristics.

We have following proposals:

Proposal 1: UE-sided data collection should be supported for conducting model training on the UE side.

Proposal 2: For UE-side data collection, following requirements should be met:

- The collected dataset should be accessible on the UE-side OTT server.

- The collected dataset should be both abundant and of high quality.

- The collected dataset may include vendor-specific and proprietary data.

- Assistance information, pertinent to RAN configuration, conditions, and scenarios, is attached to the respective dataset and is recognizable to the UE side.

Proposal 3: For UE-side data collection, RAN2 discuss the issues of RAN-related assistance information attachment to the dataset collected and data collection triggering. Other aspects are not excluded. Privacy and security issues for UE-side data collection should be studied and evaluated by SA3.

Proposal 4: RAN2 consider UE data transfer from UEs to the OTT server with the following ways. FFS on others.

  - Option 1: Data transfer from UEs to OTT server with RAN awareness

  - Option 2: Data transfer from UEs to OTT server without RAN awareness

Reference

[1] RP-231089, Data Collection for Offline Model Training at UE-side or Neutral site, Qualcomm Incorporated

[2] 3GPP TS 26.531 Data Collection and Reporting, General Description and Architecture, V17.1.0