3GPP TSG-RAN WG2 Meeting # 123  	                           	 R2-2308173

Toulouse, France, August 21-25, 2023

Agenda Item: 	7.5.4.2

Source:		Sony

Title:	Some considerations on PSI and PSIHI

Document for:	Discussion & Decision

# Introduction

In this contribution, we look into PSI and PSIHI details and propose that these are handled in PDCP layer.

# Discussions

PDU Set Importance (PSI)

RAN2_122 agreements on PSI:

- 1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how. - Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets

Different PDU sets may have low or high importance, set according to PDU Set Importance (PSI). During congestion, when a PDU set having high priority frame is stuck in the buffer and low priority PDU set packets containing another type of frame is ahead in the queue, high priority PDU set should be allowed to jump the queue and transmitted first. Out of sequence delivery is already supported so we think PSI can be supported in PDCP layer.

We think more discussion is required for in-sequence delivery, because waiting for a packet may delay the play-back of a packet and in – sequence delivery might not be useful for XR type of applications but can also be left for implementation.

Discard timer mechanism has been used to handle congestion in traditional networks and same principle can be reused. We think one option could be to have different discard timers for low and high importance PDU sets:

Proposal 1: RAN2 to agree separate PDCP discard timer for PDU set based on its importance (PSI).

On these set of agreements:

- Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets

If proposal 1 is agreed above, then in our view there is no need for the network to indicate congestion status to the UE dynamically. PSI based discard mechanism should be one-shot RRC configuration and with separate discard timer values. UE start discarding based on discard timer.

Proposal 2: PSI based discard mechanism is configured using RRC signaling and UE start discarding based on discard timer.

PDU Set Integrated Handling Indication (PSIHI)

Definition of PSIHI from 23.700-60 “Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication)

In our understanding, all packets related to the PDU set may be discarded if PSIHI is set to a value where all PDUs of a PDU set are needed by the application layer and some PDUs were lost. This can be achieved in PDCP layer by using PDCP SR. For a scenario where PSIHI is not set to this value or when some PDUs are not needed, then it can be handled by PDCP SR as well.

Proposal 3: PSIHI is handled in PDCP layer and based on PDCP SR. no enhancements are required to PDCP SR.

# Summary

In this contribution, we have the following proposals for RAN2 to agree following proposals:

Proposal 1: RAN2 to agree separate PDCP discard timer for PDU set based on its importance (PSI).

Proposal 2: PSI based discard mechanism is configured using RRC signaling and UE start discarding based on discard timer.

Proposal 3: PSIHI is handled in PDCP layer and based on PDCP SR. no enhancements are required to PDCP SR.

# References

- RP-230786, “ XR Enhancements for NR”, RANP #99, March 2023

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

- Minutes RAN2#122 meeting