3GPP TSG-RAN WG2 Meeting #123  	                            R2-2308175

Toulouse, France, August 21-25, 2023

Agenda Item:	7.3.5

Source: 	Sony

Title:	Handover enhancement for NES

Document for:	Discussion

# Introduction

In RAN2#122 meeting, connected mode mobility for NES was discussed and RAN2 reached following agreement [1]:

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

In this contribution, we will focus on the FFS part on NES connected mode mobility.

# Discussion

2.1 NES-aware CHO

In RAN2#122 meeting, RAN2 has agreed that there is specific NES CHO execution condition which is based on at least source cell’s NES mode. And existing Event A3, A4 and A5 can be configured as a CHO execution condition in the NES scenario and also time based mechanism will be considered.

Considering the above agreements and in order to differentiate with the legacy CHO events, it is envisaged that CHO execution condition for NES cell (either for source cell or target cell or both) would be different from the legacy ones, in the form of e.g., with different thresholds and/or different offsets. On the other hand, relying only on a time based mechanism can’t address all the NES cell switch off scenarios e.g., in an offloading scenario, it is practically difficult to predict how long will be configured for the UE to perform handover. Time based approach can work only for longer durations and static NES states e.g. during night time. We think that NES state transition could be for shorter durations and more dynamic in nature so that it can work during low traffic loads. This would require that NES state of the target cell is also considered and therefore agreements 1 and 2 above should apply to target cell as well. It was argued that source cell won’t even initiate the handover for a target cell if it is in NES state. But a target cell may transit to NES state after handover signalling has been accepted but before CHO evaluation has taken place.

Proposal 1: NES mode of target cell is taken into account for CHO execution condition.

Further, we think that the tigger to inform the UE to execute (perform) handover should originate from network, as listed as follows.

- Network knows the NES mode of itself as well as its neighbour cells is going to change or has changed, and network has the knowledge that which UEs may be impacted by such NES mode change (according to its configurations).

- The NES mode, especially when the UE is configured with explicit NES CHO conditions, may have changed. As a result, UE needs to be notified of such change and may start performing corresponding new CHO evaluations.

- It is the network’s decision to change its NES mode e.g., a source cell wants to offload its UEs to its neighbour cells or a cell is switched on to accept new UEs.

So, it should be the network to notify the UE to performing CHO execution due to the NES mode change. This notification could be e.g. notify the UE of the latest NES mode of related cells or configure the UE to start performing CHOs.

We have the proposals as follows.

Proposal 2: Network should notify the UE to start performing NES CHO execution when the NES mode of source or candidate cells is going to change or has changed.

RAN2 has briefly discussed the L1 based signalling and concluded that no new L1 based signalling for CHO. We still see the benefits of lower layer signalling to activate/deactivate the CHO execution conditions and therefore propose that L2 signalling e.g. MAC CE can be adopted to activate/deactivate certain configured CHO execution conditions based on the NES state of the source cell as well as target cell. Target cell can notify the source cell of its NES state once it is going to be changed.

Proposal 3: The NES CHO execution trigger is based on L2 signaling.

# Conclusion

We propose RAN2 to discuss the proposals as follows.

Proposal 1: NES mode of target cell is taken into account for CHO execution condition.

Proposal 2: Network should notify the UE to start performing NES CHO execution when the NES mode of source or candidate cells is going to change or has changed.

Proposal 3: The NES CHO execution trigger is based on L2 signaling.

# Reference

[1] R2-2306543	Report from UP, Small data, URLLC/IIoT, RACH indication, NWES and UAV Session chair (InterDigital)