3GPP TSG-RAN WG2 Meeting#123                                	R2-2308177

Toulouse, France, August 21-25, 2023

Agenda item:	5.1.3.1

Source: 	Google Inc.

Title: 	On the Absence of the DMRS Configuration for PUSCH

Document for:	Discussion and Decision

# Introduction

The DMRS configuration for a PUSCH transmission can be provided to the UE in the PUSCH-Config IE, but the provision of either the dmrs-UplinkForPUSCH-MappingTypeA or the dmrs-UplinkForPUSCH-MappingTypeB IE is optional with the Need code “Need M”, as shown in the following ASN.1 code for the PUSCH-Config IE.

PUSCH-Config ::=                  SEQUENCE {

dataScramblingIdentityPUSCH        INTEGER (0..1023)    			   OPTIONAL,   -- Need S

txConfig                           ENUMERATED {codebook, nonCodebook}  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA   SetupRelease { DMRS-UplinkConfig }  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB   SetupRelease { DMRS-UplinkConfig }  OPTIONAL,   -- Need M

...

}

As 3GPP TS 38.331 [1] does not teach UEs how to apply the DMRS configuration for a PUSCH transmission when the DMRS configuration for that PUSCH transmission is absent, different UEs may have different implementations for such a circumstance, and hence the interoperability problem may occur. This paper aims to address the issue by first analyzing all the possible UE handlings for such a circumstance, and then proceeding with the corresponding CR to align the UE behaviors with the most agreeable UE handling.

# Discussion

The DMRS configuration for a PUSCH transmission with a specific mapping type contains several child fields such as dmrs-Type, dmrs-AdditionalPosition, and maxLength. Most of these child fields are optional fields with the Need code “Need S”.

DMRS-UplinkConfig ::=               SEQUENCE {

dmrs-Type                           ENUMERATED {type2}           OPTIONAL,         -- Need S

dmrs-AdditionalPosition             ENUMERATED {pos0, pos1, pos3}   OPTIONAL,      -- Need S

phaseTrackingRS                     SetupRelease { PTRS-UplinkConfig } OPTIONAL,   -- Need M

maxLength                           ENUMERATED {len2}                  OPTIONAL,   -- Need S

...

}

Since the DMRS must be configured properly before UE performing a PUSCH transmission, some UE implementations may apply the default values to these child fields (e.g., dmrs-AdditionalPisition  = pos2) when the parent field (i.e., dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB) is absent. But such implementations are not really supported by the following description in section 6.1.2 of TS 38.331 [1].

6.1.2 Need codes and conditions for optional downlink fields [Text skipped] For downlink messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that.

Therefore, it is unclear which values the UE shall assign to the child fields of an absent dmrs-UplinkForPUSCH-MappingTypeA or an absent dmrs-UplinkForPUSCH-MappingTypeB.

Configuring DMRS is an essential step before performing the PUSCH transmission, but the UE may not know how to configure DMRS properly due to the absence of either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB.

Since there is no clear guidance on how to assign the values to the child fields of an absent dmrs-UplinkForPUSCH-MappingTypeA/B, the UE may simply choose NOT to configure the DMRS for the PUSCH of that mapping type in such a case. For instance, in one scenario, a UE is provided with a pusch-TimeDomainAllocationList having 4 pusch-TimeDomainResourceAllocation entries in the PUSCH-Config, and the dmrs-UplinkForPUSCH-MappingTypeA field is neither included in the PUSCH-Config nor signaled to the UE before. It is further assumed the mapping type of each pusch-TimeDomainResourceAllocation entry follows the table below.

Table 1. The mapping type of each entry in the PUSCH-TimeDomainAllocationList

| Entry in PUSCH-TimeDomainAllocationList | -> mappingType |
| --- | --- |
| PUSCH-TimeDomainResourceAllocation[0] | typeA |
| PUSCH-TimeDomainResourceAllocation[1] | typeA |
| PUSCH-TimeDomainResourceAllocation[2] | typeA |
| PUSCH-TimeDomainResourceAllocation[3] | typeB |

In this example, as the UE does not configure the DMRS for the ‘repetition Type A’ PUSCH due to the absence of dmrs-UplinkForPUSCH-MappingTypeA, the UE would not be able to perform the PUSCH transmission upon receiving a DCI (format 0_0) allocating a ‘repetition Type A’ PUSCH transmission. Therefore, the UE may consider it as a failure case upon receiving a DCI (format 0_0) whose ‘time domain resource assignment’ equal to 0, 1, or 2. On the other hand, if the UE receives a DCI (format 0_0) whose ‘time domain resource assignment’ equal to 3 and dmrs-UplinkForPUSCH-MappingTypeB is available to the UE, the UE can proceed with the PUSCH transmission as the UE knows how to configure the DMRS for the ‘repetition Type B’ PUSCH.

UE may considers it as a failure case upon receiving a DCI (format 0_0) allocating a PUSCH with the mappingType equal to typeA/B, if UE never received the dmrs-UplinkForPUSCH-MappingTypeA/B IE before.

In the same example, the UE may also consider it as a failure case at an earlier timing, upon receiving the pusch-TimeDomainAllocationList containing at least one PUSCH-TimeDomainResourceAllocation with the mappingType equal to typeA, if the UE has never received the dmrs-UplinkForPUSCH-MappingTypeA IE before. However, such a UE implementation requires the network to provide dmrs-UplinkForPUSCH-MappingTypeA/B before providing the pusch-TimeDomainAllocationList, or at least to provide dmrs-UplinkForPUSCH-MappingTypeA/B and pusch-TimeDomainAllocationList in the same pusch-Config IE.

UE may considers it as a failure case upon receiving a PUSCH-TimeDomainResourceAllocation with the mappingType equal to typeA/B, if UE has never received the dmrs-UplinkForPUSCH-MappingTypeA/B IE before.

With a more sophisticated UE implementation, instead of not configuring the DMRS for the PUSCH of repetition Type A in the above example, the UE may still configure the DMRS for the PUSCH of repetition Type A, by pretending the UE has already received an empty dmrs-UplinkForPUSCH-MappingTypeA (i.e., a shell dmrs-UplinkForPUSCH-MappingTypeA IE with no child field). That is, the UE assigns the default value (i.e., applies the ‘Need S’ condition) to each of the child fields under the DMRS-UplinkConfig IE, while configuring the DMRS for the PUSCH of repetition Type A.

UE may pretend it has received an empty dmrs-UplinkForPUSCH-MappingTypeA/B IE (i.e., a shell dmrs-UplinkForPUSCH-MappingTypeA/B IE with no child field), before the dmrs-UplinkForPUSCH-MappingTypeA/B IE is explicitly signaled to the UE.

Alternative to the above UE implementation, the UE may also configure the DMRS for the PUSCH of repetition Type A, by referring to the DMRS configuration resided in dmrs-DownlinkForPDSCH-MappingTypeA, if dmrs-DownlinkForPDSCH-MappingTypeA is available to the UE. If the UE has never received the dmrs-DownlinkForPDSCH-MappingTypeA before, the UE may then pretend that an empty dmrs-DownlinkForPDSCH-MappingTypeA IE with no child field is available to the UE.

UE may refer to the DMRS configuration in dmrs-DownlinkForPDSCH-MappingTypeA/B as the DMRS configuration for the PUSCH transmissions using PUSCH MappingTypeA/B, before the dmrs-UplinkForPUSCH-MappingTypeA/B IE is explicitly signaled to the UE.

Based on the above observations, the absence of the dmrs-UplinkForPUSCH-MappingTypeA/B IE may result in different UE interpretations/implementations with regard to the handling of the PUSCH DMRS configuration, where each UE interpretation/implementation does align with the current 3GPP specifications. This would produce interoperability issues between the UE and network, which are undesirable and need to be addressed properly.

The UE behavior is unpredictable/uncontrollable if the network does not provide the dmrs-DownlinkForPUSCH-MappingTypeA/B IE to the UE, which may cause the interoperability issue between the UE and the network.

To resolve the interoperability issue, we propose two options as follows.

- Option 1: Force the network to signal the dmrs-UplinkForPUSCH-MappingTypeA (or the dmrs-UplinkForPUSCH-MappingTypeB) IE at least once, before or upon configuring a PUSCH-TimeDomainResourceAllocation with the mappingType equal to typeA (or typeB).

- Option 2: Force the UE to apply a specific DMRS configuration for the typeA/B PUSCH upon the absence of the dmrs-UplinkForPUSCH-MappingTypeA/B IE.

The necessary specification changes for enabling Option 1 are captured in the CR R2-2308190 [2], while the necessary specification changes for enabling Option 2 are captured in the CR R2-2308191 [3]. Hence, we respectfully ask RAN2 to discuss and adopt either Option 1 or Option 2, based on the CRs R2-2308190 and R2-2308191.

Discuss whether to force the NW to signal the dmrs-UplinkForPUSCH-MappingTypeA/B IE at least once, or to force the UE to apply a specific DMRS configuration for PUSCH upon the absence of the dmrs-UplinkForPUSCH-MappingTypeA/B IE, based on the CRs R2-2308190 and R2-2308191.

# Conclusions

In this paper, the issues of dealing with the absence of the DMRS configuration for PUSCH are discussed, with the following observations being made.

Observation 1	Configuring DMRS is an essential step before performing the PUSCH transmission, but the UE may not know how to configure DMRS properly due to the absence of either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB.

Observation 2	UE may considers it as a failure case upon receiving a DCI (format 0_0) allocating a PUSCH with the mappingType equal to typeA/B, if UE never received the dmrs-UplinkForPUSCH-MappingTypeA/B IE before.

Observation 3	UE may considers it as a failure case upon receiving a PUSCH-TimeDomainResourceAllocation with the mappingType equal to typeA/B, if UE has never received the dmrs-UplinkForPUSCH-MappingTypeA/B IE before.

Observation 4	UE may pretend it has received an empty dmrs-UplinkForPUSCH-MappingTypeA/B IE (i.e., a shell dmrs-UplinkForPUSCH-MappingTypeA/B IE with no child field), before the dmrs-UplinkForPUSCH-MappingTypeA/B IE is explicitly signaled to the UE.

Observation 5	UE may refer to the DMRS configuration in dmrs-DownlinkForPDSCH-MappingTypeA/B as the DMRS configuration for the PUSCH transmissions using PUSCH MappingTypeA/B, before the dmrs-UplinkForPUSCH-MappingTypeA/B IE is explicitly signaled to the UE.

Observation 6	The UE behavior is unpredictable/uncontrollable if the network does not provide the dmrs-DownlinkForPUSCH-MappingTypeA/B IE to the UE, which may cause the interoperability issue between the UE and the network.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	Discuss whether to force the NW to signal the dmrs-UplinkForPUSCH-MappingTypeA/B IE at least once, or to force the UE to apply a specific DMRS configuration for PUSCH upon the absence of the dmrs-UplinkForPUSCH-MappingTypeA/B IE, based on the CRs R2-2308190 and R2-2308191.

# Reference

3GPP TS 38.331 v15.22.0, NR, Radio Resource Control (RRC) protocol specification.

R2-2308190, Corrections to ensure the presence of the DMRS configuration, Google.

R2-2308191, Corrections to the TimeDomainResourceAllocationList field description, Google.