3GPP TSG-RAN WG2 Meeting #123	R2-2308179
Toulouse, France, August 21-25, 2023

Agenda Item:	7.3.2

Source:	OPPO

Title:	Discussion on DTX/DRX mechanism

Document for:	Discussion, Decision

Introduction

This paper will discuss cell DTX/DRX, which is one objective of NES WI.

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Discussion

In the previous RAN2 meetings, RAN2 concludes the following on the cell DTX/DRX pattern.

Agreements

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

One left issue on the cell DTX/DRX is whether we can have DTX specific inactivity timer. It means that the cell DTX/DRX active time would be extended due to some event. The detail is not clear now, but in our view, the current mechanism to extend UE-specific C-DRX active time can be taken as a reference. For example, the UE/gNB starts or restarts a specific inactivity timer when conditions fulfil, and then considers the time duration between the starting and expiry of this inactivity timer as a kind of the cell DTX/DRX active time. From our perspective, such flexible cell DTX/DRX active/non-active duration complicates the UE/gNB implementation and introduces much normative work. Also, it would reduce the benefit of network energy saving due to the shortened non-active time of the network. Besides, if the cell DTX/DRX pattern is extended by one UE’s requirements but applies to all UEs in the cell, other UEs’ power would undoubtedly increase because other UEs are required to e.g. also monitor PDCCH and periodic/semi-persistent CSI-RS. Additionally, RAN2 needs to consider how other UEs know whether/how the cell DTX/DRX pattern extends. On the other hand, if the cell DTX/DRX pattern is extended and applies to a specific UE, it saves other UEs’ power but may contradict the agreement “pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell”. Thus, we prefer not to pursue the inactivity timer for cell DTX/DRX.

Not pursue the DTX-specific inactivity timer for cell DTX/DRX.

Another issue left is whether we allow exceptional SR transmission during Cell DRX non-active period. The intention is to avoid the impact on high-priority traffic, which can be an emergency call or public safety-related service (e.g. MPS/MCS).

Agreements

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

As we already agreed in RAN2#122, it relies on the network to avoid the impact on high-priority traffic, e.g. the network may deactivate Cell DTX/DRX. Thus, there is no need to introduce any exceptional SR transmission.

RAN2 confirms no exceptional handling for the SR transmission/reception during the cell DRX non-active duration.

In the SI phase, companies have confirmed the benefit of the alignment between UE C-DRX with the cell DTX as well as C-DRX among multiple UEs. On C-DRX alignment among multiple UEs (at least for part of parameters related to the UE-specific C-DRX configuration), we understand it can be implemented by the RRC configuration, thus no extra spec change is needed. On the alignment of the cell DTX/DRX and UE C-DRX, we understand the gNB would try its best to achieve the alignment. For example, the RRC configuration would let UE-specific C-DRX on-duration fall into the cell DTX on-duration. However, as UE-specific C-DRX active time would be extended when conditions are fulfilled, it would happen that the active duration of one discontinuous mechanism collides with the non-active duration of another discontinuous mechanism. We would like to analyse the collision case by case since the UE behaviour for the cell DTX/DRX non-active is different from the one for UE-specific C-DRX non-active.

Figure 1 cell DTX/DRX and UE-specific DRX

- For case ①, both the cell DTX/DRX and UE-specific C-DRX are in active time, thus, the UE/network transmits as normal.

- For case ②, the cell DTX/DRX is in active time and UE-specific C-DRX is in non-active time, which equals no cell DTX/DRX configured. Thus, the UE/network works as legacy, i.e. the UE/network performs the behaviour for UE-specific C-DRX non-active time.

- For case ③, both the cell DTX/DRX and UE-specific DRX are in non-active time. Since the expected behaviour for the cell DTX/DRX non-active is different from the one for UE-specific C-DRX non-active, we prefer the UE to follow the way expected for the cell DTX/DRX non-active, otherwise, the NES gain cannot be achieved as we designed.

- For case ④, the cell DTX/DRX is in non-active time and UE-specific C-DRX is in active time. For good energy saving gain, the UE is recommended to perform the behaviour for the cell DTX/DRX non-active, except for the exceptional cases agreed in RAN2#122 on the retransmission timer and RACH procedure.

When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX active, the UE always follows the behaviour designed for UE-specific C-DRX.

When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX non-active, the UE always follows the behaviour designed for cell DTX/DRX non-active, except for the cases agreed in RAN2#122 on the C-DRX retransmission timer.

In RAN1 Reply LS [3], the feasibility of group common L1 signalling is confirmed at least for a cell DTX/DRX activation/deactivation. Meanwhile, RAN2 is required to consider a MAC CE-based activation/deactivation indication. In our view, the benefit is unclear on the introduction of MAC CE-based indication, considering,

- MAC-CE-based indication cannot be grouped, thus it could not reduce the signalling overhead.

- If companies would like to have the signalling indicated in a dedicated way, RRC-based signalling is already there. Although the L2 signal is more dynamic than the RRC signal, there is no huge difference in-between.

- If companies concern about the decoding of common DCI, we can require common DCI decoding as mandatory for this feature.

For cell DTX/DRX, no need to introduce MAC CE-based activation/deactivation indication.

In RAN2#121, RAN2 discussed the impact on RACH/paging/SIBs and concludes the following.

Agreements

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

The majority of companies intend to avoid the impact on the legacy UEs and Rel-18 non-NES capable UEs, thus, RAN2 concludes to keep RACH, paging and SIBs as they were. This intention is clearly reflected for the UEs in the idle/inactive mode but seems missing for the UEs in the CONNECTED mode. To avoid misleading and keep companies on the same page, we expect RAN2 to confirm this missing part, i.e. no impact on RACH, SIBs and paging for both legacy UEs and Rel-18 non-NES capable UEs in the CONNECTED mode.

RAN2 confirms no impact on RACH, paging and SIBs for both legacy UEs and Rel-18 non-NES capable UEs in the CONNECTED mode.

Further, the impact on paging is not captured for Rel-18 NES capable UEs in the CONNECTED mode. In the SI phase, some companies would like to enhance paging for NES but its RAN2 impact is excluded by the approved NES WID. To be consistent with the WID and avoid any misleading, we expect further confirmation in RAN2.

- Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3].

RAN2 confirms no impact on paging for Rel-18 NES capable UEs in the CONNECTED mode.

Conclusion

We have the following proposals:

Proposal 1	Not pursue the DTX-specific inactivity timer for cell DTX/DRX.

Proposal 2	RAN2 confirms no exceptional handling for the SR transmission/reception during the cell DRX non-active duration.

Proposal 3	When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX active, the UE always follows the behaviour designed for UE-specific C-DRX.

Proposal 4	When both the cell DTX/DRX and UE-specific C-DRX are configured, in cell DTX/DRX non-active, the UE always follows the behaviour designed for cell DTX/DRX non-active, except for the cases agreed in RAN2#122 on the C-DRX retransmission timer.

Proposal 5	For cell DTX/DRX, no need to introduce MAC CE-based activation/deactivation indication.

Proposal 6	RAN2 confirms no impact on RACH, paging and SIBs for both legacy UEs and Rel-18 non-NES capable UEs in the CONNECTED mode.

Proposal 7	RAN2 confirms no impact on paging for Rel-18 NES capable UEs in the CONNECTED mode.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR

[3] R1-2306246 Reply LS on Cell DTX/DRX activation/deactivation (contact: Huawei, Intel)