3GPP TSG-RAN WG2 Meeting #123	R2-2308180
Toulouse, France, August 21-25, 2023

Agenda Item:	7.3.3

Source:	OPPO

Title:	Discussion on SSB-less Scell operation

Document for:	Discussion, Decision

Introduction

This paper will discuss SSB-less Scell operation.

Discussion

In RAN2#122, RAN2 agrees with further detailed work on SSB-less Scell if the scenario feasibility is confirmed by RAN4.

Agreements:

- If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

- If RAN4 concludes it is feasible, RAN2 can further work on at leaest the following specification impacts:

-	RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

-	UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

In RAN4#107, RAN4 discussed the potential scenarios on SSB-less SCell for inter-band CA for FR1 and co-located cells and achieved the following agreements.

- Agreements - Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. - Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

It appears that RAN4 will continue to focus on Scenario 1 and Scenario 2a unless RAN1 provides negative feedback. Although RAN4 had intended to send an LS to inform RAN1/2 about the above agreements, RAN4 finally fails to do so and now seeks clarification on whether the scenarios fall within the scope of the WI.

R4-2310087 LS on RAN1/2 impacts for SSB-less Scell operation Type: LS Out For: Approval To: RAN1, RAN2 Source: Huawei Abstract: Discussion: Session chair: No consensus to send the LS. Recommend clarifying the scenarios in the scope of the WI in RANP. Decision: Noted.

Due to the current status of RAN4, we prefer RAN2 not start a further discussion before RAN4 achieves consensus on the scenarios.

For SSB-less Scell, RAN2 does not initiate a further discussion until RAN4 achieves consensus on the scenarios.

Conclusion

We have the following proposals:

Proposal 1	For SSB-less Scell, RAN2 does not initiate a further discussion until RAN4 achieves consensus on the scenarios.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR