3GPP TSG-RAN WG2 #123	R2-2308184
Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.3

Source:	OPPO

Title:	Discussion on XR-specific power saving

Document for:	Discussion, Decision

Introduction

This paper will discuss solutions for SFN wrap-around.

Discussion

In RAN2#121bis, RAN2 agrees with the below for the SFN wrap-around issue.

- To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

In RAN2#122, further agreements are achieved.

- Not use broadcast signalling for counter and reference SFN in XR.

Combined with the agreements above, we understand the solution on H-SFN is already excluded since H-SFN is indicated by using broadcast signalling.

On whether to choose E-SFN or a generic counter, we prefer the latter one considering the reasons below.

- Compared to a generic counter, E-SFN is configured with a configurable maximum value. While, in our view, the solution on E-SFN also has a wrap-around issue when the maximum value approaches, although a proper configuration can let the probability of the issue as low as possible.

- If a generic counter is used, this counter can be initialized when DRX is configured/activated as well as be increased for a long time to ensure no interruption occurs during an XR session. Using a generic counter seems like no problem since a similar way is already introduced from Rel-16 to resolve SFN wrap-around issue for CG.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For SFN wrap-around issue in the DRX formula, introduce a generic counter which increments by one every time that SFN wraps around.

On when/how to initialize the generic counter, we understand it can be maintained at the UE side and initialized when the UE receives the IE of reference SFN that is configured via dedicated RRC (e.g. DRX-config). As the way we design for Rel-16 CG, both parameters (i.e. the generic counter and reference SFN) are added to the DRX formula. In the formula, the generic counter is initialized as 0 and the reference SFN is configured as either 0 or 512 for the RRC configuration ambiguity issue. In detail, reference SFN is configured as 0 if the dedicated RRC configuration is sent between SFN 0 and 512, otherwise, reference SFN is configured as 512. Accordingly, the SFN used for the determination of the offset for DRX is the closest SFN with the indicated number preceding the reception of the dedicated RRC signalling(e.g. DRX-config).

Similar to the way designed for Rel-16 IIoT, the network configures DRX reference SFN via dedicated signalling, whose value can be set as either 0 or 512.

Similar to the way designed for Rel-16 IIoT, both the generic counter and the DRX reference SFN are added to the DRX formula. The initial value of the generic counter is 0.

Conclusion

We have the following proposals:

Proposal 1	For SFN wrap-around issue in the DRX formula, introduce a generic counter which increments by one every time that SFN wraps around.

Proposal 2	Similar to the way designed for Rel-16 IIoT, the network configures DRX reference SFN via dedicated signalling, whose value can be set as either 0 or 512.

Proposal 3	Similar to the way designed for Rel-16 IIoT, both the generic counter and the DRX reference SFN are added to the DRX formula. The initial value of the generic counter is 0.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] TS 23.501 v18.2.2