3GPP TSG-RAN WG2 Meeting #123	R2-2308197

Toulouse, France, 21– 25 August 2023

Agenda item:	7.16.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	AI/ML Data collection

SID:	FS_NR_AIML_air (Rel-18)

Document for:	Discussion and Decision

# Introduction

A new agenda item was added for RAN2#121bis-e regarding data collection aspects.

7.16.2.2 Data Collection Expect to continue evaluation, e.g. evaluation of cases / methods wrt different LCM purposes. Determine which tangible issues if any (e.g. performance aspects) should/could be considered for later decisions on data collection.

Initial assumption that was made in the previous RAN2#120 meeting:

AIML methods: - R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. - R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). - It is allowed to discuss/determine that functionality can be done outside 3GPP system scope, i.e. OTT server. NO agreement for now on the specifics due to long discussion. - Proposal (modified) Requirements for Data collection should include data collection for model updates / offline training, and non-real-time monitoring (for decision to retrain etc) - For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

The post email discussion [1] in RAN2#120, the following proposals were loosely agreed.

Proposal 1 RAN2 to simultaneously focus on studying data collection solutions for both NW- and UE-sided AIML models, including assistance signalling and (dataset) reporting from the concerning entity. Proposal 2 Study RAN2 implications of data collection for all concerning LCM purpose, e.g., model training/monitoring/selection/update/inference/etc. Proposal 3 RAN2 to separately analyse the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted. Proposal 4 Wait for RAN1 requirements before discussing specific data collection solutions for use cases and for the related (LCM) procedures. In the meantime, RAN2 can summarize the implementation of existing frameworks while focusing on different performance metrics. Proposal 5 When summarizing the different data collection frameworks, RAN2 can start by considering the following metrics: a) the content of the data, b) the data size, c) latency and periodicity, d) signalling, entities involved, and configuration aspects. FFS on how to handle security/privacy. Proposal 6 Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed. Proposal 7 Upon receiving specific (RAN1) requirements, RAN2 to decide whether the existing frameworks can be reused/extended, or whether a new framework is required. Proposal 8 For data collection, RAN2 will simply keep progressing and will inform of concerning agreements to RAN1 when necessary.

Other agreements related to data collection in RAN2#121 [2, 3, 4] are as follows:

R2-2300708 Open issues on AI/ML model delivery and data collection in post-meeting email discussion Apple discussion Rel-18 FS_NR_AIML_air The table in this doc is endorsed as starting point R2-2302286 Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple) Apple Endorse the table as a starting point (e.g. can add more columns if needed later, modify, add rows etc). Content shall be interpreted as current content. Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose

In accordance to last RAN2#121bis-e, the following agreements have been made:

Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s). Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM). P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information. P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement. FFS how we do the formatting/presentation of the results. P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results. P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training) - - Model sidedness (UE side, NW side, two sided) FFS - - Use case mapping FFS P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

In the last RAN2#122 the following agreements were made

P1a: For the LS to RAN1 on data collection requirement, inform RAN1 that the reply should be per use case and per LCM purpose (i.e., Model training, inference and monitoring), and LCM sidedness should also be considered. RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. P2a: LS to ask RAN1 to provide the required data content per use case and per LCM purpose, when available, and to what extent said data would / should be specified (in detail). P2b: LS to ask RAN1 about the reporting type (e.g., periodic, event triggered, other) of the identified data content. P3: LS to ask RAN1 about the typical size (value or value range) of the identified data content. P4a: For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection. P4b: LS to RAN1 to confirm the WA (in P4a) on the latency requirement, and ask RAN1 about the typical latency requirement (value or value range) to transfer the identified data content. P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed. P6b: LS to RAN1 to confirm the WA (in P6a) on RRC state of data collection. P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF. P5b: LS to RAN1 to confirm the WA (in P5a) on the generation entity and termination entity of the identified data content and ask for supplement, if any.

In this contribution, we will discuss the mapping of data collection frameworks with model/functionality life cycle management and also signalling aspects of these frameworks for use cases.

# Discussion

## General aspects of data collection

### 2.1.1 Mapping with different LCM operations

To enable the deployment of more efficient AIML models to support current and future use-cases a large amount of data needs to be collected. Several data collection mechanisms have already been standardized in 3GPP, and they were also captured in the table endorsed as a starting point for RAN2 discussion of data collection [2]. With appropriate enhancements, they have the potential to enable any of the identified LCM purposes. An obvious limitation of these frameworks is that they have fixed termination points, which do not necessarily match with the locations where each of the LCM purposes will be supported.

Observation 1: The termination points of each of the existing frameworks that are being analysed in RAN2 do not necessarily match with the locations where each of the LCM purposes will be supported. The table below summarizes the termination point for each of the frameworks (as per existing agreements).

Table 1: Data Collection Frameworks and their Termination Points

| Framework | Termination point | Protocol |
| --- | --- | --- |
| Immediate MDT | TCE | RRC (to gNB) |
| L3 measurements | gNB-CU | RRC |
| L1 measurement reporting (CSI reporting) | gNB-DU | L1/L2-signalling |
| UAI | gNB-CU | RRC |
| LPP | LMF | LPP |

Proposal 1: RAN2 to include the above information to the previously endorsed table [2], i.e., Table A-1 presented in the Appendix.

Without obtaining a deeper understanding on the entity in which each of the agreed LCM purposes will be supported, it is difficult to identify the limitations of the existing frameworks and enhancements needed to support different LCM purposes. RAN2 needs to first agree on the location where each of these LCM purposes will be fulfilled.

Observation 2: It is difficult to characterize the enhancements of the agreed frameworks needed to enable the different LCM purposes without knowing where each of these purposes will be supported.

Proposal 2: RAN2 to study and define the termination points for each of the identified LCM purposes.

In the last meeting, we agreed to study the applicability of each of the data collection frameworks for the identified LCM purposes. We note that these LCM purposes are already supported for the existing legacy features, which utilize the data collection frameworks discussed in the table endorsed as a starting point. Perhaps the main difference between legacy and AIML-driven features is that the latter may require larger amount of data to support each of the identified LCM purposes. In this context, it is of interest to evaluate the suitability of the data collection frameworks for providing the data needed to enable the AIML-driven features.

Observation 3: To support AIML-enabled features a larger amount of data needs to be collected. Table 2  summarizes some of the main aspects that need to be discussed on the matter.

Table 2: Summary of the suitability of the considered legacy data collection frameworks for enabling the different LCM purposes.

| Framework | Inference | Monitoring | (Offline) training |
| --- | --- | --- | --- |
| Immediate MDT | The data collected through the immediate MDT can be used by the gNB for performing inference if the models reside in the gNB. Moreover, immediate MDT can be configured to support direct configuration of measurements in both DU and CU. | Depending on the use-case and latency requirements, the data collected through the immediate MDT can be used for monitoring done in the gNB, CN or (possibly) the NWDAF. | The information contained in the measurement reports collected through the MDT framework can be used for offline training of the models. Training would be supported gNB, OAM and possibly in the NWDAF as well. |
| L3 measurements | The termination point for the L3 measurements is in the gNB-CU. Thus, this framework would be suitable for models which share the inputs configured in the L3 measurement report and run on the gNB-CU. | Depending on the use-case and the associated latency requirements, the information contained in the L3 measurement report can be used for enabling the monitoring of the performance of the AIML-enabled feature on the gNB-CU. | Training of the models would require a large amount of the L3 measurements to be stored in the gNB-CU, as well as a lot of processing of the information from the gNB-CU. |
| L1 measurement reporting (CSI reporting) | The termination point for the L1 measurement reporting is in the gNB-DU. Useful for the use-cases wherein the inference on the NW side needs to be performed with low latency. | Can be useful for use-cases wherein the monitoring happens in the DU (e.g., BM or other MIMO-related use-cases). | The information elements contained in the L1 measurement reporting framework can be useful for training purposes. However, the DU may not be a suitable location for training large (and perhaps highly specialized) models. |
| UAI | The termination point for the UAI reporting is in the gNB-CU. The information contained in it can be used in conjunction with the other frameworks (e.g., L3 measurement reporting) to perform inference. | The information contained in the UAI is very relevant for the purposes of model monitoring. The gNB-CU can exploit the information (e.g., if the UE is overheating or experiencing any other of non-long term issues) to adapt to the potential performance oscillations. | The information elements contained in the UAI can be used as inputs to the models. This would require the CU to forward the information to the training entity. Training the models within the CU may not be feasible due to large data storage and processing requirements. |
| LPP | Suitable for transmitting the UE's location and/or intermediate features. For new types of intermediate features, LPP could be extended. | Suitable for real-time performance monitoring. LPP supports bi-directional messaging, so the UE could also be provided with information necessary for monitoring on the UE. | If the LMF is capable of training, LPP messages could be extended (through LPP message segments) to support larger volumes of training data. |

Proposal 3: Include the discussion in the Table 2 in the table that was endorsed as a starting point for the data collection frameworks comparison [2], i.e., Table A-1 presented in the Appendix.

Based on the aforementioned discussion, we can observe that the information collected from the existing legacy data collection frameworks can provide valuable input for supporting the identified LCM purposes despite of the entity in the NW wherein these LCM purposes are supported. Enhancing these frameworks would be sufficient for fulfilling the data collection requirements for the use-cases that are currently being considered in Release 18.

Observation 4: Enhancing the data collection frameworks endorsed in the table presented in [2] is sufficient for fulfilling the data collection requirements for the use-cases that are being considered in Release 18.

Proposal 4: RAN2 to prioritize studying the data collection frameworks presented in the endorsed table [2] and presented in Table A-1 in the Appendix.

### 2.1.2 Discussion on offline training of the models

In the past RAN2 meetings there has been significant discussion on the different approaches and frameworks that should be followed for training the UE and NW-sided AIML models. Observing the challenges that were faced in reaching consensus, we think that determining and endorsing a set of basic principles would facilitate the process of finding a way forward.

#### 2.1.2.1 Principles of training data collection for offline training of UE-sided models

Several discussions on the training data collection have been carried through in the past RAN2 meetings, wherein concerns were raised on the location where the UE-sided models should be trained, and how the training data would be sent to the right location (network entity/function).

Observation 5: Before defining a way forward on the framework that will be considered for enabling the data needed for training the AIML models in Rel 18, 3GPP should define a set of principles that will be followed to ensure that the requirements for data collection are met.

The need for training data collection procedures is relevant for all the Release 18 air-interface use cases currently being studied in RAN1 and RAN2. Nevertheless, data collection for analytics, including ML-enabled solutions, have been already discussed and agreed in RAN3 [5] and SA5 groups as well [6, 7, 8]. Therefore, we believe, for training data collection in ML enabled feature in Rel-18 air-interface,  the framework to be adopted needs a) to be aligned with data collection solution defined in SA; b) to be future proof; c) to be able to scale further to new use cases which will emerge in follow-up Release 19, 20, etc.

Observation 6: The training data collection framework agreed for Release 18 should be future proof and extendable to similar use-cases.

The primary concerns with any data collection in 3GPP, and in general, is the end-user privacy and security preservation. These are the main requirements which need to be obviously met by the training data collection framework.

Observation 7: The training data collection framework agreed for Release 18 should be ensure end-user security and privacy.

The currently studied Release 18 use cases are expected to require training data to ensure the ML models and functionality performance generalizes sufficiently well. Thus, it would be important that the training data collection framework to be adopted ensures that additional CP and/or UP air-interface traffic is minimized.

Observation 8: The data collection framework should ensure that additional CP and/or UP air-interface traffic is minimized.

Similarly, transferring high volumes of data may not be possible at all times and at all locations in the network, as these would be impacting the overall performance of the network as well. Accordingly, some mechanisms need to be studied to enable prioritization of the data to be transferred between the entities, e.g., based on data content or characteristics, as well as its transmission depending on the link quality (of the air interface) between the involved entities.

Observation 9: High volumes of data necessitate prioritization of data transfer among entities by considering data content/characteristics as well as link conditions on the air interface.

Last, but not least, the training data collection procedure for any specific UE and Network vendor purposes is likely to be complex and time consuming. Therefore, it would be in the interest of all parties to be able to share their collected data such that significantly large amounts of the training data would be available to each party. This training data sharing would also help reduce the additional CP and UP air-interface traffic. Naturally, RAN2 mechanisms can enable and facilitate this data sharing process.

Observation 10: The training data collection framework should enable sharing the collected data to subscribed (both internal and external to the NW) parties considering use case specific details.

Proposal 5: RAN 2 to endorse the following principles to be used for the design of the training data collection framework;

- Future proofness and extensibility (to similar use-cases),

- User security and privacy should be preserved,

- Ensure that additional CP and/or UP air-interface traffic is minimized,

- Any prioritization depending on data content/characteristics and link conditions,

- Enable sharing of training data among the subscribed parties (use case specific),

FFS if more principles need to be considered.

#### 2.1.2.2 Discussion on the frameworks for training data collection

Considering the previous meetings in RAN2, there has been a lot of discussion on whether the existing frameworks are sufficient for enabling the collection of data that can be used for the (offline) training of NW and UE-sided models. As can be anticipated, building the AIML models would require the collection of data from different deployment scenarios and radio conditions. Thus, it would be desirable to have a framework that is designed for this purpose. Among the identified frameworks, the MDT framework is designed to minimize the drive tests that need to be performed to collect different types of RRM measurements. These measurements can be used for building NW-sided AIML models for different air interface use-cases. Based on the current Release 18 study, examples of such use-cases include CSI feedback enhancements, BM, Positioning accuracy enhancements etc. More details on how MDT can be used for collecting data for performing the (offline) training of the AIML models that arise in these cases will be presented later in the paper. They can also be used for building UE-sided AIML models for different use-cases, thus making MDT compliant with Principle 1 identified in Proposal 5.

Observation 11: The MDT framework is designed to minimize the drive tests that need to be performed to collect different types of RRM measurement data, including measurements which have already been identified as inputs to the models by RAN1.

Observation 12: The measurements collected via MDT can be used for building NW and UE-sided AIML models for different air interface use-cases, thus complying with Principle 1 identified in Proposal 5.

Aspects related to user privacy and security are ensured by the MDT consent framework [9, subclause 4.9], which ensures that user consent is needed before the MDT functionality is activated.

Observation 13: Data can be securely collected via MDT from the users who have given their consent, thus complying with Principle 2 identified in Proposal 5.

There seems to be a common understanding among all the parties involved in 3GPP that the data collection process is expected to have a large overhead associated with it. Moreover, RAN1 has concluded in their study that many of the measurements collected by the UE/NW (e.g., high resolution ground truth CSI measurements for the CSI use-case, L1-RSRP measurements for the BM use-case, PRS-RSRP/RSRPP, SS-RSRP/RSRQ for the Positioning use-case etc.) can be used for training both NW and UE-sided models. Thus, it would be desirable to enable the data sharing among the parties that are interested in it, as this allows for minimizing the overhead that might arise due to the transfer of the measurements. This is possible to achieve through the MDT framework, since both the entities in the NW that will train the NW-sided models and the entities outside the NW that will train the UE-sided models can have access to the collected MDT reports.

Observation 14: Enabling the availability of the measurements among the subscribed parties that interested in using them to extract the data needed for training the AIML models would result in a decrease in the overhead caused by the process of collecting data for the (offline) training of the models, thus complying also with Principle 3 identified in Proposal 5.

Observation 15: The MDT framework enables the measurement to be collected and retrieved among the subscribed parties that are interested in extracting the data needed for training the AIML models, thus complying also with Principle 3 identified in Proposal 5.

Moreover, MDT sessions are configured from the NW. This facilitates the scheduling and control of the data volumes that are requested by the UE at different radio link conditions, points in time and locations in the NW, while minimizing the impact of the data collection for offline training purposes in the overall NW performance.

Observation 16: The MDT framework is configurable by the NW; thus, it is simpler for the NW to enable the data collection process at different radio link conditions. This minimizes the impact of data collection for offline training in the overall NW performance and is compliant with Principle 4 identified in Proposal 5.

MDT measurement reports (in the form of trace records) are transmitted from the gNB to the TCE, which is identified by an IP address. Thus, to our understanding, entities which can be both internal and external to the NW, can serve as a TCE as long as they are identified by a trusted IP address, which can be achieved by signing the appropriate agreements with the MNO.

Observation 17: The MDT framework enables secured access to data collection by interested/subscribed parties through user consent and trusted destination address, thus complying with Principle 4 identified in Proposal 5.

Another issue that was raised during the discussion in the past RAN2 meetings is related to the additional measurements that are performed by the UE, in addition to the RRM measurements that are requested by the NW. To our understanding, these can relate to sensor information, SNR, Doppler etc., and can be used as auxiliary input for training the UE-sided models. Under the current MDT framework, adding new metrics to the measurement report would require acceptance from all the working groups involved in standardizing MDT in 3GPP, which can be time-consuming. Nevertheless, considering TS 32.423 [10], we can see that record extension fields, defined as a set of manufacturer specific extensions to the record, are considered for the purpose of tracing (as an example see Table 4.30.1 therein). Within the same specification, we can see that this record extension is missing for MDT.

Observation 18: A record extension field supporting a set of manufacturer specific extensions to the record has already been defined for tracing purposes in TS 32.423, [10, Table 4.30.1], however it is missing for MDT.

In our view, the record extension placeholder can be introduced for MDT and used for the purpose of reporting the additional measurements that are needed as auxiliary input for building the UE-sided models.

Proposal 6: RAN2 to study the inclusion of the record extension field in the MDT framework. FFS on how the NW regulates the content and transmission of the record extension field.

Introducing the record extension field in the MDT framework would also require new signalling capabilities to be defined for the purpose of collecting the measurements from the UE and forwarding them to the TCE. From RAN2 viewpoint, this would require adding new fields in the RRC message to configure the UE on how this information needs to be transmitted.

Proposal 7: RAN2 to study the signalling capabilities needed to support the collection of the additional information from the UE when using the MDT framework and forwarding them towards the TCE.

## Use-case specific aspects

Here we will discuss the use-cases.

### 2.2.1 CSI feedback enhancements

For the CSI use-case, RAN1 has identified that AIML features can be enabled either by two-sided models (for compression), or by UE-sided or NW-sided models for prediction. Thus, in the sequel we will present our view on the data collection requirements and frameworks for these cases.

#### 2.2.1.1 Offline Training

For CSI compression, RAN1 has identified the following model training collaborations:

RAN1#110 Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. RAN1#111 Conclusion In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

Based on their study, RAN1 has concluded to deprioritize Type 2 training collaboration over the air interface (not including model update) and gradient-exchange based sequential training over the air interface for the Release 18 SI. Thus, we suggest that also RAN2 deprioritizes these aspects related to data collection for Type 2 training as well.

Proposal 8: RAN2 to assume offline training collaboration when considering data collection methods for model training.

To enable Type 1 training, if the model is trained at the NW side, our understanding is that UE would have to report all measurements to the NW, for the latter to train both the encoder and the decoder. This implies that both the standardized and additional measurements (e.g., non-standardized measurements, assistance information etc.) would need to be shared by the UE to the NW.

Observation 19: For Type 1 training, if NW trains the two-sided model, then it needs to collect all the relevant measurements from the UE-side.

On the other hand, if the UE trains the two-sided model, this will most likely be done in an OTT that will (possibly) be external to the NW and under the control of the UE vendor. In this case, relevant standardized and additional measurements, done by both the UE and the NW side, need to be forwarded to the OTT. If each side forwards these measurements separately, then this would increase the signalling overhead needed (e.g., alignment between the collected samples might be needed). In our view, this coordination would be simpler if the UE forwards the measurements to the NW first.

Observation 20: For Type 1 training, if UE-side trains the two-sided model, having the UE forward the data to the NW would simplify their coordination.

Among the frameworks that are currently being studied in RAN2 for collecting data for the (offline) training of the AIML models, the MDT framework enables the collection of the data for both situations which arise in Type 1 training. In case the NW trains the AIML model, the UE can be configured to provide the measurements to the NW, which can build the dataset needed to train the model. Similarly, if the UE side is to train the model first, the NW can still configure the UE to report the measurements, and then forward those to the TCE. The measurements can then be retrieved and collected from the interested and subscribed parties.

Observation 21: The MDT framework enables the collection of the data needed for the Type 1 training for the CSI compression use-case.

Proposal 9: RAN2 to prioritize the study of the MDT framework for Type 1 training for the CSI compression use-case.

Another type of training that is being considered is the Type 2 training. As discussed above, only the variants which involve gradient exchange in an offline manner are currently being considered for Type 2 training. For this training type, the only part which may have RAN2 impact is the construction of the initial datasets, on top of which the gradients are being computed. After the data is collected, the gradient exchange is done offline, thus we do not see further RAN2 impact.

Observation 22: The only part which may have an impact for Type 2 training with offline gradient exchange is the construction of the initial dataset.

To enable Type 2 training, both UE and NW would need to forward the datasets to the respective training entities. In the context of the MDT framework, this process is easy. For the UE-side, MDT can forward the data to the TCE, from which it can then be shared among the interested and subscribed parties. Similarly, for the NW-side, MDT can also be used to forward the data to the identified training entities (e.g., OAM, CN etc.).

Observation 23: The MDT framework enables the collection of the data needed to build the initial datasets needed for the Type 2 training for the CSI compression use-case. After the initial datasets are built, the gradient exchange can be done offline.

Proposal 10: RAN2 to prioritize the study of the MDT framework for collecting the information needed for enabling Type 2 training for the CSI compression use-case.

The last training type that is being considered in RAN2, is the Type 3 training, which comprises of separate training at the NW and UE side. To our understanding, this approach also has two cases, respectively, the UE-first approach and the NW-first approach. In the NW-first approach, the UE has to forward the relevant data (including non-standardized information and assistance information) to the NW for the latter to build the encoder-decoder pair. Afterwards, using the trained encoder-decoder pair, the NW generates another dataset, which is sent to the OTT under the control of the UE vendor. This part does not have impact on the air interface and is outside of RAN2 scope.

Observation 24: For Type 3 training, in case the NW trains first, only the initial collection of the data in the NW-side has an impact over the air interface.

Proposal 11: For Type 3 training, in case the NW trains first, RAN2 to prioritize the study of mechanisms that would enable the collection of the dataset from the UE, i.e., ground truth CSI information.

When considering Type 3 training, the remaining possibility is that UE trains first, and then the NW trains afterwards. Under this paradigm, our understanding is that the UE would forward the relevant information to the OTT. The NW would also forward the needed assistance information to the OTT, and the latter would do the training for the encoder and decoder pair, and would generate a dataset which would be shared with the NW. The NW would then generate the decoder based on the provided encoder. Similar to the case wherein the NW trains first, this dataset exchange falls outside the scope of RAN2.

Observation 25: For Type 3 training, in case the UE trains first, only the initial collection of the data from the UE has an impact over the air interface.

Proposal 12: For Type 3 training, in case the UE trains first, RAN2 to prioritize the study of mechanisms that would enable the collection of the dataset from the UE.

Considering the frameworks that have been agreed to be studied in RAN2, the relevant frameworks seem to be L1 measurements, UAI and MDT. Moreover, in our view, these frameworks are also compliant with the principles identified above for training of UE-sided models.

Proposal 13: For Type 3 training, RAN2 to prioritize the study of MDT, L1 measurement framework and UAI.

The remaining aspect to be considered for CSI compression is related to the venue wherein the models will be trained. In all the different training types identified above, the UE-side of the model is trained in an OTT. On the other hand, the NW-sided models can be trained either in the OAM or in the CN. In our view, details relating to the venue where the training will happen can be considered for analysis after the discussion on the data collection framework has matured.

Observation 26: For the CSI use-case, it is difficult for RAN2 to conclude on a suitable venue for training the AIML models, as OTT, OAM and/or CN fall outside the scope of RAN2.

Proposal 14: For the CSI use-case, RAN2 to prioritize aspects related to the analysis of the identified data collection frameworks and study the signalling needed to enable the collection of the data. After that is understood, RAN2 can evaluate whether the identified training venues fall within the scope of RAN2.

Another topic that is being considered in the context of CSI feedback enhancements is the CSI prediction. In this case, time series datasets of CSI measurements are required for both training and monitoring purposes. For training data collection, saving a group of CSI measurement data by the UEs and subsequent delivery of the accumulated data to the network side can provide benefits of overhead reduction and network performance compared to real-time delivery of the measurement. RAN1 has identified that it can be enabled either by NW or UE-sided models. Moreover, in either case, some additional information may be needed, e.g., in the form of non-standardized data, assistance information etc. Similar to the analysis conducted for the CSI compression use-case, the data collection needs for training UE and NW-sided models can be fulfilled by considering the MDT, L1 measurement reporting and UAI.

Proposal 15: For the CSI prediction use case, RAN2 to prioritize the study of the MDT, L1 measurement reporting and UAI.

#### 2.2.1.2 Inference and Monitoring

During inference, the main CSI for scheduling would be reported using the configuration modified to support AI/ML.  However, for data collection purposes, the legacy configuration can be used (as a baseline) with potential modifications. During monitoring, the raw measurements (ground truth CSI/ target CSI) need to be signalled along with the compressed CSI, but not necessarily with the same reporting frequency. In addition, high accuracy measurements might be desired. The impact of granularity of the collected ground-truth CSI are still under discussion in RAN1. All these requirements can be supported by the existing frameworks with the appropriate enhancements. Based on these, we can summarize the data collection requirements should focus on:

- The coordination between CSI measurement configuration and reporting

- Enhancement needed for the accuracy and frequency of the target CSI measurement reports

- Additional measurement and reporting configurations for CSI, e.g., time-series measurements for CSI prediction, grouping of CSI measurements

Moreover, the information collected via the legacy CSI measurement and reporting framework can also be embedded in the MDT reports, which can be made available to the gNB, TCE and the CN.

Observation 27: For the monitoring function of the CSI compression use case, both the CSI reporting and immediate MDT frameworks can be used to report ground-truth CSI.

Proposal 16: RAN2 to analyse the usability and any required enhancement of the legacy CSI measurement and reporting and the MDT frameworks for data collection in the CSI compression use case for the inference and monitoring functions.

### 2.2.2 Beam management

For the BM use-case, RAN1 has identified that AIML features can be enabled either by UE-side models or by NW-side models. Thus, in the sequel we will present our view on the data collection requirements and frameworks for these cases.

#### 2.2.2.1 Offline Training

Let us first start by discussing the training of the AIML models. For the BM use-case, RAN1 has concluded that inference can be done in the gNB or the UE. To our understanding, this implies that AIML models for the BM use-case can be built for being deployed in the gNB or the UE. Moreover, for Release 18, RAN2 has already agreed to limit the scope of training AIML models to (offline) training only. Thus, in the sequel we consider the data requirements and approaches for collecting data to enable the (offline) training of the corresponding models.

For UE-sided models, the identified inputs are set A and set B beams. Specifically, L1-RSRP for Set B beams used as training data together with beam ID of the best beam in Set A and/or L1-RSRP of Set A beams used for labelling the training data. Moreover, from both UE and NW-sides, it is also expected that additional information will be transmitted. Examples of this additional information can be non-standardized measurements collected by the UE and/or the NW, as well as assistance information. Assistance information may be used as input to the model for improving the model prediction performance while preserving privacy/proprietary information. A second use may be to assist the UE for categorize the data collected. For instance, by providing a NW configuration identifier and/or other characteristics information coupled to NW configuration used while the UE is collecting the data. For both the UE and NW-side models, the data would need to be collected and sent to the entities that will perform the training.

Observation 28: For either NW or UE-sided models, the data that needs to be collected for training the models is expected to consist of standardized measurements and additional information (e.g., non-standardized measurements and assistance information).

Among the frameworks that are currently being studied in RAN2 for collecting data for the (offline) training of the AIML models, the MDT framework enables the collection of both standardized and additional information for both NW and UE-sided models. Through MDT, the measurements can then be forwarded to the TCE, where they can be collected and retrieved among the subscribed parties. In our view, considering the type of information that needs to be collected for the purpose of training the AIML models, the L1 measurements and UAI can also be useful.

Observation 29: MDT, L1 measurements framework and UAI can be used to collect the data needed for the (offline) training of NW and UE-sided AIML models.

Proposal 17: RAN2 to study further enhancements needed for MDT, L1 measurement reporting framework and UAI to enable data collection for (offline) training for the BM use-case.

The remaining aspect to be considered for the BM use-case is related to the venue wherein the models will be trained. Similar to the CSI use-case, the UE-sided models are expected to be trained in an OTT. On the other hand, the NW-sided models can be trained either in the OAM or in the CN. In our view, details relating to the venue where the training will happen can be considered for analysis after the discussion on the data collection framework has matured.

Observation 30: For the BM use-case, it is difficult for RAN2 to conclude on a suitable venue for training the AIML models, as OTT, OAM and/or CN fall outside the scope of RAN2.

Proposal 18: For the BM use-case, RAN2 to prioritize aspects related to the analysis of the identified data collection frameworks and study the signalling needed to enable the collection of the data. After that is understood, RAN2 can evaluate whether the identified training venues fall within the scope of RAN2.

#### 2.2.2.2 Inference and Monitoring

As discussed in the above section, for BM both NW and UE-sided models would be feasible. In either of the cases, the inference process/function is expected to have a tight delay requirement associated with it. Inference is expected to have a tight delay requirement associated with it. Regarding the data requirements, for either of the models, the expectation is that they would be similar to the data that is collected for the purposes of (offline) training, i.e., there would be Set B measurements, as well as additional information. In Release 18, inference is expected happens directly at the UE or the gNB, it is likely that the additional information would mainly consist of assistance information given to the side that is performing the inference. For this purpose, we believe that the L1 measurements framework and the UAI can serve as a baseline for enabling the data collection for the inference purpose.

Observation 31: The L1 measurements framework and UAI can collect the data needed for performing inference while complying with its low latency requirements.

Proposal 19: RAN2 to prioritize the study the L1 measurements framework and the UAI for the purpose of data collection for inference.

Similar to inference, monitoring is also expected to have some latency requirement associated with it. Despite the latency requirement (which might be more tolerant than the one for inference), monitoring is also expected to require the transmission of the labels, similar to (offline) training.

Observation 32: For the monitoring LCM purpose, the selected framework needs to support the collection of measurements, additional information and labels, while complying with the low latency requirements.

Among the frameworks that are being considered for this purpose, the L1 measurements framework, UAI and MDT seem to be suitable for collecting the required information. Among these frameworks, the L1 measurements framework and UAI are suitable also for enabling the collection/transmission of the information at low latency.

Proposal 20: RAN2 to prioritize the study the L1 measurements framework and the UAI for the purpose of data collection for inference.

### 2.2.3 Positioning enhancements

Similar to the other use-cases, for positioning, the following cases have been identified in RAN1 [11]:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

From the above, we understand that both NW-side (namely, LMF-side and gNB-side) and UE-sided models can be suitable for enabling AIML-based positioning enhancements. Thus, in the sequel we will present our view on the data collection requirements and frameworks for these cases.

In the current positioning framework [11], LMF plays a central role, which could determine the positioning method, configure gNBs and the target UE for transmission and reception of DL PRS or UL SRS, collect measurements, and calculate the location estimate. Further, in the case of UE-based positioning, LMF may provide the positioning measurements to the UE, which can calculate its own location estimate.

Positioning measurements (such as time-based, angle-based, etc.) and its reporting between LMF and UE are well-defined within the LPP protocol, which could relate to both RAT-dependent and RAT-independent (GNSS, sensor, etc.) positioning methods. However, to support newly introduced positioning-related measurements, such as CIR, which have been agreed in RAN1 during the Rel. 18 study item, LPP protocol need to be further enhanced.

Observation 33: To support newly introduced positioning-related measurements, such as CIR, which have been agreed in RAN1 during the Rel. 18 study item, LPP protocol needs to be further enhanced.

Proposal 21: Enhance LPP to support configuration and exchange of positioning-related measurements considered in RAN1, between LMF and UE.

Similarly, the new measurements agreed in RAN1 to be conducted by gNB, e.g., UL CIR measurements, such as in the case of gNB-side models needs to be also introduced to the NRPPa protocol between gNB and LMF. This, however, falls into the RAN3 scope.

Observation 34: Enhancements to NRPPa are required to support new positioning positioning-related measurements considered in RAN1, which falls into RAN3 scope.

Proposal 22: RAN2 to consider enhancements needed for NRPPa and consult RAN3 if/when needed.

#### 2.2.3.1 Offline Training

Similar to other use-cases, for the scope of Release 18 it is expected that the NW and UE-sided models would be trained externally to the RAN. For the NW-sided models, the identified venues are the OAM and the CN. For UE-sided models, it is likely that they are trained in an OTT which is in the control of the UE vendor. Among the frameworks that are being considered for the study item in RAN2, as discussed in the previous sections, MDT enables the forwarding of the measurements to a TCE, wherein they can be collected and retrieved among the subscribed parties.

Observation 35: The MDT framework enables the forwarding of the measurements for to the TCE, from which they can be collected and retrieved among the subscribed parties.

In our view, for the positioning use-case, the study of MDT should cover at least the following two aspects: 1) The configuration of data collection and the transport of data from an LMF to a TCE; 2) The transport of the additional measurements, which can be non-standardized measurements or assistance information related to positioning, from UEs and/or NW to a data collector or TCE.

Proposal 23: For the positioning use-case, the study of MDT should at least consider the following.

- The configuration of data collection and the transport of data from an LMF to a TCE

- The transport of the additional measurements, which can be non-standardized measurements or assistance information related to positioning, from Ues and/or NW to a data collector or TCE.

To support data collection for offline training for positioning, a mechanism is required to control, configure, and trigger data collection at an LMF, and transport collected data from an LMF to a TCE. Similar to how MDT configures and collects data from a gNodeB, MDT could configure an LMF to configure Ues to collect measurements, transport them from UEs to the LMF through LPP, and then forward them to a TCE. As discussed in the earlier sections, the data can be collected and retrieved from the subscribed parties. This part is particularly important for the positioning use-case, since the information contained in the measurements can be sensitive.

Observation 36: Collecting the positioning-related measurements via MDT ensures that they can be retrieved by the subscribed parties, while at the same time preserving the user security and privacy.

In addition to standardized types of data, such as the positioning-related measurements as specified in LPP, additional data may also be used for the positioning use case. In general, UE, LMF or gNB vendors may use specific proprietary models for their AI/ML model for positioning purposes, which require additional measurements (e.g., assistance information, or additional measurements to be performed by the gNB or UE on top of what is configured by the LMF). Similarly, it is not clear from RAN2 viewpoint whether there is a need to standardize each granularity or ranges of values of measurements desired by different vendors, e.g., 0.25 dB steps of RSRP.

Observation 37: Considering the use-cases in Release 18 AIML study, RAN may need to support the transport of both standardized information and additional information.

To enable the transmission of additional measurements to the different subscribed parties, the network may define policies on the collection of additional measurements and share these with the involved entities, including Ues and gNBs. Based on such policy, the external parties may send their data collection request to the network. The policy may indicate allowed/disallowed types of measurements, vendors, area/time associated with data, protocols to be used for data collection, etc.

Proposal 24: RAN2 to work on enabling the configuration of a policy on the collection of additional information (e.g., on what measurements can be shared and how) among the different parties, including at least UE, gNB and OTT.

Considering the different cases wherein inference can happen for the positioning use-case, which have been identified in [11], the existing frameworks for collecting the data for the different models which may reside in the UE, gNB or LMF need to be enhanced. In our view, it is up to RAN2 to study the enhancements needed for this purpose.

Proposal 25: FFS what additional measurements need to be shared to support the data collection needs for the purpose of (offline) training.

Last, the additional measurements that are collected in the LMF via MDT, still need to be forwarded to the termination point of the protocol, i.e., TCE, wherein they can later be collected and retrieved from the subscribed parties.

Proposal 26: RAN2 to study the mechanisms needed to enable the transmission of additional measurements from the LMF to the TCE.

In the case of positioning, it would be more efficient to examine the already collected data and then identify the required additional data to collect to reach the required ML model accuracy. For the positioning use case, this stands for the analysis of the spatial distribution of available training data and thereafter proceed with a precise identification of the required data to complement the existing dataset. Doing so ensures both efficient data collection (avoiding unnecessary collection and reporting) and model training (with better training set).

Proposal 27: RAN2 to study mechanisms to examine and identify additional data to collect in accordance with required model accuracy.

Proposal 28: RAN2 to define Area based MDT request to collect MDT data more precisely based on the geographical location of UEs

The gateway mobile location center (GMLC), which facilitates requests of UE location estimates, is another possible entity that could accept and forward requests for data collection for AIML positioning models. It is able to trigger via the AMF and LMF LPP sessions to collect UE location estimates. As with the MDT option, the location request procedures through the GMLC are able to check for consent. However, as with the MDT option, there does not exist a mechanism to retrieve and transmit positioning measurements made by and collected by a UE or a gNodeB to any entity other than an LMF. To support this option, the GMLC or location services (LCS) framework would need to be enhanced to support the transmission of the positioning measurements from the LMF to the GMLC, which would ultimately forward the measurements to the requestor.

Observation 38: The GMLC is an entity which supports the collection of UE location estimates and is able to trigger LPP sessions through the core network to the LMF.

Observation 39: The GMLC does not support the request or reception of UE positioning measurements from the LMF and the LMF does not support the request or transmission of UE positioning measurements to other entities.

Proposal 29: Study the enhancement of the LCS framework involving the GMLC and the LMF to support the data collection request to a GMLC, the reception of the data collection request by the LMF, and the reception at the GMLC of UE positioning measurements.

#### 2.2.3.2 Inference and Monitoring

From [11], we have the following:

Model monitoring: - Data for computing monitoring metric: - If monitoring based on model output: e.g., estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input: e.g., measurement corresponding to model inference input. - Assistance signalling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring. - Assistance signalling from UE/PRU for network-side model monitoring. - If certain type of data is necessary for computing monitoring metric: - How an entity can be used to provide the given type of data for calculating monitoring metric: companies requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signalling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction: e.g., model monitoring decision indication between UE and network - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data. - Note: the measurement(s) may or may not be the same as model input. - Assistance signalling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label. - Provisioning of ground truth label and associated label quality. - Assistance signalling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision

For the purpose monitoring UE-side models using ground truth information, the LMF may need to provide the UE with measurement data, and additionally, the associated ground truth, when the UE is the entity to derive the monitoring metric, which it would transmit to the LMF. When the LMF is the entity to derive the monitoring metric, the UE transmits the inference output from the provided monitoring measurement data to the LMF, which uses the ground truth to derive the monitoring metric itself.

Currently, measurement data in positioning flows in one direction: from the UE to the LMF. Here we will consider UE-side monitoring with monitoring assistance from the LMF. The LMF could provide monitoring dataset(s) containing measurement data, e.g., measurements made on PRS such as RSRP, RSRPP, AoA, and ToA, to the UE. We have identified two modes for the LMF monitoring UE-side models: mode 1 and mode 2.

In mode 1, wherein the UE calculates the monitoring metric:

- LMF also provides the ground truth, e.g., positioning estimates or intermediate features, and KPI threshold to the UE

- UE performs inference and calculates a KPI using its model’s output and provided ground truth

- UE optionally provides KPI to the LMF

And in mode 2, wherein the LMF calculates the monitoring metric:

- UE performs inference and provides its model’s output to the LMF

- LMF calculates a KPI using the UE’s model’s output and the ground truth associated with the monitoring dataset.

Observation 40: For UE-side monitoring with LMF assistance using ground truth information, there is no mechanism in the current version of LPP to transmit measurement data, i.e., associated with the measurement dataset, from the LMF to the UE.

Proposal 30: Enhance LPP to support transmission of monitoring datasets, e.g., measurement data and/or ground truth labels, to a UE for the purpose of monitoring.

# 3	Conclusion

The document has made the following observations.

Observation 1: The termination points of each of the existing frameworks that are being analysed in RAN2 do not necessarily match with the locations where each of the LCM purposes will be supported. The table below summarizes the termination point for each of the frameworks (as per existing agreements).

Table 1: Data Collection Frameworks and their Termination Points

| Framework | Termination point | Protocol |
| --- | --- | --- |
| Immediate MDT | TCE | RRC (to gNB) |
| L3 measurements | gNB-CU | RRC |
| L1 measurement reporting (CSI reporting) | gNB-DU | L1/L2-signalling |
| UAI | gNB-CU | RRC |
| LPP | LMF | LPP |

Observation 2: It is difficult to characterize the enhancements of the agreed frameworks needed to enable the different LCM purposes without knowing where each of these purposes will be supported.

Observation 3: To support AIML-enabled features a larger amount of data needs to be collected. Table 2 summarizes some of the main aspects that need to be discussed on the matter.

Observation 4: Enhancing the data collection frameworks endorsed in the table presented in [2] is sufficient for fulfilling the data collection requirements for the use-cases that are being considered in Release 18.

Observation 5: Before defining a way forward on the framework that will be considered for enabling the data needed for training the AIML models in Rel 18, 3GPP should define a set of principles that will be followed to ensure that the requirements for data collection are met.

Observation 6: The training data collection framework agreed for Release 18 should be future proof and extendable to similar use-cases.

Observation 7: The training data collection framework agreed for Release 18 should be ensure end-user security and privacy.

Observation 8: The data collection framework should ensure that additional CP and/or UP air-interface traffic is minimized.

Observation 9: High volumes of data necessitate prioritization of data transfer among entities by considering data content/characteristics as well as link conditions on the air interface.

Observation 10: The training data collection framework should enable sharing the collected data to subscribed (both internal and external to the NW) parties considering use case specific details.

Observation 11: The MDT framework is designed to minimize the drive tests that need to be performed to collect different types of RRM measurement data, including measurements which have already been identified as inputs to the models by RAN1.

Observation 12: The measurements collected via MDT can be used for building NW and UE-sided AIML models for different air interface use-cases, thus complying with Principle 1 identified in Proposal 5.

Observation 13: Data can be securely collected via MDT from the users who have given their consent, thus complying with Principle 2 identified in Proposal 5.

Observation 14: Enabling the availability of the measurements among the subscribed parties that interested in using them to extract the data needed for training the AIML models would result in a decrease in the overhead caused by the process of collecting data for the (offline) training of the models, thus complying also with Principle 3 identified in Proposal 5

Observation 15: The MDT framework enables the measurement to be collected and retrieved among the subscribed parties that are interested in extracting the data needed for training the AIML models, thus complying also with Principle 3 identified in Proposal 5.

Observation 16: The MDT framework is configurable by the NW; thus, it is simpler for the NW to enable the data collection process at different radio link conditions. This minimizes the impact of data collection for offline training in the overall NW performance and is compliant with Principle 4 identified in Proposal 5.

Observation 17: The MDT framework enables secured access to data collection by interested/subscribed parties through user consent and trusted destination address, thus complying with Principle 4 identified in Proposal 5.

Observation 18: A record extension field supporting a set of manufacturer specific extensions to the record has already been defined for tracing purposes in TS 32.423, [10, Table 4.30.1], however it is missing for MDT.

Observation 19: For Type 1 training, if NW trains the two-sided model, then it needs to collect all the relevant measurements from the UE-side.

Observation 20: For Type 1 training, if UE-side trains the two-sided model, having the UE forward the data to the NW would simplify their coordination.

Observation 21: The MDT framework enables the collection of the data needed for the Type 1 training for the CSI compression use-case.

Observation 22: The only part which may have an impact for Type 2 training with offline gradient exchange is the construction of the initial dataset.

Observation 23: The MDT framework enables the collection of the data needed to build the initial datasets needed for the Type 2 training for the CSI compression use-case. After the initial datasets are built, the gradient exchange can be done offline.

Observation 24: For Type 3 training, in case the NW trains first, only the initial collection of the data in the NW-side has an impact over the air interface.

Observation 25: For Type 3 training, in case the UE trains first, only the initial collection of the data from the UE has an impact over the air interface.

Observation 26: For the CSI use-case, it is difficult for RAN2 to conclude on a suitable venue for training the AIML models, as OTT, OAM and/or CN fall outside the scope of RAN2.

Observation 27: For the monitoring function of the CSI compression use case, both the CSI reporting and immediate MDT frameworks can be used to report ground-truth CSI.

Observation 28: For either NW or UE-sided models, the data that needs to be collected for training the models is expected to consist of standardized measurements and additional information (e.g., non-standardized measurements and assistance information).

Observation 29: MDT, L1 measurements framework and UAI can be used to collect the data needed for the (offline) training of NW and UE-sided AIML models.

Observation 30: For the BM use-case, it is difficult for RAN2 to conclude on a suitable venue for training the AIML models, as OTT, OAM and/or CN fall outside the scope of RAN2.

Observation 31: The L1 measurements framework and UAI can collect the data needed for performing inference while complying with its low latency requirements.

Observation 32: For the monitoring LCM purpose, the selected framework needs to support the collection of measurements, additional information and labels, while complying with the low latency requirements.

Observation 33: To support newly introduced positioning-related measurements, such as CIR, which have been agreed in RAN1 during the Rel. 18 study item, LPP protocol needs to be further enhanced.

Observation 34: Enhancements to NRPPa are required to support new positioning positioning-related measurements considered in RAN1, which falls into RAN3 scope.

Observation 35: The MDT framework enables the forwarding of the measurements for to the TCE, from which they can be collected and retrieved among the subscribed parties.

Observation 36: Collecting the positioning-related measurements via MDT ensures that they can be retrieved by the subscribed parties, while at the same time preserving the user security and privacy.

Observation 37: Considering the use-cases in Release 18 AIML study, RAN may need to support the transport of both standardized information and additional information.

Observation 38: The GMLC is an entity which supports the collection of UE location estimates and is able to trigger LPP sessions through the core network to the LMF.

Observation 39: The GMLC does not support the request or reception of UE positioning measurements from the LMF and the LMF does not support the request or transmission of UE positioning measurements to other entities.

Observation 40: For UE-side monitoring with LMF assistance using ground truth information, there is no mechanism in the current version of LPP to transmit measurement data, i.e., associated with the measurement dataset, from the LMF to the UE.

The document has proposed the following.

Proposal 1: RAN2 to include the above information to the previously endorsed table [2], i.e., Table A-1 presented in the Appendix.

Proposal 2: RAN2 to study and define the termination points for each of the identified LCM purposes.

Proposal 3: Include the discussion in the Table 2 in the table that was endorsed as a starting point for the data collection frameworks comparison [2], i.e., Table A-1 presented in the Appendix.

Proposal 4: RAN2 to prioritize studying the data collection frameworks presented in the endorsed table [2] and presented in Table A-1 in the Appendix.

Proposal 5: RAN 2 to endorse the following principles to be used for the design of the training data collection framework;

- Future proofness and extensibility (to similar use-cases),

- User security and privacy should be preserved,

- Ensure that additional CP and/or UP air-interface traffic is minimized,

- Any prioritization depending on data content/characteristics and link conditions,

- Enable sharing of training data among the subscribed parties (use case specific),

FFS if more principles need to be considered.

Proposal 6: RAN2 to study the inclusion of the record extension field in the MDT framework. FFS on how the NW regulates the content and transmission of the record extension field.

Proposal 7: RAN2 to study the signalling capabilities needed to support the collection of the additional information from the UE when using the MDT framework and forwarding them towards the TCE.

Proposal 8: RAN2 to assume offline training collaboration when considering data collection methods for model training.

Proposal 9: RAN2 to prioritize the study of the MDT framework for Type 1 training for the CSI compression use-case.

Proposal 10: RAN2 to prioritize the study of the MDT framework for collecting the information needed for enabling Type 2 training for the CSI compression use-case.

Proposal 11: For Type 3 training, in case the NW trains first, RAN2 to prioritize the study of mechanisms that would enable the collection of the dataset from the UE, i.e., ground truth CSI information.

Proposal 12: For Type 3 training, in case the UE trains first, RAN2 to prioritize the study of mechanisms that would enable the collection of the dataset from the UE.

Proposal 13: For Type 3 training, RAN2 to prioritize the study of MDT, L1 measurement framework and UAI.

Proposal 14: For the CSI use-case, RAN2 to prioritize aspects related to the analysis of the identified data collection frameworks and study the signalling needed to enable the collection of the data. After that is understood, RAN2 can evaluate whether the identified training venues fall within the scope of RAN2.

Proposal 15: For the CSI prediction use case, RAN2 to prioritize the study of the MDT, L1 measurement reporting and UAI.

Proposal 16: RAN2 to analyse the usability and any required enhancement of the legacy CSI measurement and reporting and the MDT frameworks for data collection in the CSI compression use case for the inference and monitoring functions.

Proposal 17: RAN2 to study further enhancements needed for MDT, L1 measurement reporting framework and UAI to enable data collection for (offline) training for the BM use-case.

Proposal 18: For the BM use-case, RAN2 to prioritize aspects related to the analysis of the identified data collection frameworks and study the signalling needed to enable the collection of the data. After that is understood, RAN2 can evaluate whether the identified training venues fall within the scope of RAN2.

Proposal 19: RAN2 to prioritize the study the L1 measurements framework and the UAI for the purpose of data collection for inference.

Proposal 20: RAN2 to prioritize the study the L1 measurements framework and the UAI for the purpose of data collection for inference.

Proposal 21: Enhance LPP to support configuration and exchange of positioning-related measurements considered in RAN1, between LMF and UE.

Proposal 22: RAN2 to consider enhancements needed for NRPPa and consult RAN3 if/when needed.

Proposal 23: For the positioning use-case, the study of MDT should at least consider the following.

- The configuration of data collection and the transport of data from an LMF to a TCE

- The transport of the additional measurements, which can be non-standardized measurements or assistance information related to positioning, from UEs and/or NW to a data collector or TCE.

Proposal 24: RAN2 to work on enabling the configuration of a policy on the collection of additional information (e.g., on what measurements can be shared and how) among the different parties, including at least UE, gNB and OTT.

Proposal 25: FFS what additional measurements need to be shared to support the data collection needs for the purpose of (offline) training.

Proposal 26: RAN2 to study the mechanisms needed to enable the transmission of additional measurements from the LMF to the TCE.

Proposal 27: RAN2 to study mechanisms to examine and identify additional data to collect in accordance with required model accuracy.

Proposal 28: RAN2 to define Area based MDT request to collect MDT data more precisely based on the geographical location of UEs

Proposal 29: Study the enhancement of the LCS framework involving the GMLC and the LMF to support the data collection request to a GMLC, the reception of the data collection request by the LMF, and the reception at the GMLC of UE positioning measurements.
Proposal 30: Enhance LPP to support transmission of monitoring datasets, e.g., measurement data and/or ground truth labels, to a UE for the purpose of monitoring.

# 4	References

[1] R2-2301440	Outcome of [Post120][054][AIML18] Data Collection	Ericsson, vivo	discussion	Rel-18 FS_NR_AIML_air

[2] R2-2300708	Open issues on AI/ML model delivery and data collection in post-meeting email discussion	Apple

discussion	Rel-18	FS_NR_AIML_air

[3] R2-2302286 	Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)	Apple

[4] 3GPP TSG-RAN WG2 Meeting #121, Greece, Athens, Chair Notes R2_121 Chair Notes EOM.docx (live.com)

[5] TR 38.214 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancement for Data Collection for NR and EN-DC (Release 17)”, Version 17.0.0, 3GPP

[6] TS 28.104 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management Data Analytics (MDA) (Release 18)”, Version 18.0.1, 3GPP

[7] TS 28.105 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management (Release 18)”, Version 18.0.0, 3GPP

[8] TS 28.908 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management (Release 18)”, Version 1.2.0, 3GPP

[9] TS 32.422 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Subscriber and equipment trace; Trace control and configuration management (Release 17)”, Version 17.10.0, 3GPP

[10] TS 32.423 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Subscriber and equipment trace; Trace data definition and management (Release 17)”, Version 17.6.0, 3GPP

[11] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Version 0.1.0, 3GPP

[12] TR 38.305 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17)”, Version 17.5.0, 3GPP

# Appendix

Table A-1: Progress table of analyzing data collection framework (Apple)	Apple [5].

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic report and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |

Note:

* The payload size doesn't consider signaling overhead.

** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.

*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g. entering CONNECTED state, report interval)

****Air interface signaling latency is the latency to transmit one report, e.g. RRC signaling latency or PUCCH signaling latency.