3GPP TSG-RAN WG2 Meeting #123			         R2-2308200

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.2.1

Source: 	LG Electronics Inc.

Title:         	PTM configuration and session deactivation

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss remaining issues on the PTM configuration.

Discussion

  - PTM configuration

For multicast reception in RRC_INACTIVE, RAN2 agreed to support two different ways to provide the PTM configuration to UEs. The PTM configuration can be transmitted to individual UEs via DCCH when the RRC connection is suspended, or it can be broadcast via multicast MCCH. However, it is still unclear whether network should always transmit the same PTM configuration using two different channels, or can select one channel to use for transmission of the PTM configuration depending on the situation.

When network determines to simultaneously suspend the RRC connection of numerous UEs receiving multicast session to mitigate RAN congestion, it may prefer to broadcast the PTM configuration using multicast MCCH to reduce the signalling overhead, rather than transmitting the same PTM configuration to individual UEs using DCCH. Though the PTM configuration is not provided during RRC release, UE can acquire it before/after entering RRC_INACTIVE.

Some multicast interruption may happen until acquiring the PTM configuration via multicast MCCH. However, such multicast interruption is already accepted when UE receiving the multicast session in RRC_INACTIVE moves to another cell. After cell re-selection, the UE can acquire the PTM configuration only via MCCH. Therefore, network should be able to save dedicated radio resource by not transmitting the PTM configuration to each UE via DCCH.

Observation 1	If the PTM configuration is being broadcast via multicast MCCH, the network may not want to separately provide the same PTM configuration to each UE via DCCH.

Proposal 1 	It is up to network implementation whether to provide the PTM configuration in RRCRelease message when suspending RRC connection.

The multicast MCCH would be useful when the PTM configuration needs to be simultaneously provided to numerous UEs, e.g., when the PTM configuration is updated or when offloading numerous UEs to RRC_INACTIVE, but continuously broadcasting the PTM configuration throughout the congestion just for UEs that may come at any time is very signalling inefficient. Though UE finds out that the new serving cell does not broadcast the PTM configuration, the UE can initiate the RRC connection resume and acquire the PTM configuration via DCCH. RAN2 already agreed to support the scenario that UE initiates the RRC connection resume if the PTM configuration is not available in the re-selected cell. Therefore, network should be able to suspend or resume the multicast MCCH transmission even for multicast sessions which can be received in RRC_INACTIVE.

Observation 2	Network may want transmit the PTM configuration via multicast MCCH, only when the PTM configuration needs to be simultaneously provided to numerous UEs, e.g., when the PTM configuration is updated.

Proposal 2	It is up to network implementation whether to transmit the PTM configuration via multicast MCCH for multicast sessions which can be received in RRC_INACTIVE.

Since RRCRelease with suspend config message can be used to deliver the PTM configuration, if the network knows that the RRC resume is initiated to acquire the PTM configuration upon receiving RRCResumeRequest from UE, the network can provide the PTM configuration via the RRCResumeRequest-RRCRelease with suspend configuration sequence. Then, UE is able to acquire the PTM configuration without transitioning to RRC_CONNECTED. This approach could be a good alternative to periodic broadcast of the PTM configuration via multicast MCCH, especially when the gNB doesn't know which multicast sessions UEs are receiving in RRC_INACTIVE from neighbour cells. To achieve this the UE needs to use a new resume cause, such as ptm-Configuration, so that the network can decide to include the PTM configuration in the RRCRelease message in response to the RRCResumeRequest.

Proposal 3	UE can request the transmission of the PTM configuration using RRCResumeRequest message with a new resume cause, such as ptm-Configuration.

Proposal 4	In response to the RRC resume request, gNB can provide the PTM configuration via RRCRelease with suspend configuration.

  - Notification for session activation/de-activation

Regarding the notification for session activation, followings was agreed in the last meeting:

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

The group paging has been used to notify the multicast session activation since Rel-17. The only enhancements required to support multicast reception in RRC_INACTIE is to indicate whether the activated multicast session can be received in RRC_INACTIVE or not. Since RAN2 agreed to introduce a new indication per tmgi in the group paging for this purpose, no further enhancement is needed.

Proposal 5	Apart from the new indication per tmgi agreed in the previous meeting, no further group paging enhancements is needed.

RAN2 also agreed following regarding the notification for session de-activation:

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

UE is notified of the session stop by the broadcast MCCH message not including the session identity of the corresponding multicast session. When the session is de-activated, the same mechanism can be used, and this is why most companies preferred to use MCCH to notify the session de-activation. When a multicast session is de-activated, network will first transmit the multicast MCCH change notification to UEs and exclude the de-activated multicast session from the multicast MCCH message. If an UE finds that the PTM configuration of the multicast session that the UE is receiving disappears in the multicast MCCH message, the UE can consider the multicast session is de-activated and stop monitoring corresponding G-RNTI.

Proposal 6	If the PTM configuration of the multicast session that the UE is receiving disappears in the multicast MCCH message, UE considers the multicast session is de-activated and stop monitoring corresponding G-RNTI.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1	If the PTM configuration is being broadcast via multicast MCCH, the network may not want to separately provide the same PTM configuration to each UE via DCCH.

Proposal 1 	It is up to network implementation whether to provide the PTM configuration in RRCRelease message when suspending RRC connection.

Observation 2	Network may want transmit the PTM configuration via multicast MCCH, only when the PTM configuration needs to be simultaneously provided to numerous UEs, e.g., when the PTM configuration is updated.

Proposal 2	It is up to network implementation whether to transmit the PTM configuration via multicast MCCH for multicast sessions which can be received in RRC_INACTIVE.

Proposal 3	UE can request the transmission of the PTM configuration using RRCResumeRequest message with a new resume cause, such as ptm-Configuration.

Proposal 4	In response to the RRC resume request, gNB can provide the PTM configuration via RRCRelease with suspend configuration.

Proposal 5	Apart from the new indication per tmgi agreed in the previous meeting, no further group paging enhancements is needed.

Proposal 6	If the PTM configuration of the multicast session that the UE is receiving disappears in the multicast MCCH message, UE considers the multicast session is de-activated and stop monitoring corresponding G-RNTI.