3GPP TSG-RAN WG2 Meeting #123			         R2-2308202

Toulouse, France, August 21-25, 2023

Agenda Item:	7.25.1

Source: 	LG Electronics Inc.

Title:         	Measurement reporting for FR2 unknown SCell activation enhancement

Document for: 	Discussion and Decision

Introduction

In the last meeting RAN2 provided the following answers to RAN4 questions regarding FR2 unknown SCell activation enhancement,

Question 1: How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement [RAN2 Answer to Q1] - The unkown SCell measurement report is configured via RRC measurement configuration (i.e. measConfig). Question 2: How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement [RAN2 Answer to Q2] - The unkown SCell measurement report is triggered by SCell activation command MAC CE. Question 3: Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report [RAN2 Answer to Q3] - Legacy RRC MeasurementReport message is used for the unkown SCell measurement report, which can already carry the beam level L3 measurement result and beam index.

In this contribution, we discuss detailed signalling design and impact on RAN2 spec.

Discussion

When an unknown SCell is activated, the measurement results on other serving cells than the activated SCell may not need to be reported. However, in the existing measurement report procedure, the measurement results on all serving cells for which servingCellMO is configured are mandatorily reported in Measurement Report message. Considering the SCell activation does not occur very often, it does not seem to be desirable to modify the measurement reporting procedure to exclude the measurement results on other serving cells just for the purpose of signalling reduction. Therefore, it is proposed that when the measurement report is triggered by SCell activation command, the UE reports SSB based measurement results on all serving cells, i.e. cell results and measurement information per beam, for which servingCellMO is configured.

Proposal 1	If the measurement report is triggered by SCell activation command, UE reports SSB based measurement results on all serving cells, i.e., cell results and measurement information per beam, for which servingCellMO is configured without measurement results on neighbour cells.

UE doesn’t know which SCell is unknown cell from network perspective. To avoid the UE reporting measurement results when it is not needed, i.e. when a known SCell is activated, the network should explicitly inform the UE of whether the L3 measurement reporting is necessary or not, when transmitting the SCell activation command.

Proposal 2	When activating unknown SCell, network explicitly indicates whether to report the L3 measurement results of serving cells, e.g., via enhanced SCell activation command.

For FR2 unknown SCell activation enhancement, the measurement reporting does not need to be repeated and the measurement information per beam should be included always. Furthermore, since the measurement results on neighbor cells do not need to be reported, there is no parameters that need to be configured in the reporting configuration. Therefore, it would be better to define a new report type for FR2 unknown SCell activation enhancement, rather than extending an existing report type.

Proposal 3	Define a new report type for FR2 unknown SCell activation enhancement.

Though a single SCell is activated, if the measurement report procedure is used to report L3 measurement results of the activated SCell, the measurement results of other SCells will be reported together. Therefore, the measurement identity does not need to be separately configured per each SCell, and network only needs to configure at most one measurement identity associated with the new report type across all CGs.

Proposal 4	Network can configure at most one measurement identity associated with the new report type across all CGs.

Proposal 5	Upon receiving enhanced SCell activation command, UE considers the measurement reporting is triggered for a measurement ID which is associated with the new report type.

If the proposal 4 is accepted, we also need to discuss which measurement object can be associated with the new type of reporting configuration for FR2 unknown SCell activation enhancement. Actually, no matter which measurement object is associated with the new type of reporting configuration, the measurement results of serving cells including the activated unknown SCell will be normally reported. However, since the measurement reporting includes only measurement results of serving cells, it seems natural to associate the new type of reporting configuration with one of servingCellMO.

Proposal 6	The new type of reporting configuration can be associated with one of servingCellMO.

Conclusion

Proposal 1	If the measurement report is triggered by SCell activation command, UE reports SSB based measurement results on all serving cells, i.e., cell results and measurement information per beam, for which servingCellMO is configured without measurement results on neighbour cells.

Proposal 2	When activating unknown SCell, network explicitly indicates whether to report the L3 measurement results of serving cells, e.g., via enhanced SCell activation command.

Proposal 3	Define a new report type for FR2 unknown SCell activation enhancement.

Proposal 4	Network can configure at most one measurement identity associated with the new report type across all CGs.

Proposal 5	Upon receiving enhanced SCell activation command, UE considers the measurement reporting is triggered for a measurement ID which is associated with the new report type.

Proposal 6	The new type of reporting configuration can be associated with one of servingCellMO.