GPP TSG-RAN WG2 Meeting #123	R2-2308230

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Msg3 indication for PUCCH repetition for Msg4 HARQ-ACK

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached some working assumptions. In the RAN1 LS R1-2306105, RAN2 is expected to confirm if the WA reached by RAN1 is feasible. The topic has been briefly discussed in RAN2#122 meeting while it was postponed to RAN2#123 for further discussion.

In this document, we elaborate more and provide our view on how to indicate the information related to PUCCH repetition for Msg4 HARQ-ACK.

# 2	Discussion

To indicate the PUCCH repetition for Msg4 HARQ-ACK, RAN1 agreed the working assumption that UE should indicate the ‘repetition request or capability report’ via higher layer signalling in Msg3 PUSCH.

On the information content, RAN1 further provide an additional working assumption for details. RAN1 assume it is a two-state information in which state 1= ‘repetition request or capability report’ and state 2 = ‘no indication’. Furthermore, in state1, they are not differentiated in the signaling for either repetition request or capability report.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions. - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication. - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission. - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling. - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.

In our understanding, the RAN1 LS implies that a 1-bit information is enough to indicate the two-state information related to ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 1: RAN2 to confirm the understanding for RAN1 LS that UE indicates 1-bit information related to ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

When the UE initial access to network from RRC idle, UE includes the CCCH SDU in the Msg3 containing the UE identity for requesting setup of an RRC connection to the network. A CCCH SDU contains UL-CCCH-Message or UL-CCCH1-Message, and in turn included in a MAC PDU together with a MAC subheader. The MAC subheader associated to a MAC SDU containing UL CCCH consists of the two header fields R/LCID, wherein the LCID field is of size 6 bits whereas the two R fields are 2bits.

Figure1: R/LCID MAC subheader

For the UL-CCCH message, it is a set of 48-bits RRC messages that may be sent from the UE to the network, including RRC messages such as rrcSetupRequest, rrcResumeRequest, rrcReestablishmentRequest and rrcSystemInfoRequest. For the UL-CCCH1 message, it is a 64-bits RRC message such as rrcResumeRequest1. The RRCSetupRequest RRC message is defined in TS 38.331 as follows:

RRCSetupRequest ::= SEQUENCE { rrcSetupRequest RRCSetupRequest-IEs } RRCSetupRequest-IEs ::= SEQUENCE { ue-Identity InitialUE-Identity, establishmentCause EstablishmentCause, spare BIT STRING (SIZE (1)) } InitialUE-Identity ::= CHOICE { ng-5G-S-TMSI-Part1 BIT STRING (SIZE (39)), randomValue BIT STRING (SIZE (39)) } EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess, spare6, spare5, spare4, spare3, spare2, spare1}

To deliver the 1-bit information carried by higher layer signalling in Msg3 PUSCH, in theory, the information can be carried either in RRC layer or MAC layer. For example:

- Option1: define new LCID(s) index indicating not only the size of CCCH in the Msg3 (48 and 64 bits) but also PUCCH repetition UE capability or request. or

- Option2: re-purpose the reserved R bits in the MAC subheader of the Msg3, or

- Option3: re-purpose the spare bit currently present in the UL CCCH or UL CCCH1 messages (e.g. in RRCSetupRequest message as above).

All the options will not cause the increase of the Msg3 size which is good for the UL coverage.  Option1 is a typical method to deliver the UE capability to network and MAC has used some reserved LCIDs for UE capabilities such as whether the UE is a RedCap UE. As indicated below from TS38.321 Table 6.2.1-2, there are seven reserved LCID values which can be potentially used for the PUCCH repetition indication. Considering the support of RedCap UE, four of the seven reserved LCID index would be needed to support simultaneous indication of CCCH for a RedCap/non-RedCap UE and CCCH size of 48 and 64 bits. The option1 drawback is it would leave only few available reserved LCIDs for future applications. Option2 and option3 both re-purpose one reserved/spare bit to indicate the 1-bit information, which has no impact to early indication of RedCap UE and non-RedCap UE via LCID. However, in option2, one of the two reserved bits (R bits) would be used and cause even less flexibility than the case of LCID for future applications. In option3, the one spare bit would be exhausted while it has the advantage of limiting the reporting of UE capability to only when UE does not have a configured dedicated PUCCH resource configuration and is requesting the setup of such RRC configuration.

Observation 1: The single bit proposed by RAN1 can be included in Msg3 without increasing the size of Msg3.

Table 1: Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

We tend to suggest following legacy behaviour and use option1 as baseline but we are open to option2 and option3 for further discussion. Since either of opiton1/2/3 is workable, we believe RAN2 can confirm the working assumption included in the RAN1 LS.

Proposal 2: RAN2 to confirm it is feasible to support Option B proposed by RAN1 LS (i.e., to indicate the ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK via higher layer signalling in Msg3 PUSCH).

Proposal 3: RAN2 to discuss below options for ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

- Option1: define four new LCID(s) index indicating not only the size of CCCH in the Msg3 (48 and 64 bits) but also PUCCH repetition UE capability or request.

- Option2: re-purpose the reserved R bits in the MAC subheader of the Msg3.

- Option3: re-purpose the spare bit currently present in the RRC messages contained in UL CCCH or UL CCCH1.

For Rel-18, it is possible multiple WIs may use (e)LCID(s) as identities of new MAC CEs and/or UE capabilities. The cross WI coordination can be further discussed whether and how to optimize the (e)LCID usage. For example, instead of reserving an LCID or an eLCID for each MAC CE to be introduced in Rel-17 in a fixed manner in the specification, network configuration of an (e)LCID for MAC CE(s) can be considered instead. Furthermore, the LCID-based capability indication may be different for different UL messages. This way, more features can benefit from the lower overhead the LCID can offer and the difficult exercise of selecting the MAC CEs that should only benefit from the LCID can be avoided.

Proposal 4: If option1 is to be used, whether and how to optimize the LCID usage can be discussed for Rel-18 (e.g., considering the cross WI coordination on LCID allocation).

# 3	Conclusion

This document has made the following observations:

Observation 1: The single bit proposed by RAN1 can be included in Msg3 without increasing the size of Msg3.

And proposed the following:

Proposal 1: RAN2 to confirm the understanding for RAN1 LS that UE indicates 1-bit information related to ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 2: RAN2 to confirm it is feasible to support Option B proposed by RAN1 LS (i.e., to indicate the ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK via higher layer signalling in Msg3 PUSCH).

Proposal 3: RAN2 to discuss below options for ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

- Option1: define four new LCID(s) index indicating not only the size of CCCH in the Msg3 (48 and 64 bits) but also PUCCH repetition UE capability or request.

- Option2: re-purpose the reserved R bits in the MAC subheader of the Msg3.

- Option3: re-purpose the spare bit currently present in the RRC messages contained in UL CCCH or UL CCCH1.

Proposal 4: If option1 is to be used, whether and how to optimize the LCID usage can be discussed for Rel-18 (e.g., considering the cross WI coordination on LCID allocation).