3GPP TSG-RAN WG2 Meeting #123	R2-2308232

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QoE for RRC IDLE and RRC INACTIVE

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

To support QoE in RRC IDLE and RRC INACTIVE, RAN2 and RAN3 has progressed with running CRs reflecting agreed principles. This contribution continues discussion on detailed configuration and reporting principles to support QoE Measurements Collection for MBS.

# 2	Discussion

## 2.1	Area Scope configuration

The area scope handling for MBS QoE measurement in different RRC states has been discussed in RAN2-122 meeting. Though RAN3 agreed that in case QoE measurements are collected by the UE in RRC IDLE/INACTIVE state the area scope is checked by the UE, it is not clear for UE in RRC CONNECTED state the area scope can be checked by the UE or the network, or by both the UE and the network. RAN2 raised the identified questions on LS R2-2306569 to other involved groups. Even though, there were questions asked on potentially conflicting requirements (i.e., whether the Location Filter in QoE configuration container can be configured together with Area Scope of QMC in NGAP), meanwhile it became clear that RAN3 restriction to have Area Config mandatory over NGAP has been changed:

RAN3 agreement: Presence of NGAP QoE area scope is changed from MANDATORY to OPTIONAL. CR to TS 38.413 agreed in R3-233430.

Observation 1: For SA4-defined areas scope: Location Filter handling needs further clarification from SA4. For RAN3-defined area scope handling has been relaxed.

As the restriction is specified in TS 26.247 , coordination between LocationFilter and NGAP ‘Area scope of QMC’ needs further clarification and alignment from SA4/SA5/RAN3.

On the area scope checking performed by UE with the configured LocationFilter, it is not clear the checking should be performed in UE’s application layer (AL) or AS layer. The area scope checking in UE’s AL is the most straight-forward way. However, in the previous RAN2  discussions, there are two concerns raised by companies for this option.

- AS layer has to inform the AL the RRC state of the UE.

- AS layer has to inform the AL the cell id after each cell change

For the first concern, it can be addressed if the UE can perform area scope checking for UE in RRC Connected state (on top of the agreed RRC idle/inactive state). Since in Rel-17 the area scope checking for a connected UE can only be done in network side, whether the checking can be done simultaneously in both UE and network depends on SA4/5 response on the coordination between LocationFilter and ‘Area scope of QMC’. For the second concern, it will burden the AS layer to deliver the infromation after each cell change to different applications, considering many applications need to implement the area handling. On the other hand, if the area scope checking is performed in UE’s AS layer, it seems both concerns can be addressed because AS layer itself know everything about RRC state change and cell change.

RAN2 can wait for more progress on the coordination between LocationFilter and ‘Area scope of QMC’, to decide whether the area scope checking in UE should be performed in UE’s AL or AS layer.

Proposal 1: For UE with MBS broadcast services, RAN2 to further discuss the area scope check for QoE should be handled by UE’s AL or AS layer, based on the LS response from SA4/5/RAN3.

## 2.2	QoE configuration for MBS service

RAN2 agreed that the QoE configuration for MBS broadcast service will be provided via dedicated signalling such as RRCReconfiguration and RRCResume message but not RRCRelease message. The UE will then store the configuration and perform QoE measurements accordingly when it transfers to RRC IDLE and INACTIVE. As a UE may be configured with a list of application layer measurement configurations for MBS, when the UE is transferred from CONNECTED to RRC IDLE and INACTIVE state the gNB should indicate which QoE configuration is to be continued (i.e., applicable in RRC IDLE/INACTIVE state).

As a working assumption (WA), RAN2 agreed explicit indicator will be used in AS-layer on whether a QoE configuration is applicable in RRC-IDLE/INACTIVE states. But it is still open whether the explicit indicator is needed if a new service type of MBS is to be introduced which may be used as implicit indicator.

RAN2 agreements: The QoE configuration indicates the applicable states (i.e. that the QoE measurements for CONNECTED are supposed to be gathered also in RRC_IDLE/INACTIVE). FFS whether this is explicit or implicit. As working assumption, RAN2 will use explicit indicator in AS-layer on whether a QoE configuration is also applicable in RRC-IDLE/INACTIVE states. Can be revisited if RAN3 decides to introduce a service type.

RAN3 is asking SA4 on the service type for MBS in the LS R3-233457. MBS can be considered as a new “service type” or alternatively as a “communication service” for QoE measurement collection for the existing service type. It is up to SA4 to decide which option should be pursued in Rel-18, but the decision will impact whether the RAN2 WA can be confirmed.

Excerpt from LS R3-233457: - Option 1: Consider MBS as a new “Service Type” for QMC of existing service types (e.g., DASH, MTSI, VR) and for MBS specific QoE metrics. This might require SA4 to define MBS-specific QoE metrics. - Option 2: Consider MBS as a communication service, which is based on its definition in TS 23.247. In this context, the intention is to support QMC for existing service types carried via an MBS broadcast session or MBS multicast session. In this case, the QoE measurement configuration would consider the MBS delivery mode (e.g., broadcast or multicast) used in the session.

Proposal 2: Wait for RAN3/SA4 progress on how to define MBS service for QMC to decide whether the WA about explicit indicator can be confirmed.

## 2.3	UEs selection for QMC

Given the MBS broadcast service characteristics it can be provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast service area are authorized to receive the data). To ensure controlled data collection through all RRC states of the UEs, RAN2#119 identified an FFS to discuss the way how the gNB selects the MBS users:

Agreement: FFS how does gNB determine which UEs can be configured with MBS QoE measurements

In our understanding, there are two issues need to be addressed for UE selection.

The first issue is how network can identify the candidate UEs which will take actual involvement in receiving the MBS services (and in particular broadcast services). When UE is in connected state then network can know the UE has interest for reception of broadcast services, since UE can indicate its current interest in a specific session ID via the MBS interest indication (MII) message. However, the message is only optional. A UE can only indicate the MBS interest in current RRC state (connected state) for a certain session ID that is about to start or that is ongoing, without distinction whether the MBS session is of interest for the future such as in RRC IDLE/INACTIVE. Due to no precise nor reliable mean to determine the actual involvement of the UE in MBS session, one method for QoE configuration is to only select the UE who reports MII message, but this means network only has very limited opportunities to configure QoE measurements since most of the UEs receiving MBS broadcast session will remain in RRC_IDLE or INACTIVE as much as possible and will only connect to the network due to other reasons. The other method is that gNB blindly configure the QoE configurations to UEs. However, the QoE configuration blindly configured to UEs may cause a waste of signalling since most of these configurations will never be used by the UEs (as no MBS reception at all).

To address the first issue, RAN2 may need to consider a reliable mean to determine the actual involvement of the UE in MBS session. For example, when the UE in connected mode, gNB may request the UE to report in case that it has some MBS session ID it is interested in the future for instance when the UE might be in IDLE/INACTIVE state (“likely interest in MBS session indication”) which could be tied to QoE measurement. Therefore, gNB can configure the UE QoE configurations based on the actual involvement of MBS reception in the future.

Proposal 3: RAN2 to consider a reliable mean to determine the actual involvement of the UE in MBS session, to enable gNB identify proper candidate UEs to configure the QoE configuration for MBS.

The second issue is how to avoid massive data reporting due to long QoE sessions for UE in RRC idle and inactive. In contrary to Rel-17, Rel-18 QoE for MBS broadcast service might result in a massive reporting. To save the Uu interface signalling overhead, there is no need to configure all the candidate UEs in an area to collect QoE measurements. In fact, from network point of view, it is sufficient to configure only a limited number of UEs in an area for QoE collection to benchmark the QoE of the MBS service. In other words, to ensure the constrains with buffering data and generating higher load, the QoE configuration should support the option to choose UEs in a selective manner.

The gNB may select UEs based on the requirements on how to benchmark MBS service quality. For example, network may select the UEs in a randomized way in which the QoE reports from the selected UEs can represent the overall MBS service quality of a dedicated area. Another way is that network may select UEs who have potential to have poor MBS service experience.

Proposal 4: Among the candidate UEs involved in MBS session, the gNB can select UEs for QMC either in a randomized way or only select UEs that experience poor MBS service experience.

## 2.4	QoE reporting

According to the agreements in RAN2#120 meeting, as a baseline, UE does not trigger RRC connection setup or resume just for the sake of reporting QoE measurement results. It is FFS whether there are cases where UE can trigger RRC connection Setup for the sake of reporting QoE.

Agreement: As a baseline, UE does not tigger RRC Resume – RRC Setup just for the sake of reporting QoE. FFS whether there are cases where we deviate from this baseline.

Since the UE needs to buffer the QoE data for as long as it stays in RRC idle or inactive, it means the stored data may impact the UE memory seriously. When the UE’s AS layer buffer is full, the UE has to discard the old QoE reports and store the new one received Application from layer, or vice versa.  For the MBS transmission observability, e.g., to compare reception of the service in idle/inactive UEs, we believe Rel-18 solution should provide certain means to overcome discarding QoE data at a high rate. One possibility is to extend UE memory requirement, but it can only mitigate the issue instead of resolving the issue. From network point of view, if the system load is low enough, it makes no sense to forbid UE to trigger the RRC connection for the QoE data report before the UE discards it. Therefore, we would like to suggest that gNB could control whether the UE is allowed to resume or setup the RRC connection to report the QoE data which is to be discarded.

Proposal 5: When UE’s QoE buffer is full, the UE is allowed to trigger RRC Resume or Setup to report the QoE data if it is allowed by network.

Furthermore, RAN2 agreed that UE’s AS layer should discard the QoE data if the AS layer buffer is full. As default behaviour, the UE should discard older reports to make room for the new one. It is FFS any assistance information should be provided to UE.

Agreement: As a default behavior, when the UE’s buffer for storing QoE reports is full and a new report arrives, the UE should discard older report(s) to make room for the new one. FFS whether it is possible to provide information (e.g. priority, service type, etc.) to UE about buffering for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full.

As indicated in the SA5 LS (S5-232760) , in case of the UE’s QoE buffer is full, different consumers may have different selection policies. Therefore, the gNB may need to provide assistance information to UE to decide which reports should be discarded.

SA5 has the following responses to RAN2's questions: Question 4: In case of limited storage space for QoE reports at the UE, is there any preference from the OAM side on which QoE reports should be reported and which should be discarded, e.g. is there a principle that newer or older reports are more useful for the network? SA5: From SA5’s point of view, some selection policies from consumers could be possible, but there are no use cases for it yet. But as a consistent behavior is wanted from all UEs, a default behavior should be to prioritize new data.

In RAN3 discussion, as indicated in LS R3-232047, most companies preferred to introduce a priority per QoE configuration that can be used as the assistance information. However, considering there may have multiple types of consumers for receiving the QoE reports, RAN3 is still asking SA5 to confirm whether the OAM can provide the explicit reporting priority per QoE configuration to gNB, or provide  the “characteristics of reporting” for different “type of consumer” to gNB.

Proposal 6: gNB can provide assistance information to UE on how to discard QoE reports when the AS buffer becomes full. The exact content of the assistance information includes at least priority information, FFS on additional content needed.

# 3	Conclusion

This document has made the following observations:

Observation 1: For SA4-defined areas scope: Location Filter handling needs further clarification from SA4. For RAN3-defined area scope handling has been relaxed.

And proposed the following:

Proposal 1: For UE with MBS broadcast services, RAN2 to further discuss the area scope check for QoE should be handled by UE’s AL or AS layer, based on the LS response from SA4/5/RAN3.

Proposal 2: Wait for RAN3/SA4 progress on how to define MBS service for QMC to decide whether the WA about explicit indicator can be confirmed.

Proposal 3: RAN2 to consider a reliable mean to determine the actual involvement of the UE in MBS session, to enable gNB identify proper candidate UEs to configure the QoE configuration for MBS.

Proposal 4: Among the candidate UEs involved in MBS session, the gNB can select UEs for QMC either in a randomized way or only select UEs that experience poor MBS service experience.

Proposal 5: When UE’s QoE buffer is full, the UE is allowed to trigger RRC Resume or Setup to report the QoE data if it is allowed by network.

Proposal 6: gNB can provide assistance information to UE on how to discard QoE reports when the AS buffer becomes full. The exact content of the assistance information includes at least priority information, FFS on additional content needed.