3GPP TSG-RAN WG2 Meeting #123	R2-2308233

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Rel-17 leftovers

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 agreed in RAN2#122 the following the respect to RVQoE reporting:

Agreement: - RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer. - RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting. - RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18.

Moreover, with respect to QoE support for shared spectrum the following was agreed by RAN2#122:

Agreement: - Application layer measurement configuration and reporting for shared spectrum channel access is not supported in R18.

In this contribution we elaborate further on the Rel-17 leftover topics in context of the required RAN2 enhancements.

# 2	Discussion

## 2.1	RAN visible QoE threshold-based triggering

Based on the SA4 reply LS to RAN2 in [1], where SA4 confirmed RAN2’s preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18, RAN2 agreed in RAN2#122 that buffer level threshold-based triggering of RVQoE reporting is triggered by the application layer.

However, RAN2 did not further progress on the buffer level threshold-based RVQoE reporting details as discussions are currently pending in RAN3. In RAN3#119bis-e it was agreed that no radio-related event triggers are supported for RVQoE reporting in Rel-18. RAN2 in turn confirmed this agreement in RAN2#122.

Furthermore, in RAN3#119bis-e it was agreed that additional discussion is needed with respect to the methods how the UE should send the RVQoE reports after the buffer level threshold is met. As part of the discussion the following options were listed:

- RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options:

  - Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

  - Option 2: Periodically based on a gNB configured reporting periodicity

  - Option 3: A certain number of times based on gNB configured report amount

Yet, RAN3 did not make any final decision with respect to the aforementioned options. Therefore, no further RAN2 agreements on the above option can be added before RAN3 makes a decision.

Observation 1: RAN2 can wait for RAN3’s final decision on one of the above options with respect to UE’s behaviour for RVQoE reporting after the threshold is met before working further on the topic.

However, RAN3 in RAN3#120 agreed the following with respect to buffer level threshold-based RVQoE reporting:

- The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

Although RAN3 agreed the above with respect to the configuration of RVQoE thresholds, a recent LS [2] sent from SA4 to RAN2, SA4 indicates that it reverts its previous decision and that it does not confirm the RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18. Furthermore, SA4 in their LS to RAN2 detail that alternatives are being considered and RAN2 will receive the respective updates.

Observation 2: Given the new SA4 LS, RAN2 cannot continue the work on the specification of buffer-level based RvQoE reporting without further updates from SA4.

Proposal 1: RAN2 to not specify buffer-level based RVQoE reporting before a new update is received from SA4.

Proposal 2: RAN2 to send LS response to SA4 to clarify whether the buffer-level RVQoE reporting is still valid for Rel-18 and identify the next steps from SA4.

## 2.2	QoE measurement handling at RAN overload

For RAN overload handling the following agreements were reached in RAN3#119:

- In case assistance information for handling of QoE reporting upon RAN overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 to further discuss what the assistance information is. From RAN3 perspective, there is no need to send assistance information to UE.

Based on the above, RAN3 provided an LS to RAN2 in [3]. Furthermore, RAN3 in RAN3#119bis-e has provided an LS to SA5 in [4] to ask about the feasibility of introducing assistance information for handling of QoE reporting and for SA5 to clarify the details and contents of such assistance information. However, no reply has been provided by SA5 so far.

Although no reply from SA5 is available yet, from our understanding assistance information could be useful at the UE side especially for the use cases of RAN overload or when the UE is in RRC-IDLE/INACTIVE state and performs QoE measurements for MBS services. In those scenarios, the contents of the assistance information sent by the gNB to the UE may be used by the UE to decide for instance which of the QoE measurements are to be discarded or not when the access stratum buffer becomes full.

Proposal 3: gNB may provide assistance information to the UE including at least priority information, FFS on additional content needed.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN2 can wait for RAN3’s final decision on one of the above options with respect to UE’s behaviour for RVQoE reporting after the threshold is met before working further on the topic.

Observation 2: Given the new SA4 LS, RAN2 cannot continue the work on the specification of buffer-level based RvQoE reporting without further updates from SA4.

And proposed the following:

Proposal 1: RAN2 to not specify buffer-level based RVQoE reporting before a new update is received from SA4.

Proposal 2: RAN2 to send LS response to SA4 to clarify whether the buffer-level RVQoE reporting is still valid for Rel-18 and identify the next steps from SA4.

Proposal 3: gNB may provide assistance information to the UE including at least priority information, FFS on additional content needed.

# 4	References

[1] S4-230461/R2-2302042, Reply LS on buffer level threshold-based RVQoE reporting, SA4

[2] S4-231119/R2-2307074, Reply LS on buffer level threshold-based RVQoE reporting, SA4

[2] R3-231028, LS on assistance information for handling of QoE reporting upon RAN overload, RAN3

[3] R3-232047, LS on the feasibility of introducing assistance information for handling of RVQoE reporting during RAN overload, RAN3