3GPP TSG-RAN WG2 Meeting #123											R2-2308280

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.3

Title: 				Discussion on SSB-less SCell operation for NES

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#98, a new WID [1] of Network energy savings for NR has been agreed. And the objective about SSB-less Cell is the following:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In this paper, we will focus on the SSB-less SCell operation for inter-band CA, and discuss the impact on RAN2 specification.

- Discussion

In RAN4#107, the agreements about SSB-less SCell for inter-band CA are reached as below. According to the agreements, RAN4 has agreed to continue work on SSB-less SCell for inter-band CA including Scenario 1 and Scenario 2a.

Agreements o Continue RAN4 work on the following SSB-less SCell scenarios - Scenario 1: SCell without SSB transmission and with TRS transmission - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective. o Deprioritize RAN4 work on the following SSB-less SCell scenario - Scenario 2: SCell without SSB transmission and without TRS transmission

In RAN2#122, the potential RAN2 impact has been analyzed and discussed with agreements made:

Agreements: 1. If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input. 2. If RAN4 concludes it is feasible, RAN2 can further work on at least the following specification impacts: - RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell. - UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

RAN2 understands at least two issues require specification update, which are common for both scenario 1 and 2a:

- Issue 1: Timing reference of SSB-less SCell

In the field description for absoluteFrequencySSB indicating that no SSB frequency is absent for this cell if the field is defined for the SCell-less SCell only for the intra-band case, as highlighted below:

FrequencyInfoDL field descriptions

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is only supported in case the SCell for which the UE obtains the timing reference is in the same frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, on handover, corresponds to the cell-defining SSB.

To support scenario 1 and 2a, the field description needs to be updated, e.g. by saying that “If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is only supported in case the SCell for which the UE obtains the timing reference is in the same frequency band or a different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.”

- Issue 2: UE capability reporting

A UE capability scellWithoutSSB is defined to indicate whether the UE supports configuration of SCell that does not transmit SSB in the intra-band CA case as shown below:

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA but not supported for inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

To support scenario 1 and 2a, the capability needs to be updated, e.g. by saying that “Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signaling for intra-band CA but not supported for and/or inter-band CA.”

Proposal 1: Agree the following changes to support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, which are common for both scenario 1 and 2a:

FrequencyInfoDL field descriptions

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is supported in case the SCell for which the UE obtains the timing reference is in the same frequency band or a different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, on handover, corresponds to the cell-defining SSB.

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA and/or inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 2: Send an LS to RAN4 to inform the RAN2 decision and impacts.

Some other enhancement can also be considered for UE to turn off its receiver RF chain to further reduce power consumption in scenario 2a with the following examples given:

- Example 1: Explicit NW indication to inform UE that there would be no other DL transmission and UE would turn off RF upon reception of such indication.

- Example 2: Allow UE to turn off its receiver RF chain if PDCCH/PDSCH is not configured in SCell.

But whether to have such enhancements depend on further input from RAN4.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Agree the following changes to support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, which are common for both scenario 1 and 2a:

FrequencyInfoDL field descriptions

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is supported in case the SCell for which the UE obtains the timing reference is in the same frequency band or a different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, on handover, corresponds to the cell-defining SSB.

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA and/or inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 2: Send an LS to RAN4 to inform the RAN2 decision and impacts.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR

- Annex-Draft LS to RAN4

Title:	[Draft] LS on supporting SSB-less SCell operation for inter-band CA

Release:	Rel-18

Work Item:	Netw_Energy_NR -Core

Source:	ZTE [to be RAN2]

To:	RAN4

Contact Person:

Name:				Yuan Gao

E-mail Address:	gao.yuan66@zte.com.cn

1. Overall Description:

RAN2 has agreed the following changes to support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, which are common for both scenario 1 and 2a:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is supported in case the SCell for which the UE obtains the timing reference is in the same frequency band or a different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, on handover, corresponds to the cell-defining SSB.

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA and/or inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#123-bis          2023-10-09		2023-10-13	Xiamen, CN

RAN2#124      	   	  2023-11-13		2023-11-17	Chicago, US