3GPP TSG RAN WG2#123	R2-2308294

Toulouse, France, August 21-25, 2023

Agenda Item:	7.7.2

Source:	CMCC

Title:	Discussion on the LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK

Document for:	Discussion, Decision

Introduction

An LS [1] on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK is sent from RAN1 to RAN2 during the last meeting.

Title: LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK Response to: - Release: Rel-18 Work Items: NR_NTN_enh-Core Source: RAN WG1 To: RAN WG2 CC: - Contact Person: #### Name: Shohei Yoshioka E-mail Address: syouhei.yoshioka.py@nttdocomo.com Send any reply LS to: 3GPP Liaisons Coordinator: mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption. It is noted that this working assumption will be confirmed if Option B is feasible from RAN2 perspective. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH. It is noted that an additional working assumption was reached for repetition request or capability report. Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions. - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication. - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission. - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling. - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.. 2. Actions To TSG RAN WG2 ACTION: RAN1 respectfully asks RAN2 to provide feedback on the feasibility of Option B, and if feasible, to specify the details of Option B.

However RAN2 did not reply to the LS in the end of the last meeting, and in this contribution, we would like to provide some considerations on option B.

Discussion

## 2.1 Feasibility of option B

As discussion in RAN1, regarding coverage enhancements on PUCCH repetition for Msg4 HARQ-ACK, UE behavior(i.e. request repetition or reporting capability) is needed to specify. And in last RAN1 meeting, how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK has been discussed and reached the working assumption about option B as mentioned beforehand.

From RAN2 perspective, 1 bit is enough to indicate repetition request or capability. Msg3 PUSCH contains a C-RNTI MAC CE or CCCH SDU in the current mechanism. And there are 2 reserved bits in the MAC header (as shown in the following Figure 6.1.2-3 of TS 38.321) corresponding to the C-RNTI MAC CE. For CCCH SDU case, there is also no problem to include additional 1 bit in the RRC message(e.g. RRCSetupRequest) carrying the CCCH SDU. Therefore option B (higher layer signaling in Msg3 PUSCH) is feasible.

Figure 6.1.2-3: R/LCID/(eLCID) MAC subheader

Proposal 1: From RAN2 perspective, option B (higher layer signaling in Msg3 PUSCH) is feasible to indicate the repetition request or capability report from UE side.

## 2.2 Specify details of option B

If msg3 PUSCH contains a C-RNTI MAC CE, we could leverage the R bits in the MAC header as mentioned aforesaid. Further, if UE reports repetition request or capability, using any one bit of the two R bits is sufficient.

Proposal 2: For msg3 PUSCH contains a C-RNTI MAC CE, if UE reports repetition request or capability, using any one bit of the two R bits in the MAC header is sufficient.

If msg3 PUSCH contains a CCCH SDU, and if we choose repetition request as the final description(maybe we could consider to down select one description in the repetition request and capability report), for example, for initial random access, we could add repetition request in the corresponding RRC message(i.e. RRCSetupRequest) as follows.

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

repetitionRequest                  RepetitionRequest

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

RepetitionRequest ::=              ENUMERATED {true}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

And for other RRC messages carrying the CCCH SDU not transmitted in initial random access, we could consider to add a similar signaling structure.

Proposal 3: For msg3 PUSCH contains a CCCH SDU, we could add repetition request (if repetition request is the final description) indication in the corresponding RRC message(e.g. RRCSetupRequest) as bellow:

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

repetitionRequest                  RepetitionRequest

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

RepetitionRequest ::=              ENUMERATED {true}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on the LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK and have the following proposals:

Proposal 1: From RAN2 perspective, option B (higher layer signaling in Msg3 PUSCH) is feasible to indicate the repetition request or capability report from UE side.

Proposal 2: For msg3 PUSCH contains a C-RNTI MAC CE, if UE reports repetition request or capability, using any one bit of the two R bits in the MAC header is sufficient.

Proposal 3: For msg3 PUSCH contains a CCCH SDU, we could add repetition request (if repetition request is the final description) indication in the corresponding RRC message(e.g. RRCSetupRequest) as bellow:

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

repetitionRequest                  RepetitionRequest

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

RepetitionRequest ::=              ENUMERATED {true}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

Reference

- R2-2306771_R1-2306105_LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK

- 3GPP TS 38.321 V17.5.0 NR; Medium Access Control (MAC) protocol specification (Release 17)