3GPP TSG-RAN WG2 Meeting #123	R2-2308304

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.2.1

Source:	CMCC

Title:	Discussion on multicast reception in RRC_INACTIVE CP issues

Document for:	Discussion

Introduction

At the RAN2#122 meeting[1], the following agreements were reached regarding the control plane of  mmulticast reception in RRC_INACTIVE：

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline. To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline. Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS. It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling. For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline. As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH. FFS how existing MRBs are handled. Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE. MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI. This is assumed to have no/minor impact on RAN1/PHY

In this contribution, we provide our view on the following remaining open issues of CP design:

- Frequency prioritization

- RRC Resume scenarios

- Session deactivation notification

- Special UE handling

- Inactive UE join

- Initial PTM configuration acquisition

Discussion

Service continuity

Frequency prioritization and neighbour cell list mechanism

In RAN2#121bis-e meeting, service continuity for UE in RRC_INACTIVE receiving multicast was discussed, and it was agreed to use frequency prioritization and neighbour cell list to ensure the UE’s service continuity, while the detail is still under discussion.

For a Rel-17 broadcast UE, it may consider that frequency to be the highest priority during the MBS broadcast session if it is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, as the re-selected cell’s SIB1 contains SIB20 scheduling information, and one or more MBS FSAI(s) of that frequency is indicated in SIB21 and/or in USD [2].

Specifically, in Rel-17, SIB20 is used to provide the information required to acquire the MCCH configuration for MBS broadcast, while SIB21 is used to indicate mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI).

Observation 1: Rel-17 broadcast UE may prioritize the frequency providing MBS service when SIB20 is scheduled in SIB1 of the re-selected cell and the mapping FSAI(s) is indicated in SIB21 and/or in USD.

For Rel-17 broadcast UE, only SIB+MCCH approach is used for PTM configuration and neighbour cell information acquirement, while for Rel-18 multicast UE, the mixed approach is adopted, and a new SIB with similar function as SIB20 is agreed to be introduced. Therefore, it is straightforward that for Rel-18 multicast UE’s frequency prioritization, prioritization related information like FSAI can be provided by dedicated signalling, or system information, or both, except for indicated in USD.

Moreover, it was agreed in RAN2#121 meeting, RRCRelease message can be used to deliver PTM configuration to the UE for multicast reception in RRC_INACTIVE, and in current spec, RRCRelease message can provide cell re-selection prioritization/de-prioritization information for UE, which can override the cell re-selection prioritization/de-prioritization information indicated in the system information. Thus, one potential way is to reuse the CellReselectionPriorities in RRCRelease, which can be configured based on UE’s receiving MBS services. But the limitation is that in case an RRC_INACTIVE UE is interested in other MBS services not receiving now, it cannot prioritize the corresponding frequency. Though indicating FSAI information directly in RRCRelease message can solve the problem mentioned above, it should be noticed that whether the PTM configuration is provided by the gNB via dedicated signaling is optional as agreed in RAN2#121 meeting: If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured [3].  Therefore, UE may have no frequency prioritization related to MBS reception if the frequency information is only provided in RRC dedicated signaling. And this may also occur to the UE re-selecting to a new cell, it will not request RRC connection resumption if MCCH is delivered in the new cell.

Observation 2：UE have no frequency prioritization information if it is only provided in RRCRelease Message due to PTM configuration is not provided in RRCRelease Message or cell re-selection to a cell providing PTM configuration via MCCH.

Besides, the Rel-17 broadcast way can still be introduced for Rel-18 multicast UE by introducing a new SIB to indicate the mapping of between the current and/or neighbouring carrier frequencies and FSAI, it can avoid UE’s RRC state transition, which is align with the Rel-18 multicast principle as well.

Observation 3: USD is used for frequency prioritization.

Proposal 1: A new SIB can be introduced to indicate the mapping of between the current and/or neighbouring carrier frequencies and FSAI for multicast reception. And optionally, the FSAI information can be indicated in RRCRelease message.

Based on the discussion and proposals above, for a Rel-18 multicast UE in RRC_INACTIVE, it may consider that frequency to be the highest priority during the MBS multicast session if it is receiving or interested to receive an MBS multicast service(s) and can only receive this MBS multicast service(s) by camping on a frequency on which it is provided, as the re-selected cell’s SIB1 contains SIB20 scheduling information, and one or more MBS FSAI(s) of that frequency is indicated in RRCRelease message and/or SIB21 and/or in USD.

Proposal 2: For a Rel-18 multicast UE in RRC_INACTIVE, it may consider that frequency to be the highest priority during the MBS multicast session if is receiving or interested to receive an MBS multicast service(s) and can only receive this MBS multicast service(s) by camping on a frequency on which it is provided, as the re-selected cell’s SIB1 contains SIB20 scheduling information, and one or more MBS FSAI(s) of that frequency is indicated in RRCRelease message and/or SIB21 and/or in USD.

RRC Resume Scenarios

As we mentioned in the previous section, one of the Rel-18 multicast reception for RRC_INACTIVE UE design principle is to avoid UE’s RRC state transition as much as possible. And some of mechanisms are introduce for this purpose, such as mixed approach, frequency prioritization and neighbour cell list. However, there are still some cases that RRC connection resumption is needed [4]：

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality. UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

In our opinion, it’s hard to find a straight forward parameter to evaluate reception quality of the multicast data, therefore, we propose to use a channel quality threshold, e.g., RSRP threshold indicated by the network to UE as part of the configuration used in RRC_INACTIVE for the UE to trigger the RRC resume.

Proposal 3: The network can indicate a channel quality threshold, e.g., RSRP threshold to trigger the RRC resumption.

Whether a UE receives multicast in RRC_INACTIVE or an MBS session can be received in RRC_INACTIVE is decided by the gNB considering many factors, for example the cell load, the UE’s location, and the UE’s priority indicated by CN and so on. In case UE is released to RRC_INACTIVE for multicast reception due to serving cell’s load balancing strategy or congestion, once it re-selects to the other cells providing multicast serving only several UEs or with light load, it’s reasonable for the UE to resume to RRC_CONNECTED mode. As shown in Figure 1, in Cell 1, it serves a lot of UEs which results in the congestion in Cell 1, and UE 1(depicted in green) is switched to RRC_INACTIVE for multicast reception, while Cell 2 providing the same MBS service only serves several UEs, it’s reasonable for UE1 to trigger RRC connection resumption for better service once re-select to Cell 2 which is not overload. To avoid network’s rejection to the RRC resumption request, UE can indicate the reason/cause value.

Figure 1 UE re-select to a new cell with light load

Proposal 4: UE can indicate the reason/cause value why it triggers the RRC connection resume, such as current RRC_INACTIVE state is due to previous serving cell’s load balance strategy.

Session deactivation notification

In the previous meetings, some agreements were achieved for session state change notification. For session activation, it will be informed to UE via enhanced group notification, while for session deactivation and PTM configuration change, MCCH change notification mechanism is used, but the detail needs further discussion.

In Rel-17 broadcast, the MCCH change notification is transmitted with a 2-bit bitmap. The MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBS service(s). The LSB in the 2-bit bitmap, when set to '1', indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g., modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighboring cell information modification.

For Rel-18 multicast reception in RRC_INACTIVE, due to session activation is indicated via enhanced group notification, therefore, we can only one bit for the notification of other purpose, e.g.  modification of a configuration of an on-going MBS session(s), MBS session(s) deactivation or neighboring cell information modification, since all the changes results in UE acquiring the new MCCH.

Besides, based on RAN1’s reply to the LS, DCI form4-0 can be reused for multicast MCCH reception in RRC_INACTIVE [5]. Compared to Rel-17 broadcast MCCH change notification, only the MSB of the 2-bit bitmap is used, while the LSB is reserved.

RAN1 thanks RAN2 for the LS R1-2304325/ R2-2304330 on multicast reception in RRC_INACTIVE. RAN1 would like to provide the answers to the following questions: - Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed? - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used. Reply: From RAN1’s perspective, the above RAN2 assumption is feasible. For the detailed DCI formats, DCI format 4_0 can be reused for multicast MCCH reception in RRC_INACTIVE. DCI format 4_1 can be reused for multicast MTCH reception in RRC_INACTIVE. There is no consensus to support DCI format 4_2 for multicast MTCH reception in RRC INACTIVE in RAN1.

Proposal 5: Only one bit for the notification of other purpose than session activation in the DCI, e.g.  modification of a configuration of an on-going MBS session(s), MBS session(s) deactivation or neighboring cell information modification.

Special UE handling

The Special UE indicated by 5GC handling when the session is deactivated was discussed in last meeting, and it was agreed that it can be released to RRC_INACTIVE. Though the special UE indicated by 5GC assistance information is to ensure its multicast reception is in RRC_CONNECTED, we have agreed both group paging and UE-specific paging can be used for Rel-18 multicast UE. If the special UE does not resume the RRC connection after group paging, UE-specific can be used to move it to RRC_CONNECTED. Therefore, the current mechanism is enough to ensure special UE to resume to RRC_CONNECTED upon session activation.

Proposal 6: If the special UE is not in RRC_CONNECTED after group paging, the UE-specific paging can be used to resume to RRC_CONNECTED upon session activation.

Inactive UE join

At the beginning of the discussion of multicast reception for RRC_INACTIVE UEs, two scenarios are identified and agreed in RAN2 [6]:

- - Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception. - - Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session

For Scenario 1, UE joins in the multicast session in RRC_CONNECTED first for multicast reception and then switch to RRC_INACTIVE to continue the multicast reception, therefore, in this scenario, join procedure can be performed based on Rel-17 spec without extra UE impact.

Similarly, in Scenario 2, UE joins the multicast in RRC RRC_CONNECTED first but without data reception and then directed to RRC_INACTIVE for multicast reception. The join procedure can be done as Rel-17 spec, too.

Observation 4: For both two scenarios, UE joins the multicast session in RRC_CONNECTED first, and then switch to RRC_INACTIVE.

Besides the two scenarios, there’s the third scenario (Scenario 3) that is reasonable but not discussed as shown in Figure 2. In this scenario, UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s), which means the UE need to join in the MBS session(s) first.

Figure 2 Scenario 3

Observation 5: There’s the third scenario that UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s) and needs to join in the MBS session.

Proposal 7: RAN2 is kindly asked to discuss the Scenario 3: a UE is released to RRC_INACTIVE first and then perform the join procedure for multicast reception.

If the current procedure is used, UE needs to switch to RRC_CONNECTED to complete the join procedure. And one possible outcome is after UE’s join, it will be released to RRC_INACIVE state for MBS data reception based on gNB’s decision for an MBS session or the specific UE. Assuming the time when UE joins the session, the multicast data transmission has started for a period of time, the UE will spend extra time and UE power consumption for the multicast data reception due to the twice RRC states switches. And in some sense, UE will miss some multicast data due to the extra latency.

Observation 6: In Scenario 3, UE will spend extra time and power consumption for the twice RRC states switches to perform join procedure.

Since that from the core network point of view, both RRC_CONNECTED and RRC_INACTIVE are CM_CONNECTED state, which has no difference to it, and considering the potential impact mentioned above, we propose to support that the join procedure can be triggered or performed by UE in RRC_INACTIVE state without RRC states switch

Proposal 8: Join procedure can be triggered by UE in RRC_INACTIVE state without RRC states switch.

Based on [4], the main purpose of the MBS session join procedure is to indicate the MBS Session ID(s) to the core network to identify the MBS Session UE interested in. Therefore, at least the MBS Session ID(s) should be indicated to the core network when UE is in RRC_INACTIVE. To achieve this, small data transmission mechanism can be considered.

Proposal 9: Small data transmission mechanism can be considered to transmit necessary information of join procedure, for example, MBS Session ID(s).

Besides, based on the current progress of multicast reception for RRC_INACTIVE, it’s gNB to decide which MBS session can be delivered for UEs in RRC_INACTIVE and which UE can receive MBS session in RRC_INACTIVE. Therefore, after the join procedure in RRC_INACTIVE, the gNB can decide the UE’s MBS reception state and indicates in the further procedure.  If UE is indicated to stay in RRC_INACTIVE state for MBS reception, the gNB should provide the related PTM configuration or at least MCCH configuration for the UE.

Figure 3 Enhancements for Scenario 3

Initial PTM configuration acquisition

For the PTM configuration acquisition, the mixed approach was agreed in RAN2#120 meeting [7]:

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - We assume that the UE can only receive multicast service after it joined the session.

According to the agreements, UE may get the initial PTM configuration via dedicated message if no mobility happens and there’s a precondition that UE has joined an MBS session. And based on the previous discussion, RAN2’s consensus is to avoid switching UE to RRC_CONNECTED once it is released to RRC_INACTIVE.

However, as mentioned previous, whether the PTM configuration provided is optional, therefore, it’s a possible case that the joined UE has no PTM configuration when released to RRC_INACTIVE state. Besides, for the case we discussed in section 2.4, a UE in RRC_INACTIVE state may be interested to receive MBS service which need to perform join procedure first, if proposal 11 was agreed, UE will stay in RRC_INACTIVE without RRC state switching, and it cannot get the initial PTM configuration via RRCRelease message.

In the above two cases, it’s rational for the UE to get the initial PTM configuration via MCCH, and the key issue is how to ensure UE to perform join procedure.

Proposal 10: For the case that UE has joined and has no PTM configuration provided via RRCRelease message, UE can acquire the initial PTM configuration via MCCH.

Conclusions

In this contribution, we analyse the open issues of CP design of multicast reception in RRC_IANCTIVE. Following are our observations and proposals.

Observations:

Observation 1: Rel-17 broadcast UE may prioritize the frequency providing MBS service when SIB20 is scheduled in SIB1 of the re-selected cell and the mapping FSAI(s) is indicated in SIB21 and/or in USD.

Observation 2：UE have no frequency prioritization information if it is only provided in RRCRelease Message due to PTM configuration is not provided in RRCRelease Message or cell re-selection to a cell providing PTM configuration via MCCH.

Observation 3: USD is used for frequency prioritization.

Observation 4: For both two scenarios, UE joins the multicast session in RRC_CONNECTED first, and then switch to RRC_INACTIVE.

Observation 5: There’s the third scenario that UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s) and needs to join in the MBS session.

Observation 6: In Scenario 3, UE will spend extra time and power consumption for the twice RRC states switches to perform join procedure.

Proposals:

Proposal 1: A new SIB can be introduced to indicate the mapping of between the current and/or neighbouring carrier frequencies and FSAI for multicast reception. And optionally, the FSAI information can be indicated in RRCRelease message.

Proposal 2: For a Rel-18 multicast UE in RRC_INACTIVE, it may consider that frequency to be the highest priority during the MBS multicast session if is receiving or interested to receive an MBS multicast service(s) and can only receive this MBS multicast service(s) by camping on a frequency on which it is provided, as the re-selected cell’s SIB1 contains SIB20 scheduling information, and one or more MBS FSAI(s) of that frequency is indicated in RRCRelease message and/or SIB21 and/or in USD.

Proposal 3: The network can indicate a channel quality threshold, e.g., RSRP threshold to trigger the RRC resumption.

Proposal 4: UE can indicate the reason/cause value why it triggers the RRC connection resume, such as current RRC_INACTIVE state is due to previous serving cell’s load balance strategy, and so on.

Proposal 5: Only one bit for the notification of other purpose than session activation in the DCI, e.g.  modification of a configuration of an on-going MBS session(s), MBS session(s) deactivation or neighboring cell information modification.

Proposal 6: If the special UE is not in RRC_CONNECTED after group paging, the UE-specific paging can be used to resume to RRC_CONNECTED upon session activation.

Proposal 7: RAN2 is kindly asked to discuss the Scenario 3: a UE is released to RRC_INACTIVE first and then perform the join procedure for multicast reception.

Proposal 8: Join procedure can be triggered by UE in RRC_INACTIVE state without RRC states switch.

Proposal 9: Small data transmission mechanism can be considered to transmit necessary information of join procedure, for example, MBS Session ID(s).

Proposal 10: For the case that UE has joined and has no PTM configuration provided via RRCRelease message, UE can acquire the initial PTM configuration via MCCH.

References

- R2_122_meeting_report;

- 3GPP TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state;

- R2_121_meeting_report;

- R2_121bis_meeting_report;

- R1-2306243 Reply LS on multicast reception in RRC_INACTIVE;

- R2_119_meeting_report;

- R2_120_meeting_report.